SA WG2 Meeting #121	S2-174009

27-31 March 2017, Busan, Korea	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, AT&T, SK Telecom, Huawei, HiSilicon, Orange?, China Mobile

Title:	TS 23.502: UDM services identification

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes UDM services and service operations

# 1	Background

During the last SA2, following was agreed:

- NF services are derived from the information flows that describe end to end functionality, where applicable (see 3GPP TS 23.502 [3], Annex B drafting rules).

- Complete end to end message flows can be described by a sequence of NF service invocations.

NOTE:	While the use of NF services is not restricted to the identified consumers, no specific effort will be made in this release of the specification, to develop the description of NF services beyond what is necessary to support their use in information flows.

It was also agreed that:

“Each of the NF services offered by a Network Function shall be self-contained, reusable and use management schemes independently of other NF services offered by the same Network Function (e.g. for scaling, healing, etc).”

“NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service operation: An elementary unit a NF service is composed of. “

It is proposed that we summarize the NF services in a table with service description, input/output parameters, other relevant information and have a column to determine whether the given call flow is referring to NF service operation or NF service.

# 2	Criteria

We consider the following set of criteria for defining the services:

- each NF service operates on its own set of context(s)

- services operations that operate on a common context data shall be defined within a single service. Any direct access of a context owned by a NF service are only made by the service operations of that NF service.

- One non-service based interface shall be tied to at most one NF service in an NF.

# 3	Considerations for identifying UDM services and service operations

The services exhibited by UDM should mainly include features that are essential for other Network functions would need as described in e2e call flows in section 4.

To start with, the list of functions that UDM performs need to be considered. From 3GPP TS 23.501 section 6.2.7 the following are the list of UDM functions:

The UDR stores data required for functionalities provided by UDM-FE, plus policy profiles required by PCF. The data stored in the UDR includes:

-	User subscription data, including subscription identifiers, security credentials, access and mobility related subscription data and session related subscription data

-	Policy data, as defined in A.3.1.4.

The UDM-FE accesses subscription information stored in a User Data Repository (UDR) and supports the following functionality:

-	Authentication Credential Processing.

-	User Identification Handling.

-	Access Authorization.

-	Registration/Mobility management.

-	Subscription management.

-	SMS management.A front end implements the application logic and does not require an internal user data storage. Several different front ends may serve the same user in different transactions.

NOTE 2:	The interaction between UDM and HSS is implementation specific.

It is proposed to update TS 23.502 to include the following services for UDM

If this is agreeable, then it is proposed to update the call flows in section 5 accordingly (i.e. remove the ones that are not relevant as per e2e call flows in section 4).

* * * * Start Change * * *.*

## 5.2	Network Function services

Editor's note:	Procedures that can be supported as a service by NF.

Editor's note:	The identified NF service(s) described in this clause need be aligned with the end to end call flow.

Editor's note:	The granularity and structure of NF service(s) in this clause are to be evaluated.

Editor's note:	The NF services listed in this section are non-exhaustive and new NF services will be added per progress of the normative work.

### 5.2.1	General

Editor's note:	It is expected that all control plane interactions specified in section 4 for CP NFs will be covered by final list of services and its service operations defined for CP NFs.

### 5.2.3	UDM Services

Editor's note:	The identified NF service(s) in this clause are to be evaluated. 5.2.3.1 "Register UE serving NF" service.

Editor's note:	How to define a service based procedure for "Purge of subscriber data from AMF" is to be determined.

#### 5.2.3.1	General

The following table illustrates the UDM Services.

Table 5.2.3-1: NF Services provided by UDM

| NF service | Description | NF service operations | Consumer | Ref. |
| --- | --- | --- | --- | --- |
| Subscriber Data Management | Retrieve, update, delete, Subscriber data | Get | AMF, SMF, AUSF | Many call flows in section 4 e.g. 4.5.2, 4.5.1 |
|  |  | UpdateNotification | NEF, SMF, SMSF, AUSF, |  |
| UE context management | Register, Deregister and retrieve UE’s Serving NF and other UE related information. | Registration, | AMF, SMF, SMSF | Many call flows in section 4, 4.5.2, 4.2.5.2 |
|  |  | RemoveNotification | AMF, SMF, SMSF |  |
|  |  | Purge | AMF |  |
|  |  | Get | NEF |  |
|  |  | UE_Reachability Notification | NEF |  |
| Authentication | Provides Authentication related data | Get | AUSF |  |

Editor's note:	Whether common or separate services should be defined for Mobility subscription management, Session subscription management and SMS subscription management is FFS.

Next change

#### 5.2.3.2	Nudm_UE Context Management service

#### Editor’s note: whether NF type can be derived from other information or NF type is needed explicitly is FFS.

#### 5.2.3.2.1	Nudm_ UE Context Management _Registration

Service operation name: Nudm_ UE Context Management _Registration

Description: Register UE's serving NF on the UDM

Inputs, Required: Requester NF ID, SUPI, NF type.

Inputs, Optional: subscription data retrieval indication.

Outputs, Required: Result indication.

#### Outputs, Optional: UE subscription data is retrieved by the Requester NF ID, if the subscription data retrieval indication is included.

##### 5.2.3.2.2	Nudm_ UE Context Management_RemoveNotification

Service operation name: Nudm_ UE Context Management_RemoveNotification

Description: UDM notifies the Requester NF, which has subscribed the remove notification before, the Requester NF ID has been removed from the UDM due to a new serving NF for the UE registered in the UDM.

Known NF Consumers: AMF, SMF, SMSF

Inputs, Required: Serving NF registered in UDM implicitly/explicitly

Inputs, Optional: None.

Outputs, Required: SUPI, serving NF change reason SUPI, serving NF removal reason.

##### Outputs, Optional: None

##### 5.2.3.2.3	Nudm_ UE Context Management_Purge

Service operation name: Nudm_ UE Context Management Purge

Description: UDM deletes the information related to the Requester NF in the UE context.

Known NF Consumers: AMF, SMF, SMSF

Inputs, Required: SUPI, NF type

Inputs, Optional: None

Outputs, Required: Result Indication.

##### Outputs, Optional: None

##### 5.2.3.2.4	Nudm_ UE Context Management_Get

Service operation name: Nudm_ UE Context Management_Get

Description: The Requester NF request the UDM to get the serving NF of the UE.

Known NF Consumers: NEF

Inputs, Required: UE ID, NF Type

Inputs, Optional: None

Outputs, Required: SUPI, Serving NF ID of the NF type requested by Requester NF.

Outputs, Optional: None

#### 5.2.3.2.5	Nudm_UE Context Management_Reachability Notification

Service operation name: Nudm_ UE Context Management Reachability Notification

Description: NF consumer retrieves the UE reachability status from the UDM

Known NF Consumers: NEF

Inputs, Required:.UE ID

Inputs, Optional: None.

Outputs, Required: UDM notifies the requester NF when the corresponding UE is reachable

Outputs, Optional: None

#### 5.2.3.3	Subscriber Data Management Service

Editor’s note: whether NF type can be derived from other information or NF type is needed explicitly is FFS.

##### 5.2.3.3.1	Nudm_Subscriber_Data_Get

Service name: Nudm_Subscriber Data_Get

Description: Requester NF gets the subscriber data from UDM

Known NF Consumers: SMF, SMSF, AMF, AUSF

Inputs, Required: SUPI, NF ID, NF type.

Inputs, Optional: DNN if the NF type is SMF.

Outputs, Required: The Requester NF gets the requested subscription data.

##### Outputs, Optional: None

##### 5.2.3.3.2	Nudm_SubscriptionData_UpdateNotification

Service or service operation name: Nudm_Subscription Data_UpdateNotification

Description: The UDM notifies NF consumer of the updates of UE's Subscriber Data.

Known NF Consumers: AMF, SMF, SMSF

Inputs, Required: NF type.

Inputs, Optional: None.

Outputs, Required: SUPI, Updated subscription Data. The UDM checks the UE ID and NF type to retrieve corresponding subscriber data and providing the data to the Requester NF. In case of Requester NF is SMF, the subscriber data may contain e.g., PDU type(s), authorized SSC mode(s), Default QoS profile.

Outputs, Optional: None

#### 5.2.3.3	Nudm_Authentication Service

Editor’s note: whether NF type can be derived from other information or NF type is needed explicitly is FFS.

##### 5.2.3.3.1	Nudm_Authentication_Get

Service operation name: Nudm_Authentication_Get

Description: Requester NF gets the authentication data from UDM

Known NF Consumers: AUSF

Inputs, Required: SUPI.

Inputs, Optional:

Outputs, Required: Retrieve Authentication data for a certain UE

Outputs, Optional: None

End changes