SA WG2 Meeting #122 Bis	S2-175552

21 - 25 August 2017, Sophia Antipolis, France

Source:	Huawei, CATR, China Mobile

Title:	Voice service procedure for the UE camping on 5GS

Document for:	Approval

Agenda Item:	6.5.8.1

Work Item / Release:	5G-Ph1 / Rel-15

Abstract of the contribution: Discusses the need and solutions for voice fallback and proposes text in TS 23.502.

- Discussion

Refer to the discussion in a companion paper submitted for this meeting (S2-175551).

2	Proposal

It is proposed to Update TS 23.502 as follows.

######################### TEXT PROPOSAL FOR TS 23.502 ###########################

4.9.1	Handover procedures in 3GPP access

[…]

4.9.1.x  RAT fallback to E-UTRA connecting to 5GC

When NG RAN receives a N2 PDU Session Request from the AMF which includes a SM information from the SMF requesting to set up a 5QI/QFI=1 QoS flow, the NG RAN shall decide to handover the UE to E-UTRAN connecting to 5GC if voice services cannot be obtained on the current RAT, and 5G RAN sends a N2 PDU Session Ack indicating 5QI/QFI=1 QoS flow setup is rejected, with a reason that HO procedure is in progress. The handover procedure is specified by reusing the Inter NG-RAN node handover without Xn interface procedure as specified in clauses 4.9.1.2 where the two NG-RAN nodes are replaced by a NG-RAN node and an E-UTRAN node.

######################### NEXT PROPOSAL FOR TS 23.502 ###########################

4.11.1	Handover procedures for single-registration mode

[…]

4.11.1.x  RAT fallback to E-UTRA connecting to EPC

When 5G RAN receives a N2 PDU Session Request from the AMF which includes a SM information from the PGW-C+SMF requesting to set up a 5QI/QFI=1 QoS flow, the 5G RAN shall decides to handover the UE to E-UTRAN connecting to EPC if voice services cannot be obtained on the current RAT , and 5G RAN sends a N2 PDU Session Ack indicating 5QI/QFI=1 QoS flow setup is rejected, with a reason that HO procedure is in progress. The handover procedure is specified by reusing the 5GS to EPS handover using Nx interface procedure as specified in clauses 4.11.1.1.

#########################END of CHANGE IN TS 23.502 ###########################