SA WG2 Meeting #122BIS	S2-175654

21- 25 Aug 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon

Title:	TS 23.502: Update of Registration Procedure

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to update Registration procedure to clarify the CM state of performing mobility registration and the generation of registration area.

# Introduction

### 1#: Mobility registration update in CM-CONNECTED mode

It is agreed in 5.3.2.2.3 of TS 23.501 that Mobility Registration Update is performed when UE move out of current TA list in spite of UE CM state:

In the RM-REGISTERED state, the UE shall: - perform mobility Registration Update procedure if the current TA I of the serving cell (see TS 37.nnn) is not in the list of TAIs that the UE has received from the network in order to maintain the registration and enable the AMF to page the UE;

But in 4.2.2.2.1 of TS 23.502 it mentioned that the mobility registration update is performed only in idle state:

The Registration procedure is used when the UE needs to perform initial registration to the 5G system, mobility registration update upon changing to a new Tracking area (TA) outside the UE's registration area in idle mode, when the UE performs a periodic registration update (due to a predefined time period of inactivity), and additionally when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure.

The descriptions in TS 23.501 and TS 23.502 are not consistent.

When UE is in CM-CONNECTED state, during handover, if the UE is out of the old TA list, the Registration Update procedure shall be used to e.g. allocate new TA list to the UE and update location of AMF to the UDM.

Proposal 1: Remove “in idle mode” to align with TS 23.501, to allow mobility registration update in CM-CONNECTED state.

### 2#: Registration area in registration accept message should be optional

In current TS 23.502, Registration area is included in Registration Accept:

21. New AMF to UE: Registration Accept (5G-GUTI, Registration area. Mobility restrictions, PDU session status, NSSAI, Periodic registration update timer, LADN Information and accepted MICO mode).

For some registration procedures, e.g. periodic registration and AMF does not change, the registration area does not have to be changed. In this case, the old registration area need not be send to the UE again. And the UE shall consider the old registration area as valid.

Proposal 2: If the AMF allocates a new registration area, it shall send the registration area to UE via Registration Accept message. If there is no registration area included in the Registration Accept message, the UE shall consider the old registration area as valid.

### 3#: Registration area management based on last visited TAI

Similar as discussed in EPC, in order to reduce the ping-pong effect, the last visited TAI should be taken into consideration when the AMF allocates registration area to UE.

Proposal 3: If available, the UE should report the last visited TAI to the AMF for Registration area allocation.

### 4#: editorial change

The following step number in 4.2.2.2.2 should be modified to align with current call flow:

If the Registration type indicated by the UE is periodic registration update, then steps 20 may be omitted.

# 2	Proposals

It is proposed to adopt the following changes in TS 23.502.

* * * * Begin of first Changes * * * *

### 4.2.2	Registration Management procedures

#### 4.2.2.1	General

#### 4.2.2.2	Registration procedures

Editor's note:	Procedure for the UE to register itself with the CN of the selected PLMN. Procedure includes aspects required to support network slicing, and policy control. Procedure may include e.g. authentication/authorisation, RRM, Capability handling, DRX aspects. Network sharing is considered as part of the procedure description.

##### 4.2.2.2.1	General

A UE needs to register with the network to get authorised to receive services, to enable mobility tracking and to enable reachability. The Registration procedure is used when the UE needs to perform initial registration to the 5G system, mobility registration update upon changing to a new Tracking area (TA) outside the UE's registration area, when the UE performs a periodic registration update (due to a predefined time period of inactivity), and additionally when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure.

The General Registration call flow in clause 4.2.2.2.2 applies on all these registration procedures, but the periodic registration need not include all parameters that are used in other registration cases.

Editor's note:	Further details due to network slicing is FFS.

Editor's note:	Aspects related to dual registration in 3GPP and non-3GPP access is FFS.

Editor's note:	The identities needs to be aligned with the agreed identities.

Editor's note:	Enhancements of the normative wording (i.e. adding shall/should/may) is FFS.

During the initial registration the Permanent Equipment Identifier is obtained from the UE. The AMF operator may check the PEI with an EIR. The AMF passes the PEI (IMEISV) to the UDM, to the SMF and the PCF.

##### 4.2.2.2.2	General Registration

Figure 4.2.2.2.2-1: Registration procedure

1.	UE to (R)AN: AN message (AN parameters, RM-NAS Registration Request (Registration type, SUPI or 5G-GUTI, last visited TAI (if available), Security parameters, NSSAI, UE 5GCN Capability, PDU session status, PDU session(s) to be re-activated, Follow on request, and MICO mode preference)).

Editor's note:	The required AN parameters are FFS.

In case of 5G-RAN, the AN parameters include e.g. SUPI or the 5G-GUTI, the Selected Network and NSSAI.

In case of NG-RAN, the AN parameters also include Establishment cause. The Establishment cause provides the reason for requesting the establishment of an RRC connection.

The Registration type indicates if the UE wants to perform an "initial registration" (i.e. the UE is in RM-DEREGISTERED state), a "mobility registration update " (i.e. the UE is in registered state and initiates a Registration procedure due to mobility) or a "periodic registration update" (i.e. the UE is in registered state and initiates a Registration procedure due to the periodic update timer expiry, see clause 4.2.2.2.1). The UE performing an initial registration (i.e., the UE is in RM-DEREGISTERED state) to a PLMN for which the UE does not already have a 5G-GUTI, the UE shall include its SUPI in the registration attempt. In other cases, the 5G-GUTI is included and it indicates the last serving AMF. If the UE is already registered via a non-3GPP access in a PLMN different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the 3GPP access, the UE shall not provide over the 3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the non-3GPP access. Also, if the UE is already registered via a 3GPP access in a PLMN (i.e. the registered PLMN), different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the non-3GPP access, the UE shall not provide over the non-3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the 3GPP access.

If available, the last visited TAI shall be included in order to help the AMF produce a good list of TAIs for the UE.

The Security parameters are used for Authentication and integrity protection. NSSAI indicates the Network Slice Selection Assistance Information (as defined in clause 5.15 of TS 23.501 [2]. The PDU session status indicates the previously established PDU sessions in the UE. The PDU session(s) to be re-activated is included to indicate the PDU session(s) that the UE intends to activate. The Follow on request is included when the UE has pending uplink signalling and the UE doesn't include PDU session(s) to be re-activated.

Editor's note:	The content of the Security parameters is FFS.

Editor's note:	Changes required to support "stickiness" is FFS.

Editor's note:	It is FFS if HO attach indication is to be added.

Editor's note:	It is FFS how UE Radio capabilities are provided.

2.	If a SUPI is included or the 5G-GUTI does not indicate a valid AMF the (R)AN, based on (R)AT and NSSAI, if available, selects an AMF

The (R)AN selects an AMF as described in TS 23.501 [2], clause 6.3.5.

If the (R)AN cannot select an appropriate AMF, it forwards the Registration Request to an AMF which has been configured, in (R)AN, to perform AMF selection.

3.	(R)AN to new AMF: N2 message (N2 parameters, RM-NAS Registration Request (Registration type, Subscriber Permanent Identifier or 5G-GUTI, Security parameters, NSSAI and MICO mode preference)).

When 5G-RAN is used, the N2 parameters include the Location Information, Cell Identity and the RAT type related to the cell in which the UE is camping.

When NG-RAN is used, the N2 parameters also include the Establishment cause.

If the Registration type indicated by the UE is periodic registration update, then steps 4 to 17 may be omitted.

4.	[Conditional] new AMF to old AMF: Namf_Communication_UEContextTransfer  (complete Registration Request).

If the UE's 5G-GUTI was included in the Registration Request and the serving AMF has changed since last registration, the new AMF may invoke the Namf_Communication_UEContextTransfer service operation on the  old AMF including the complete Registration Request IE, which may be integrity protected, to request the UE's SUPI and MM Context. See clause 5.2.2.2.2 for details of this service operation. The old AMF uses the integrity protected complete registration request IE to verify if the context transfer service operation invocation corresponds to the UE requested.

The old AMF also transfers the event subscriptions information by each consumer NF, for the UE, to the new AMF.

NOTE: The consumer NFs does not need to subscribe for the events once again with the new AMF after the UE is successfully registered with the new AMF.

5.	[Conditional] old AMF to new AMF: Response to Namf_Communication_UEContextTransfer (SUPI, MM Context, SMF information).

Old AMF responds  to the new AMF for the Namf_Communication_UEContextTransfer invocation by including the UE's SUPI and MM Context.

If old AMF holds information about active PDU Sessions, the old AMF includes SMF information including SMF identities and PDU session identities.

If old AMF holds information about active N2AP UE-TNLA bindings to N3IWF, the old AMF includes information about the N2AP UE-TNLA bindings.

6.	[Conditional] new AMF to UE: Identity Request ().

If the SUPI is not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE.

If the new AMF has already received UE contexts from the old AMF during handover procedure, then step 4 and 5 shall be skipped.

7.	[Conditional] UE to new AMF: Identity Response ().

The UE responds with an Identity Response message including the SUPI.

8.	The AMF may decide to invoke an AUSF. In that case, the AMF, shall be based on SUPI, select an AUSF as described in TS 23.501 [2], clause 6.3.4.

9a.	The AUSF shall initiate authentication of the UE.

The authentication is performed as described in clause X.

Editor's note:	It is FFS how UDM is selected in this step.

Editor's note:	It is FFS whether the AUSF or the AMF initiates the authentication.

If network slicing is used, the AMF decides if the Registration Request needs to be rerouted as described in clause 4.2.2.2.3, where the initial AMF refers to the AMF.

9b	The AMF shall initiate NAS security functions.

The NAS security is performed as described in clause X.

10.	[Conditional] new AMF to old AMF: Namf_Communication_RegistrationCompleteNotify ().

If the AMF has changed the new AMF notifies the old AMF that the registration of the UE in the new AMF is completed by invoking the Namf_Communication_RegistrationCompleteNotify service operation.

If the authentication/security procedure fails, then the Registration shall be rejected, and the new AMF invokes the Namf_Communication_RegistrationCompleteNotify service operation with a reject indication reason code towards the old AMF. The old AMF continues as if the UE context transfer service operation was never received.

See clause 5.2.2.2.3 for details of Namf_Communication_RegistrationCompleteNotify service operation.

11.	[Conditional] new AMF to UE: Identity Request ().

If the PEI was not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE to retrieve the PEI.

12.	Optionally the new AMF initiates ME identity check by invoking the N5g-eir_MEIdentityCheck_Get service operation (see clause 5.2.4.2.2).

The PEI check is performed as described in clause 4.7.

13.	If step 14 is to be performed, the new AMF, based on the SUPI, selects a UDM.

The AMF selects an UDM as described in TS 23.501 [2], clause X.

14a. If the AMF has changed since the last registration, or if the UE provides a SUPI which doesn't refer to a valid context in the AMF, or if the UE registers to the same AMF it has already registered to a non-3GPP access (i.e. the UE is registered over a non-3GPP access and initiates this registration procedure to add a 3GPP access), the new AMF invokes the Nudm_UEContextManagement_Registration service operation towards the UDM. If there is no subscription context for the UE in the AMF, the "subscription data retrieval indication" is included. The new AMF provides the access type it serves for the UE to the UDM and the access type is set to "3GPP access". The UDM stores the associated access type together with the serving AMF.

See sub-clause 5.2.3.2.1 for details of Nudm_UEContextManagement_Registration service operation.

Editor's note:	Whether interactions with UDM and PCF includes mobility restriction related information is FFS.

Editor's note:	The PEI is to be provided to the UDM in the Update Location procedure.

14b. [Conditional] If "the subscription data retrieval" indication was included in step 14a, the UDM invokes the Nudm_SubscriptionData_UpdateNotification service operation to provide the subscription data from the UDM. See clause 5.2.3.3.2 for details of this service operation. The new AMF creates an MM context for the UE after getting the mobility related subscription data from the UDM.

14c. When the UDM stores the associated access type together with the serving AMF as indicated in step 14a, it will cause the UDM to initiate a Nudm_UEContextManagement_RemoveNotification (see clause 5.2.3.2.2) to the old AMF corresponding to 3GPP access, if one exists. The old AMF removes the MM context of the UE. If the serving NF removal reason indicated by the UDM is "Initial Registration", then the old AMF invokes the Namf_EventExposure_Notify service operation towards all the associated SMFs of the UE to notify that the UE is de-registered from old AMF. The SMF shall release the PDU session(s) on getting this notification.

15.	Conditionally the new AMF, based on the SUPI, selects a PCF.

The AMF selects a PCF as described in TS 23.501 [2], clause X.

Editor's note:	It is FFS whether the Permanent User ID is used for PCF selection.

Editor's note:	The conditions when the AMF selects a PCF is FFS.

16.	[Optional] new AMF to PCF: Npcf_PolicyControl_PolicyCreate (SUPI).

If the AMF has not yet obtained Access and Mobility policy for the UE or if the Access and Mobility policy in the AMF are no longer valid, the AMF requests the PCF to apply operator policies for the UE by creating a policy control session with the PCF through the Npcf_PolicyControl_PolicyCreate service operation (see clause 5.2.5.2.2). In roaming case, the interaction between H-PCF and V-PCF is required for the provision of Access and Mobility policy.

PCF to new AMF: Response to Npcf_PolicyControl_PolicyCreate (Access and Mobility policy data).

The PCF responds to the Npcf_PolicyControl_PolicyCreate service operation and provides the Access and Mobility policy data for the UE to the AMF.

17.	[Conditional] new AMF to SMF: Namf_EventExposure_Notify ().

The AMF invokes the Namf_EventExposure_Notify (see clause 5.2.2.3.4) in one or both of the following scenarios:

-	If the AMF is changed, the new AMF notifies each SMF of the new AMF serving the UE by informing the UE reachability status including the PDU session status from the UE relevant for each SMF. In case the AMF has changed, it is assumed that the old AMF provides the available SMF information. Based on the PDU session status provided by the new AMF checks the PDU session status and in the Namf_EventExposure_Notify service operation the SMF either re-activates the PDU sessions by executing the steps 4 to 12 specified in clause 4.2.3.2 to complete the User Plane(s) setup without sending MM NAS Service Accept from the AMF to (R)AN described in step 9 of clause 4.2.3.2 or  releases any network resources related to PDU sessions that the UE indicated as not established.

-	If the UE was in MICO mode and the AMF had notified an SMF of the UE being unreachable and that the SMF needs not to send DL data notifications to the AMF, the AMF informs the SMF that the UE is reachable.

-	If the AMF had notified an SMF of the UE being reachable only for regulatory prioritized service and the UE enters into Allowed area, the AMF informs the SMF that the UE is reachable.

NOTE 1:	the AMF will also notify any other NF that subscribed to UE reachability that the UE is reachable.

-	If an SMF has subscribed to UE location change notification via Namf_EventExposure_Subscribe service operation and if the AMF detects that the UE has moved out the area of interest subscribed by an SMF serving the UE, the AMF invokes Namf_EventExposure_Notify service operation to inform the SMF of the new location information of the UE.

NOTE 2: Whether notification Ack need a separate message or be realized in the transport layer will be determined in stage 3.

The SMF may decide to trigger e.g. new intermediate UPF insertion or UPF relocation as described in step 5 in clause 4.2.3.2.

If the Registration type indicated by the UE is periodic registration update, then steps 20 may be omitted.

Editor's note:	SMF relocation in relation to a Registration procedure is FFS.

18. New AMF to N3IWF: N2 Request().

The AMF may decide to modify the N2AP UE-TNLA-binding toward N3IWF as described in 4.2.7.1. This is done in case AMF is changed and old AMF have existing N2AP UE-TNLA-bindings toward a N3IWF for the UE.

19.	N3IWF to new AMF: N2 Response().

Editor's note:	It is FFS whether further description is needed on how the old AMF releases the existing N2AP UE-TNLA-bindings toward a N3IWF is FFS.

20.	[Conditional] old AMF to PCF: Npcf_PolicyControl_PolicyDelete ().

If the old AMF previously requested UE context to be established in the PCF, the old AMF terminates the UE context in the PCF by invoking the Npcf_PolicyControl_PolicyDelete service operation (see clause 5.2.5.2.4).PCF to old AMF: Response to Npcf_PolicyControl_PolicyDelete ().

21.	New AMF to UE: Registration Accept (5G-GUTI, Registration area, Mobility restrictions, PDU session status, NSSAI, Periodic registration update timer, LADN Information and accepted MICO mode).

The AMF sends a Registration Accept message to the UE indicating that the registration has been accepted. 5G-GUTI is included if the AMF allocates a new 5G-GUTI. If the AMF allocates a new Registration area, it shall send the Registration area to the UE via Registration Accept message. If there is no Registration area included in the Registration Accept message, the UE shall consider the old registration area as valid. Mobility restrictions is included in case mobility restrictions applies for the UE. The AMF indicates the established PDU sessions to the UE in the PDU session status. The UE removes locally any internal resources related to PDU sessions that are not marked as established in the received PDU session status and for which the UE has requested PDU session establishment and not received SMF response yet. If the PDU session status information was in the Registration Request, the AMF shall indicate the PDU session status to the UE. The NSSAI includes the Allowed S-NSSAIs. If the UE subscription data includes subscribed LADN identification information, the AMF shall include in the Registration Accept message the LADN Information for LADNs, defined in TS 23.501 [2] clause 5.6.5, that are available within the Registration area determined by the AMF for the UE. If the UE included MICO mode in the request, then AMF responds whether MICO mode should be used.

When the Follow on request is included, the AMF shall not release the signalling connection immediately after the completion of the registration procedure.

Editor's note:	The exact reference to steps of UE triggered Service Request procedure in clause 4.2.3.2 will be updated when the UE triggered Service Request procedure is stable.

Editor's note:	It is FFS whether NSSAI is provided to RAN by AMF.

Editor's note:	It is FFS whether LADN subscription information (i.e. information which indicates whether the UE subscribes the LADN service) is included in UE subscription data or is managed by policy information by PCF.

22.	[Conditional] UE to new AMF: Registration Complete ().

The UE sends a Registration Complete message to the AMF to acknowledge if a new 5G-GUTI was assigned.

When the "PDU session(s) to be re-activated" is not included in the Registration Request, the AMF releases the signalling connection with UE, according to clause 4.2.6. When the Follow on request is included in the Registration Request, the AMF shall not release the signalling connection immediately after the completion of the registration procedure.

Editor's note:	Details of N2 interaction is FFS.

* * * * End of first Changes * * * *