SA WG2 Meeting #122bis	S2- 175791

21 - 25 August 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	LG Electronics, SK Telecom

Title:	TS 23.502: Handover of LADN PDU Session

Document for:	Approval

Agenda Item:	6.5.3 Session management and continuity functions and flows

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: It is proposed that SMF sends LADN service area information to the RAN in order to save radio resources during the Xn based handover.

Discussion

When a UE has LADN PDU session and moves out of service area of the LADN, the PDU session is either deactivated or released by the SMF. Such procedure can be occurred during the handover procedure.

When a UE has an active LADN PDU session and RAN may decides to handover the LADN PDU session and as a result, the UE may moves out of service area. If RAN triggered Xn based handover, target RAN assigns all acceptable radio resources during the handover preparation including the LADN PDU session without knowing the service area of LADN PDU session. After completing resource allocation, target RAN sends Path Switch Request message to the AMF and AMF notifies the SMF of UE location information. Then SMF invokes PDU session deactivation or release procedure for the LADN PDU session. This behaviour increases unnecessary radio resources usage in target RAN.

There are two options to solve this issue.

First one is enforcing the RAN to trigger the Inter NG-RAN node handover without Xn interface when there is LADN PDU session. This method increases handover delay when there is LADN PDU session.

Another solution is providing LADN service area information to the RAN when the PDU session is established and user plane is activated. Based on the LADN service area information, if source RAN decided to handover the UE to the outside of LADN service area, source RAN does not send LADN related RAN information to the target RAN. Then target RAN does not assign any resources related with the LADN PDU session and the LADN PDU session is deactivated.

So we propose to provide LADN service area information to the RAN and RAN uses this information to release user plane resource during the handover procedure.

Proposal

Based on the above discussion the following changes to TS 23.502 are proposed.

* * * * Start of 1st Change * * * *

4.3.2.2.1	Non-roaming and Roaming with Local Breakout

This clause specifies PDU Session establishment in the non-roaming and roaming with local breakout cases. The procedure is used to establish a new PDU session as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access.

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, Request type, N1 SM information).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM information. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The Request Type indicates "Initial request" if the PDU Session Establishment is a request to establish a new PDU Session and indicates "Existing PDU Session" if the request refers to an existing PDU Session between 3GPP access and non-3GPP access.

The NAS message sent by the UE is encapsulated by the AN in a N2 message towards the AMF that should include User location information and Access Technology Type Information.

The N1 SM information may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

The AMF receives from the AN the NAS SM message (built in step 1) together with User Location Information (e.g. Cell Id in case of the RAN).

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU session according to the UE subscription. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU session ID and the SMF ID.

The case where the Request Type indicates "Existing PDU Session", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an SMF ID corresponding to the DNN constitutes an error case.

3.	From AMF to SMF: Nsmf_PDUSession_CreateSMRequest (Subscriber Permanent ID, DNN, S-NSSAI, PDU Session ID, AMF ID, N1 SM information (PDU Session Establishment Request), User location information, Access Technology Type, PEI).

The AMF ID uniquely identifies the AMF serving the UE. The AMF forwards the PDU Session ID together with the N1 SM information containing the PDU Session Establishment Request received from the UE. The PEI (Permanent Equipment Identifier) is defined in TS 23.501 [2] clause 5.9.3.

4.	SMF invokes: Nudm_SubscriberData_GetRequest  (Subscriber Permanent ID, DNN).

If the Request Type in step in step 3 indicates "Existing PDU Session" the SMF determines that the request is due to handover between 3GPP access and non-3GPP access. The SMF identifies the existing PDU Session based on the PDU Session ID.

If the SMF has not yet retrieved the SM-related subscription data for the UE related with the DNN, the SMF requests this subscription data.	UDM responds to SMF: with Nudm_SubscriberData_GetResponse.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), Default QoS profile.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If that is not the case the SMF rejects the UE request via NAS SM signalling by responding to the AMF with Nsmf_PDUSession_CreateSMResponse including a relevant SM rejection cause. The SMF indicates to the AMF that the PDU session ID is to be considered as released and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs perform secondary authorization/authentication during the establishment of the PDU session by a DN-AAA server as described in clause 5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in clause 4.3.2.3.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection.

6b.	The SMF may invoke Npcf_PolicyControl_PolicyCreateRequest including PDU Session ID to establish a PDU CAN session and get the default PCC Rules for the PDU Session. If the Request Type in step 3 indicates "Existing PDU Session", the PCF initiates a PDU-CAN Session Modification instead, by invoking the Npcf_PolicyControl_PolicyUpdateNotification service.

Editor's note:	The purpose of step 5 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 5 can be skipped.

7.	If the Request Type in step 3 indicates "Initial request", the SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1. For Unstructured PDU Type the SMF may allocate an IPv6 prefix for the PDU session and N6 point-to-point tunnelling (based on UDP/IPv6) as described in TS 23.501 [2] clause 5.6.9.4.

8a.	SMF invokes the Npcf_PolicyControl_PolicyCreateRequest: If dynamic PCC is deployed and the PDU-CAN Session Establishment has not yet been done, the SMF invokes PCF service to establish PDU-CAN session to get the default PCC Rules for the PDU Session.

8b. SMF provides EventExposure_Notify service operation: Otherwise, if Request Type is "initial request" and dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF notifies the PCF with the allocated UE IP address/prefix.

9.	If Request Type indicates "initial request" and step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

10.	SMF to AMF: Nsmf_PDUSession_CreateSM Response (Cause, N2 SM information (PDU Session ID, QoS Profile(s), CN Tunnel Info, S-NSSAI, LADN Service area), N1 SM information (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode, S-NSSAI, allocated IPv4 address))).

Editor's note:	It is FFS how the AMF and SMF associate the N11 transactions with a corresponding UE context.

The N2 SM information carries information that the AMF shall forward to the (R)AN which includes:

-	The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

-	The QoS Profile provides the (R)AN with the mapping between QoS parameters and QoS Flow Identifiers. Multiple QoS profiles can be provided to the (R)AN. This is further described in TS 23.501 [2] clause 5.7.

-	The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

-	If the SMF provided LADN Service area, the RAN use the information during the handover procedure in order not handover the LADN PDU session when the UE moves out of LADN service area.

S-NSSAI is corresponding to the PDU session.

The N1 SM information contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM information.

The SM Response further contains PDU Session ID and information allowing the AMF to know which the target UE as well to determine which access towards the UE to use.

NOTE 1:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to determine which access towards the UE to use.

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, NAS message (PDU Session ID, PDU Session Establishment Accept)).

The AMF sends the NAS message containing PDU Session ID and PDU Session Establishment Accept targeted to the UE and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN N3 tunnel information for the PDU Session. In case of Dual Connectivity, the Master RAN node may assign some (zero or more) QFIs to be setup to a Master RAN node and others to Secondary RAN node. The RAN tunnel information includes a tunnel endpoint for each involved RAN node, and the QFIs assigned to each tunnel endpoint. A QFI can be assigned to either the Master RAN node or the Secondary RAN node and not to both.

(R)AN forwards the NAS message (PDU Session ID, N1 SM information (PDU Session Establishment Accept)) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, (R)AN Tunnel Info, List of accepted/rejected QoS profile(s))).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: Nsmf_PDUSession_ UpdateSMContext Request (N2 SM information).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

NOTE 2:	If the PDU Session Establishment Request was due to mobility between 3GPP and non-3GPP access, the downlink data path is switched towards the target access in this step.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	SMF to AMF: Nsmf_PDUSession_ UpdateSMContext Response (Cause).

After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

18.	If the PDU Session Establishment request is due to a handover between 3GPP access and non-3GPP access, i.e. the Request type is set to "existing PDU Session" the SMF executes the steps X-Y specified in clause 4.3.4 in order to release the user-plane over the source access (3GPP or non-3GPP access).

Editor's note:	The exact reference to steps of network initiated PDU session release procedure will be defined when the procedure is available.

19.	If the SMF identity is not included in step 4b by the UDM in the DNN subscription context, the SMF invokes the Nudm_UEContextManagement_Registration service operation  including the SMF address and the DNN. The UDM stores the SMF identity, address and the associated DNN.

If during the procedure the PDU session establishment is not successful, the SMF informs the AMF. Once the AMF associates an SMF with a PDU session ID, the SMF is automatically subscribed to notifications of incoming N1 signalling associated with this PDU session ID. These notifications automatically provide any User Location information and Access Type that the AMF has received from the (R)AN in association with N1 signalling.

* * * * Start of 2nd Change * * * *

4.9.1.1.1	Xn based inter NG RAN handover without User Plane function relocation

This procedure is used to hand over a UE from a source RAN to target RAN using Xn when the AMF is unchanged and the SMF decides to keep the existing UPF. The UPF referred in this clause is the UPF which terminates N3 interface in the 5GC. The presence of IP connectivity between the Source UPF and Target UPF is assumed.

The call flow is shown in figure 4.9.1.1.1-1.

Figure 4.9.1.1.1-1 - Xn based inter NG RAN handover without UPF relocation

0.	If the SMF provided LADN Service area and the Source RAN triggers handover to the Target RAN not included in the LADN Service area, the Source RAN exclude LADN PDU session related information when it requests handover to the Target RAN during the handover preparation.

1.	The Target RAN sends an N2 Path Switch Request message to an AMF to inform that the UE has moved to a new target cell and a list of PDU Sessions to be switched. Depending on the type of target cell, the Target RAN includes appropriate information in this message.

For the QoS flows to be switched to the Target RAN, the N2 Path Switch Request message shall include the list of accepted QoS flows.

2.	The AMF sends N2 SM information by using an N11 Message to each SMF associated with the list of PDU Sessions received in the N2 Path Switch Request.

For the PDU Sessions to be switched to the Target RAN, upon receipt of the N11 Message, each of these SMFs determines whether the existing UPF can continue to serve the UE. If the existing UPF cannot continue to serve the UE and it is not a PDU session anchor, steps 3-11 of clause 4.9.1.2 are performed. Otherwise, the following steps 3 to 6 are performed by each of these SMFs if their existing UPFs can continue to serve the UE.

If all the QoS flows of the PDU session(s) are not accepted by the Target RAN, the SMF deactivates those PDU session(s) after the HO procedure.

Editor's note:	It is FFS whether to release or deactivate the PDU session(s) if all of their  QoS flows are not accepted by the Target RAN.

If only partial QoS flows of the PDU session(s) are accepted by the target RAN, the SMF may initiate the PDU session modification procedure to remove  non-accepted QoS flows from the PDU session(s) after the HO procedure.

For the PDU session(s) which are deactivated before HO procedure, the SMF(s) keep the deactivation status after HO procedure. AMF sends the UE's location information to the corresponding SMF if the mobility event is subscribed. SMF may reselect a UPF according to the current UE's location for the PDU session.

NOTE 1:	If there was a LADN PDU session and UE moves out of the LADN service area, the AMF notifies the SMF that the UE has moved out of the service area even if the target RAN does not include LADN PDU session information in N2 Path Switch Request. Based on the notification, the SMF may deactivate PDU session or release the LADN PDU session.

3.	For PDU Sessions requested by the Target RAN, the SMF sends an N4 Session Modification Request (RAN address, tunnel identifiers for downlink User Plane) message to the UPF.

4.	The UPF returns an N4 Session Modification Response (Tunnel identifiers for uplink traffic) message to the SMF after requested PDU Sessions are switched.

5.	In order to assist the reordering function in the Target RAN, the UPF sends one or more "end marker" packets on the old path immediately after switching the path. The UPF starts sending downlink packets to the Target RAN.

NOTE 2:	Step 6 can occur any time after receipt of N4 Session Modification Response at the SMF.

6.	The SMF sends an N11 Message Ack (CN Tunnel Information) to the AMF for PDU Sessions which have been switched successfully.

7.	Once the N11 Message Response is received from all the SMFs, the AMF aggregates received CN Tunnel Information from these responses and sends this aggregated information as a part of N2 SM Information in N2 Path Switch Request Ack to the Target RAN. If none of the requested PDP Sessions have been switched successfully, the AMF shall send an N2 Path Switch Request Failure message to the Target RAN.

8.	By sending a Release Resources message to the Source RAN, the Target RAN confirms success of the handover. It then triggers the release of resources with the Source RAN.

* * * * End of Changes * * * *