SA WG2 Meeting #122bis	S2-176143

August 21 – 25, 2017, Sophia Antipolis, France	(was S2-17----)

Source:	CATT

Title:	TS 23.501: Clarification on N26 hole and corresponding corrections

Document for:	Discussion / Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to clarify the N26 hole case, and proposes some updates.

- Discussion

It is agreed that,

“Networks that support 5GS-EPS interworking procedures without N26 do not need to provide the UEs with mapped target system parameters (e.g. QoS parameters, bearer IDs/QFI, PDU session ID, etc.) of the target system when UE is in the source network.”

The current text actually is describing a deployment scenario where the network has no N26 interface with any MME, but for N26 hole case, i.e., there is not clear description.

Therefore, it is propose to clarify that the network supporting both interwork with N26 and interworking without N26 needs to provide the mapped QoS parameter to the UE.

When an AMF receives a request to allocate an EBI(s) for a QoS flow(s) from a PGW-C+SMF, if it has no N26 interface with any MME, it needs not to (even is not able to) assign the EBI(s). therefore, the PGW-C+SMF will not receive the EBI(s), and then shall not provide the mapped EPS QoS parameters associated with the received EBI(s) to the UE. However, if the AMF has N26 interfaces with some MMEs and has no N26 interfaces with other MMEs, the AMF shall assign the EBI(s) and return it to the PGW-C+SMF, which allows the PGW-C+SMF to provide the mapped QoS parameters.

2.	Proposal

It is propose to update the related clause in the TS 23.501.

* * * * First Change * * * *

5.17.2.3	Interworking Procedures without N26 interface

5.17.2.3.1	General

For interworking without the N26 interface, IP address continuity is provided to the UEs on inter-system mobility by storing and fetching PGW-C+SMF and corresponding APN/DDN information via the HSS+UDM. Such networks also provide an indication that dual registration mode is supported to UEs during initial Registration in 5GC. This indication is valid for the entire PLMN. UEs that support dual-registration mode may use this indication to decide whether to register early in the target system.

To support mobility for dual-registration mode UEs, the following are the key highlights of interworking procedure without N26 interface that are supported by the network:

1.	When UE performs initial Attach in EPC and provides a 4G-GUTI mapped from 5G-GUTI, the MME does not include "initial attach" indicator to the HSS+UDM. This results in HSS+UDM not cancelling the registration of AMF, if any.

2.	When UE performs initial Registration in 5GC and provides 5G-GUTI mapped from 4G-GUTI, the AMF does not include "initial attach" indicator to the HSS+UDM. This results in HSS+UDM not cancelling the registration of MME, if any.

3.	When PDN connections are created in EPC, the MME stores the PGW-C and APN information in the HSS+UDM.

4.	When PDU session are created in 5GC, the PGW-C+SMF stores its information along with DDN in the HSS+UDM.

5.	The HSS+UDM provides the information about dynamically allocated PGW-C+SMF and APN/DNN information to the target CN network during location update procedure.

To provide IP address preservation to UEs operating in single-registration mode when the UE moves from 5GC to EPC, the network supports 1, 4, 5 and the following below:

6.	When UE performs TAU in EPC and provides a 4G-GUTI mapped from 5G-GUTI, the MME determines that the old node is AMF and rejects the procedure with a "Handover PDN Connection Setup Support" indication to the UE.

NOTE-1:	Items 4 and 5 are also supported in networks that support interworking with N26 procedures. This enables a VPLMN that does not deploy N26 interface to provide IP address continuity to roamed-in single-registration mode UEs from a HPLMN that only supports interworking with N26 procedures.

To provide IP address preservation to UEs operating in single-registration mode when the UE moves from EPC to 5GC, the network supports 2, 3, 5 and the following below:

7.	When UE performs mobility Registration in 5GC and provides a 5G-GUTI mapped 4G-GUTI, the AMF determines that the old node is MME and proceeds with the procedure and provides a "Handover PDU Session Setup Support" indication to the UE in the Registration Accept message.

When an AMF without support of N26 interface receives a request to allocate an EBI(s) for a QoS flow(s) from a PGW-C+SMF, it doesn’t provide the EBI(s).

NOTE-2:	A UE in a VPLMN that supports interworking without N26 may be provided with mapped QoS parameters from PGW-C+SMF in HPLMN for home-routed PDN connection, if the HPLMN supports interworking procedures with N26 interface.

A UE that operates in dual registration mode ignores any received mapped target system parameters (e.g. QoS parameters, bearer IDs/QFI, PDU session ID, etc.).

* * * * End of Change * * * *