SA WG2 Meeting #122 bis	S2-176125

21 - 25 August 2017, Sophia Antipolis, France	(revision of S2-175554)

Source:	Huawei, HiSilicon

Title:	TS 23.502: Paging optimization for RRC Inactive

Document for:	Discussion/Approval

Agenda Item:	6.5.7.5

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution updates the UE context release flow and the Service Request flow to support intelligent RAN-based notification area allocation when the UE entering in RRC Inactive state.

# Discussion

As per section 5.3.3.2.5, “The AMF, based on network configuration may provide assistance information to the NG RAN, to assist the NG RAN's decision whether the UE can be sent to RRC Inactive state.”

This paper analyses if some additional information is needed in RRC Inactive assistance information. To put it more detail, this paper discusses the need of the paging information from RAN based history knowledge for the intelligent RAN-based notification area allocation.

Different UEs may have different mobility pattern, and hence there exists a need that intelligent paging area allocation can be supported and different UEs can be provided with different optimal RAN notification area (RNA), compared with static RNA allocation based on the configuration on RAN. For example, a UE with a high mobility shall more likely be allocated with a larger RNA compared to a UE with low mobility.

RNA is managed and allocated by NG- RAN. If intelligent RNA allocation were supported, the UE’s RAN based history knowledge (e.g. history location information) could be very useful for the RAN when allocating the RNA.

However, when the UE turns into idle state, the UE context on the RAN is released, resulting in no continuous RAN based history knowledge for the UE on the RAN ,and the RAN is not able to utilize this information for allocating an intelligent paging Area.

It is proposed that the RAN informs the AMF UE’s history related data(e.g. list of cells /TAs and duration) , which is not processed by the AMF, in the UE Context Release Request. The AMF stores these information and informs to RAN during the service request procedure to assist the NG RAN’s decision on RNA allocation when the UE can be sent to RRC inactive state. This proposal is similar to EPC where the “The eNodeB may include the Information On Recommended Cells And eNodeBs For Paging in the S1 UE Context Release Complete message”.

# Proposal

It is proposed to accept the following changes to TS 23.502.

***    1st Change   ***

### 4.2.6	UE context release in the AN

This procedure is used to release the logical N2-AP signalling connection and the associated N3 User Plane connections.

When the N2-AP signalling connection is lost due to (R)AN or AMF failure, the UE context release in the AN procedure is performed locally by the AMF or the (R)AN as described in the procedure flow below without using or relying on any of the signalling shown between (R)AN and AMF. The UE context release in the AN causes all PDU sessions of the UE to be deactivated.

The initiation of UE context release in the AN procedure may be due to:

-	(R)AN-initiated with cause e.g. O&M Intervention, Unspecified Failure, AN (e.g. Radio) Link Failure, User Inactivity, Release due to UE generated signalling connection release, etc.; or

-	AMF-initiated with cause e.g. Unspecified Failure, etc.

Both (R)AN-initiated and AMF-initiated UE context release in the AN procedures are shown in Figure 4.2.6-1.

For this procedure, the impacted SMF and UPF are all under control of the PLMN serving the UE, e.g. in Home Routed roaming case the SMF and UPF in HPLMN are not involved.

# Figure 4.2.6-1: UE context release in the AN Procedure

1.	If there is some confirmed AN conditions (e.g. Radio Link Failure) or for other (R)AN internal reason, the (R)AN may decide to initiate the UE context release in the AN. In this case, the (R)AN sends a N2 UE Context Release Request (Cause) message to the AMF. Cause indicates the reason for the release (e.g. AN Link Failure, O&M intervention, unspecified failure, etc.).

The decision to initiate a UE context release in the AN Procedure may also be triggered by an internal AMF event

2.	[Conditional] For each of the PDU sessions of the UE whose UP is active, the AMF requires the SMF to deactivate the PDU session: the AMF sends a N11 PDU session deactivation Request (PDU session ID, Release Cause).

Editor's note:	The  handling of the GBR QoS flows upon specific Release Causes will be determined as part of QoS discussions .

Editor's note:	This step could also be Namf_ UE Mobility Event Notification (SMF): The AMF sends (N11 message) UE Mobility Event Notification, which indicates the N2 Release event, to the SMF(s) who serves active PDU session(s) of the UE as defined in clause 5.2.2.2. The message includes SUPI, the RAN information, and cause code.

3a	[Conditional] SMF to UPF: N4 Session Modification Request (AN tunnel info to be removed, Buffering on/off).

The SMF initiates a N4 Session Modification procedure indicating the need to remove AN Tunnel Info.  Buffering on/off indicates whether the UPF shall buffer incoming DL PDU or not.

See clause 4.4 for more details.

3b.	[Conditional] UPF to SMF: N4 Session Modification Response acknowledging the SMF request.

See clause 4.4 for more details.

4.	[Conditional] SMF to AMF: N11 PDU session deactivation Response.

Editor's note:	This step could also be Nsmf_ PDU Session Event Report Notification (AMF): The SMF sends (N11 message) PDU Session Event Report Notification as defined in clause 5.2.7.1. The notification includes a UE Identifier (SUPI), PDU Session ID(s), and event trigger (deletion of (R)AN tunnel information).

5.	After the AMF collects all the PDU Session Event Report Notification in step 4 from the SMF(s) that were notified in Step 2.The AMF sends to the (R)AN a N2 UE Context Release Request. In case the AN is a RAN this step is described in detail in RAN specifications. In case the AN is a N3IWF this step is described in clause 4.12

Editor's note:	It is FFs whether AMF collects all the PDU Session Event Report Notification or can issue a N2 UE Context Release Request immediately

6.	If the AN connection (e.g. RRC connection) with the UE is not already released (step 1), the (R)AN requests the UE to release the AN connection. Upon receiving AN connection release confirmation from the UE, the (R)AN deletes the UE's context.

7.	The (R)AN confirms the N2 Release by returning an N2 UE Context Release Complete () message to the AMF. The (R)AN provides the list of recommended cells / TAs / NG-RAN node identifiers for paging to the AMF. With this, the signalling connection between the AMF and the (R)AN for that UE is released.

Editor's note:	During this procedure the AN may provide location information to the AMF. Details on the location information (e.g. Cell ID, TAI) provided by the AN are FFS.

*** End of 1st Changes ***

*** Start of 2nd Change ***

#### 4.2.3.2	UE triggered Service Request in CM-IDLE state

Editor's note:	Procedure includes aspects required to support network slicing.

The Service Request procedure is used by a 5G UE in CM-IDLE state to request the establishment of a secure connection to an AMF. The UE in CM-IDLE state initiates the Service Request procedure in order to send uplink signalling messages, user data, or response to a network paging request. After receiving the Service Request message, the AMF may perform authentication, and the AMF shall perform the security procedure. After the establishment of a secure signalling connection to an AMF, the UE or network may send signalling messages, e.g. PDU session establishment from UE to the network, or the SMF, via the AMF, may start the User Plane resource establishment for the PDU sessions requested by network and/or indicated in the Service Request message.

For any Service Request, the AMF may respond with a response message to synchronize PDU session status between UE and network. The AMF may also respond with Service Reject message to UE, if the Service Request cannot be accepted by network.

For Service Request due to user data, network may take further actions if User Plane resource establishment is not successful.

The procedure in this clause 4.2.3.2 is applicable to the scenarios with or without intermediate UPF, and with or without intermediate UPF reselection.

NOTE 1:	The procedure in this clause 4.2.3.2 is not applicable for an access network (once the UE is registered in the network) in which the UE is always considered as in CM-CONNECTED state and in which the User Plane resource is always considered established for an active PDU session.

Editor's note:	The names of N2 message and N11 message are FFS.

# Figure 4.2.3.2-1: UE triggered Service Request procedure

1.	UE to (R)AN: AN message (AN parameters, MM NAS Service Request (PDU session(s) to be activated, security parameters, PDU session status)).

Editor's note:	The required AN parameters are FFS.

In case of NG-RAN, the AN parameters include Establishment cause. The Establishment cause provides the reason for requesting the establishment of an RRC connection.

The UE sends NAS  Service Request message towards the AMF encapsulated in an RRC message to the RAN. The RRC message(s) that can be used to carry the 5G Temporary ID and this NAS message are described in RAN specifications.

If the Service Request is triggered for user data, the UE identifies the PDU session(s) to be activated in NAS Service Request message. If the Service Request is triggered for signalling only, the UE doesn't identify any PDU session(s) to be activated. When this procedure is triggered for paging response, if the UE needs to use some PDU session(s), the UE identifies the PDU session(s) to be activated in MM NAS Service Request message. Otherwise the UE will not identify any PDU session(s) to be activated.

If the Service Request over 3GPP access is triggered for response to the paging indicating non-3GPP access, the NAS Service Request message shall contain the list of PDU sessions associated with the non-3GPP access that can be re-activated over 3GPP, as described in clause 5.6.8 of TS 23.501 [2].

The PDU session status indicates the PDU sessions available in the UE.

Editor's note:	Detailed procedure for network-initiated Service Request over 3GPP access for response to the paging indicating non-3GPP access is FFS.

2.	(R)AN to AMF: N2 Message (N2 parameters, MM NAS Service Request). Details of this step are described in RAN specifications. If the AMF can't handle the Service Request it will reject it.

When NG-RAN is used, the N2 parameters include the 5G Temporary ID, Location information, and RAT type.

When NG-RAN is used, the N2 parameters also include the Establishment cause.

5G Temporary ID is obtained in RRC procedure. RAN selects the AMF according to Temporary ID. The Location Information and RAT type relates to the cell in which the UE is camping.

Based on the PDU session status, the AMF may initiate PDU session release procedure if the PDU session is not available in the UE.

Editor's note:	The name for the global cell-id e.g. whether ECGI is used or some other term is FFS and will need to be determined also in collaboration with RAN WG.

Editor's note:	Procedures for the monitoring events for "Availability after DDN failure" are FFS.

Editor's note:	Procedures and parameters for CSG paging are FFS depending on whether CSG will be supported.

3.	If the Service Request was not sent integrity protected or integrity protection is indicated as failed, the AMF shall initiate NAS authentication/security procedure as defined in clause 4.6.

If the UE triggered the Service Request to establish a signalling connection only, after the security exchange the UE and the network can send signalling and steps 4 to 8 and 11 to 17 are skipped.

4.	[Conditional] AMF to SMF: Nsmf_PDUSession_UpdateSMContext Request (PDU session ID(s), Cause(s), UE location information).

The Nsmf_PDUSession_UpdateSMContext Request  is invoked in one or more of the following scenarios:

-	If the UE identifies PDU session(s) to be activated in the MM NAS Service Request message, or this procedure is triggered by SMF but PDU session IDs from UE correlates to other SMFs than the one triggering the procedure, the AMF sends Nsmf_PDUSession_UpdateSMContext Request to SMF(s) associated with the PDU session ID(s) with Cause set to indicate "establishment of user plane resources" for a PDU session.

-	If the SMF has subscribed to the AMF for UE location change notification, and the UE has moved out or in the subscribed area.

-	If the UE was in MICO mode and the AMF had notified an SMF of the UE being unreachable and that SMF needs not send DL data notifications to the AMF, the AMF informs the SMF that the UE is reachable by using Namf_EventExposure_Notify service operation.

NOTE 2: the AMF will also notify any other NF that subscribed to UE reachability that the UE is reachable.

5.	Based on the new location info, the SMF checks the UPF Selection Criteria according to clause 6.3.3 of TS 23.501 [2], and determines to perform one of the following:

-	continue using the current UPF(s);

-	selects a new intermediate UPF, if the UE has moved out of the service area of UPF that is connecting to RAN, while maintaining the UPF(s) acting as PDU Session Anchor; or

-	trigger re-establishment of the PDU Session to perform relocation of the UPF acting as PDU Session anchor, e.g. the UE has moved out of the service area of the anchor UPF which is connecting to RAN.

6a.	[Conditional] SMF to new UPF (intermediate): N4 Session Establishment Request

If the SMF selects a new UPF to act as intermediate UPF for the PDU session, an N4 Session Establishment Request message is sent to the new UPF, providing Packet detection, Data forwarding, enforcement and reporting rules to be installed on the intermediate UPF. The PDU session anchor info for this PDU Session is also provided to the intermediate UPF.

If the service request is triggered by the network, and a new UPF is selected by the SMF to replace the old (intermediate) UPF, SMF includes the Data forwarding indication.

6b.	new UPF (intermediate) to SMF: N4 Session Establishment Response

The new intermediate UPF sends an N4 Session Establishment Response message to the SMF. In case the UPF allocates CN tunnel info, it provides CN DL tunnel info for the UPF acting as PDU Session Anchor and UL tunnel info (i.e. CN N3 tunnel info) to the SMF. If the Data forwarding indication is received, the new (intermediate) UPF acting as N3 terminating point also sends CN DL tunnel info for the old (intermediate) UPF to the SMF. The SMF starts a timer, to be used in step 20a to release the resource in old intermediate UPF if there is one.

7a.	[Conditional] SMF to UPF (PSA): N4 Session Modification Request

If the SMF selects a new UPF to act as intermediate UPF for the PDU session, the SMF sends N4 Session Modification Request message to PDU session anchor UPF, UPF (PSA), providing the DL tunnel information from new intermediate UPF.

7b.	UPF (PSA) to SMF: N4 Session Modification Response

The UPF (PSA) sends N4 Session Modification Response message to SMF.

8a. [Conditional] SMF to old UPF (intermediate): Nsmf_PDUSession_UpdateSMContext Request

If the service request is triggered by the network, and a new UPF is selected by SMF to replace the old (intermediate) UPF, the SMF sends the N4 Session Modification Request message to the old (intermediate) UPF, providing the DL tunnel information from the new (intermediate) UPF acting as N3 terminating point. The SMF starts a timer to monitor the forwarding tunnel.

8b.	old UPF (intermediate) to SMF: Nsmf_PDUSession_UpdateSMContext  Response

The old (intermediate) UPF sends Nsmf_PDUSession_UpdateSMContext Response message to SMF.

9.	[Conditional] old UPF (intermediate) to new UPF (intermediate): buffered downlink data forwarding

The old (intermediate) UPF forwards its buffered data to the new (intermediate) UPF acting as N3 terminating point.

10.	[Conditional] SMF to AMF: Nsmf_PDUSession_UpdateSMContext Response (N1 SM information (PDU session ID, PDU Session re-establishment indication), N2 SM information (PDU Session ID, QoS profile, CN N3 Tunnel Info, S-NSSAI)) to the AMF.

Upon reception of the Nsmf_PDUSession_UpdateSMContext Response in step 4 with cause including "establishment of user plane resources", the SMF determines whether UPF reallocation is performed, based on the UE location information, UPF service area and operator policies:

-	For PDU session that SMF determines to be served by the current PDU Session Anchor UPF in step 5, the SMF generates only N2 SM information and sends Nsmf_PDUSession_UpdateSMContext Response to the AMF to establish the User Plane(s). The N2 SM information contains information that the AMF shall provide to the RAN.

-	For PDU sessions that SMF determines that UPF relocation for PDU Session Anchor UPF is needed in step 5, SMF sends Nsmf_PDUSession_UpdateSMContext Response containing only N1 SM information to UE via AMF. The N1 SM information includes the corresponding PDU session ID and PDU session re-establishment indication, which is the same as step 2 described in clause 4.3.5.1.1.

Upon reception of the N11 Message in step 4 with an indication that the UE is reachable, if the SMF has pending DL data the SMF sends N11 Message to the AMF to establish the User Plane(s) for the PDU sessions, otherwise the SMF resumes sending DL data notifications to the AMF in case of DL data.

11.	AMF to (R)AN: N2 Request (N2 SM information received from SMF, security context, AMF Signalling Connection ID, Handover Restriction List, MM NAS Service Accept, list of recommended cells / TAs / NG-RAN node identifiers).

RAN stores the Security Context, AMF Signalling Connection Id, QoS Information for the QoS Flows of the PDU Sessions that are activated and N3 Tunnel IDs in the UE RAN context. The step is described in detail in RAN specifications. Handover Restriction List is described in TS 23.501 [2] clause 5.3.4.1 “Mobility Restrictions”.

MM NAS Service Accept includes PDU session status in AMF. If N1 SM information is received from step 8, the Service Accept message also includes N1 SM information.

AMF shall include at least one N2 SM information from SMF if the procedure is triggered for PDU session User Plane setup. AMF may send additional N2 SM information from SMFs in separate N2 message(s) (e.g. N2 tunnel setup request), if there is any. Alternatively, if multiple SMFs are involved, the AMF may send one N2 Request message to (R)AN after all the Nsmf_PDUSession_ UpdateSMContext Response service operations from all the SMFs associated with the UE are received. In such case, the N2 Request message includes the N2 SM information received in each of the Nsmf_PDUSession_ UpdateSMContext Response and information to enable AMF to associate responses to relevant SMF.

If the NG-RAN node had provided the list of recommended cells / TAs / NG-RAN node identifiers during the UE Context Release procedure in the AN (see clause 4.2.6), the AMF shall include it in the N2 Request. The RAN may use this information to allocate the RNA when the RAN decides to enable RRC Inactive state for the UE.

12.	(R)AN to UE: The RAN performs RRC Connection Reconfiguration with the UE depending on the QoS Information for all the QoS Flows of the PDU Sessions activated and Data Radio Bearers. The User Plane security is established at this step, which is described in detail in RAN specifications.

The RAN forwards the MM NAS Service Accept to the UE. The UE locally deletes context of PDU sessions that are not available in 5G CN.

If the N1 SM information is present in the Service Accept and indicates that the any PDU session needs to be re-established, the UE initiates PDU session re-establishment after Service Request procedure is complete. That is, for SSC mode 2, step 3 and step 4 is defined in clause 4.3.5.1.1 is performed. For SSC mode 3, step 3 and 4 defined in clause 4.3.5.2 is performed.

NOTE 3:	The reception of the Service Accept message does not imply the successful activation of the User Plane radio resources.

Editor's note:	When/how the UE is notified about the user plane activation for the successful completion of access network resource is FFS.

13.	After the User Plane radio resources are setup, the uplink data from the UE can now be forwarded to RAN. The 5G RAN sends the uplink data to the UPF address and Tunnel ID provided in the step 4.

14.	[Conditional] (R)AN to AMF: N2 Request Ack (N2 SM information (RAN Tunnel info, List of accepted QoS Flows for the PDU Sessions activated, List of rejected QoS Flows for the PDU Sessions activated). This step is described in detail in RAN specifications.

The message may include N2 SM information(s), e.g. RAN tunnel information. RAN may respond N2 SM information with separate N2 message (e.g. N2 tunnel setup response) if AMF sends separate N2 message in step 5.

If multiple N2 SM information are included in the N2 Request message in step 5, the N2 Request Ack includes multiple N2 SM information and information to enable the AMF to associate the responses to relevant SMF.

Editor's note:	Further detailing of the messages in N2 is FFS and is to be coordinated with RAN3.

15.	[Conditional] AMF to SMF: Nsmf_PDUSession_ UpdateSMContext Request (N2 SM information (RAN Tunnel info), RAT Type) per accepted PDU Session to the SMF.

If the AMF received N2 SM information (one or multiple) in step 8, then the AMF shall forward the N2 SM information to the relevant SMF. If the UE Time Zone has changed compared to the last reported UE Time Zone then the AMF shall include the UE Time Zone IE in this message.

Editor's note:	It is FFS how to handle the non-accepted QoS Flows.

Editor's note:	Procedure to pause charging by SMF is FFS.

16.	[Optional] SMF to PCF: If dynamic PCC is deployed, SMF may initiate IP-CAN Session Modification and provides new location information to the PCF.

See clause x.y.z for details.

17a.	[Conditional] SMF to new intermediate UPF: N4 Session Modification Request(RAN tunnel info).

If the SMF selected a new UPF to act as intermediate UPF for the PDU session in step 5, the SMF initiates a N4 Session Modification procedure and provides RAN Tunnel Info. See clause x.y.z for more details.

17b.	[Conditional] UPF to SMF: N4 Session Update Response.

See clause x.y.z for more details.

18.	[Conditional] SMF to AMF: Nsmf_PDUSession_ UpdateSMContext Response.

19a.	[Conditional] SMF to new UPF (intermediate): N4 Session Modification Request.

If forwarding tunnel has been established and if the timer SMF set for forwarding tunnel at step 8a has expired, SMF sends N4 Session modification request to new (intermediate) UPF acting as N3 terminating point to release the forwarding tunnel.

19b.	[Conditional] new UPF (intermediate) to SMF: N4 Session modification response.

New (intermediate) UPF acting as N3 terminating point sends N4 session modification response to SMF.

20a.	[Conditional] SMF to old intermediate UPF: N4 Session Modification Request or N4 Session Release Request

If the SMF decided to continue using the old intermediate UPF in step 5, the SMF sends an N4 Session Modification Request, providing (R)AN tunnel information.

If the SMF decided to select a new UPF to act as intermediate UPF in step 5, the SMF initiates resource release, after timer in step 6b expires, by sending an N4 Session Release Request (Release Cause) to the old intermediate UPF.

NOTE 4:	If there is no intermediate UPF, the SMF sends an N4 Session Modification Request to the PSA UPF, providing (R)AN tunnel information.

20b.	Old intermediate UPF to SMF: N4 Session Modification Response or N4 Session Release Response

The old UPF acknowledges with an N4 Session Modification Response or N4 Session Release Response message to confirm the modification or release of resources.

NOTE 5:	If there is no intermediate UPF, the PSA UPF sends an N4 Session Modification Response to the SMF to confirm the modification of resources.

*** End of 2nd Changes ***