SA WG2 Meeting #122	S2-174333

26 - 30 June 2017   San Jose Del Cabo, Mexico

Source:	Huawei

Title:	TS23.502: Clarifications on NWDA Service Based Interface

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This document proposes alignment between TS23.501 and TS23.502 on the definition of the reference point N23 as service-oriented interface exhibited by NWDA.

Discussion

TS 23.502 v0.4.0 section sec 5.2.11 lists and describes the services provided by NWDA. In that section, N23 is described as a service-oriented interface exhibited by NWDA. However, TS23.501 v0.5.0 only defines N23 as a reference point between NWDA and PCF.

In addition, TS 23.502 v0.4.0 section sec 5.2.11 defines the possibility of PCF to retrieve data analytics from NWDA via the N23_Request/Response services, hence not necessarily subscribing to NWDA events.

Editorial clarifications for TS23.502 v0.4.0 are proposed.

Proposal

The following (tracked) changes to TS 23.502 v0.4.0 are proposed.

***** START of 1st CHANGE *****

5.2.11	NWDA Services

Editor's note:`How NWDA services work while roaming is FFS.

5.2.11.1	Nnwda_Subscribe services

Service description: Subscribes to or Cancels subscription to NWDA event.

Table 5.2.11-1: Subscribed Services provided by NWDA

| Service Description | Parameters |
| --- | --- |
| Subscription or cancellation of subscription request for information on congestion level in a slice. Subscription request may also include whether the consumer/requester would like to be notified on a periodic basis or only if a congestion threshold is crossed. | Request/Cancel: Slice Congestion Information (SCI), Slice Information Editor's note: The Slice Information rather than the ID regarding to the SCI generation should be provided. The detailed information is FFS. Periodicity (optional), SCI Threshold value (optional) Notify: SCI value |

Editor's note:	For different Nnwda_subscription services, the detailed Slice Information could be different and the content is FFS.

Input: Subscription Request or Cancel Subscription Request. Includes parameters listed in Table 5.x.x-1 for the specific NWDA subscribed service being requested or cancelled.

Output: Confirmation to the Subscription or Cancel Subscription Request

Service Procedure:

Figure 5.2.11-1: NWDA Subscription service

1.	The consumer/requester  subscribes to or cancel subscription to an NWDA service with service specific parameters.

2.	The NWDA confirms subscription or cancellation of the requested event.

5.2.11.2	Nnwda_Notify services

Service description: NWDA notifies the consumer/requester instance of the event that has subscribed to the specific NWDA service. Depending upon type of subscription this notification is either on a periodic basis or triggered whenever a threshold is crossed.

Input: Earlier asynchronous subscription to the event

Output: Message Notify with parameter(s) to the consumer/requester  instance that had subscribed to the event

Service procedure:

Figure 5.2.11-1: Nnwda_Notify services

5.2.11.3	Nnwda_Request/Response services

Service description: Requests a NWDA service.

Table 5.2.11.3-1: Services provided by NWDA on Request

| Service Description | Parameters |
| --- | --- |
| Request for Operator specific Analytics. These represent operator specific analytics that have a meaning only in its network. . | Request: Analytic ID identifying the requested Operator specific Analytic Response: Requested Analytic |

Editor's note:	Format and details of Analytic ID is FFS.

Input: NWDA service request that includes parameters listed in Table 5.x.x-2 for the specific NWDA service being requested.

Output: NWDA service response with parameters indicated in Table 5.x.x-2 for the specific NWDA service that was responded

Service Procedure:

Figure 5.x.x-1: NWDA Subscription service

1.	The consumer/requester  requests an NWDA service with service specific parameters.

2.	The NWDA responds with service specific parameters.

***** END of 1st CHANGES *****