SA WG2 Meeting #122	S2-174632

26 - 30 May 2017, San Jose Del Cabo, Mexico	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.501: LADN to non-3GPP access

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution considers Non-3GPP aspects for LADN.

Discussion

We would like to discuss the access type of PDU session for LADN and propose to agree the followings.

First, LADN may apply to 3GPP access and non-3GPP access.

For the LADN service, the PDU session may be established based on DNN Selection Policy of UE Route Selection Policy (URSP).

It is noted that DNN selection policy may indicate the access type (3GPP or non-3GPP) on which a PDU session to a certain DNN should be requested

Second, the AMF determines the UE location based on access specific information and network policy.

For 3GPP access, the AMF determines the UE location based on 3GPP specific location information (e.g. TAI, Cell ID).

For Non-3GPP access,

- the AMF determines the UE location based on Non-3GPP specific location information (e.g. N3IWF node, UE local IP address and optionally UDP source port number, (B)SSID, etc.), or

- the AMF determines the UE location based on 3GPP specific location information as well as Non-3GPP specific location information.

That is, any non-3GPP specific location information may not provide the actual UE location, so based on network policy, the AMF determines whether using the 3GPP specific location information or not. If it is possible, the AMF determines if the UE is located in the service area of LADN. It depends on operator’s policy.

NOTE: In this case, RM/CM context change of 3GPP access may be notified from AMF to SMF (if it is subscribed by SMF) to help the SMF to handle the PDU session over non-3GPP access for LADN.

Third, the final decision how to handle the PDU session (e.g. keep the PDU session for LADN service or not) is charge of the SMF based on

- the information received from AMF (i.e. UE location related),

- the network policy

Proposal

* * * * Start of 1st Change * * * *

5.6.5	Support for advertising the availability of a local area data network

5G System shall provide support for the UEs to be made aware of the availability of a local area Data Network (LADN) based on the UE location.

When the UE performs a successful registration procedure, the AMF notifies the UE of the Availability information in the Registration Accept message, which indicates the specific LADNs that are available to the UE in the registration area, based on the local configuration information (e.g. via OAM), the operator's policy, and the UE subscription information.

LADN Availability Information provided to the UE by the AMF consists of LADN DNN and LADN service area information. The LADN service area information may be provided for a set of Tracking Areas. The AMF does not create Registration Area based on the availability of LADNs.

Editor's Note: additional levels of granularity are FFS.

Editor's note: other scenarios in which the UE is not configured with any LADNs and discovers the supported LADNs are FFS.

Based on the LADN Availability information in the UE, the UE may request a PDU session establishment for an available LADN wen the UE is located in the area of availability of the LADN. The UE should not request a PDU session for a LADN when the UE is located outside the area of availability of the LADN, and the SMF shall reject any such requests. The UE shall not trigger Service Request for the establishment of user plane for an LADN when the UE is located out of LADN service area, and the network shall reject such request.

A PDU session for LADN may applied over to a non-3GPP access based on DNN Selection Policy of UE Route Selection Policy (URSP).

The AMF determines the UE location based on access specific information and network policy. For 3GPP access, the AMF determines the UE location based on 3GPP specific location information (e.g. TAI, Cell ID). For Non-3GPP access,

- the AMF determines the UE location based on Non-3GPP specific location information (e.g. N3IWF node, UE local IP address and optionally UDP source port number, (B)SSID, etc.), or

- the AMF determines the UE location based on 3GPP specific location information as well as Non-3GPP specific location information.

That is, any non-3GPP specific location information may not provide the actual UE location, so based on network policy, the AMF determines whether using the 3GPP specific location information or not. If it is possible, the AMF determines if the UE is located in the service area of LADN. It depends on operator’s policy.

NOTE1: In this case, RM/CM context change of 3GPP access may be notified from AMF to SMF (if it is subscribed by SMF) to help the SMF to handle the PDU session over non-3GPP access for LADN.

When the SMF detects that the UE has moved out of the service area of an availability of the LADN based on information received from the AMF, the SMF may, based on network policies, either:

- Disconnect the PDU session, or

- release the user plane resources for the PDU session and maintain the PDU session. The network may at any time, based on network policies, disconnect the PDU session. The SMF may also notify the UPF to discard downlink data for the PDU sessions and/or not send out Data Notification message to the SMF.

Upon leaving the area of availability of the LADN, the UE shall consider the user plane for the LADN PDU session released, unless the UE receives an explicit PDU session release request from the network.

The network shall reject requests from the UE to establish user plane resources for a PDU session corresponding to an LADN when the UE is outside the area of availability of the LADN. This includes rejecting a service request in which the UE indicates only the PDU session corresponding to the LADN.

The SMF shall not trigger user plane re-establishment for a PDU session corresponding to an LADN if the SMF is aware that the UE is outside the area of availability of the LADN.

When the SMF detects that the UE has returned to the area of availability of an LADN based on information received from the AMF, if the SMF or UPF has pending DL data the SMF performs a Network Triggered Service Request to establish the User Plane(s) for the PDU sessions, otherwise the SMF will inform the UPF to resume sending DL data notifications to the SMF in case of DL data.

* * * * End of Changes * * * *