SA WG2 Meeting #122	S2-174640

26 - 30 May 2017, San Jose Del Cabo, Mexico	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.502: AMF Event Exposure service update (Namf_EventExposure service)

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution updates Namf_EventExposure service

Discussion

This paper considers AMF EventExposure service.

First, we propose to add the parameters of location changes for Non-3GPP access, e.g. the radio access specific information such as UE local IP address and optionally UDP source port number, and (B)SSID. It can be used for AMF EventExposure service.

Second, we propose to add the optional input or output per “access type” for AMF EventExposure service.

It is a goal to have the event notification per access type. For example, currently, AMF associates multiple access-specific RM context. So the update of registration state context for a UE can be notified per access type by AMF.

Proposal

* * * * Start of 1st Change * * * *

5.2.2.3	Namf_EventExposure service

5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about the following

-	Location changes (TAI, Cell ID, N3IWF node, UE local IP address and optionally UDP source port number, (B)SSID);

-	AN type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status along with optional list of sessions to be activated; and

-	Mobility statistics.

The following service operations are defined for the Namf_EventExposure service.

5.2.2.3.2	Namf_EventExposure_Subscribe service operation

Service operation name: Namf_EventExposure_Subscribe.

Description: The consumer NF uses this service operation to subscribe for a specific UE mobility information.

Known NF Consumers: NEF, SMF, PCF, UDM

Inputs, Required: NF ID, UE ID, Event Filter, Event co-relation ID.

Inputs, Optional: Immediate one time notification flag, Access type(s).

Outputs, Required: None.

Outputs, Optional: None.

See step 10 of clause 4.3.2.2.1 and step 14 of clause 4.3.2.2.2 for details on the usage of this service operation. This service operation can be invoked along with the Namf_Communication_N1MessageTransfer service operation invocation. The consumer NF subscribes to the event notification by invoking Namf_EventExposure_Subscribe (NF ID, UE ID, Event filter, Event co-relation ID) to the AMF. UE ID identifies the UE, a valid UE ID includes SUPI. Event filter identifies the events that the consumer NF is interested in. The following are some example events.

-	UE registration state change;

-	UE reachability state along with optional list of sessions to activate;

-	Location change; and

-	Periodic mobility statistics.

The event co-relation ID identifies the ID to be used by the AMF whenever it is notifying the subscribed events to the consumer NF. The event co-relation ID helps the consumer NF to co-relate a notification against a corresponding event subscription. If the UE mobility event notification subscription is authorized by the AMF, the AMF records the association of the event trigger and the consumer NF identity. If the optional, immediate one time notification flag is set, then the AMF notifies the current status of the subscribed event immediately to the consumer NF. Optionally, it indicates to subscribe the event per access type.

Editor's note:	The contents of the event filter and whether the event filter should have an optional parameter value pattern to match for triggering the events are FFS.

5.2.2.3.3	Namf_EventExposure_UnSubscribe service operation

Service operation name: Namf_ EventExposure_UnSubscribe.

Description: The consumer NF uses this service operation to unsubscribe for a specific UE mobility information.

Known NF Consumers: NEF, SMF, PCF

Inputs, Required: NF ID, UE ID, Event Filter, Event co-relation ID.

Inputs, Optional: Access type(s).

Outputs, Required: None.

Outputs, Optional: None.

The consumer NF unsubscribes the event notification by invoking Namf_EventExposure_Unsubscribe (NF ID, UE ID, Event filter, event co-relation ID) to the AMF. UE ID identifies the UE, a valid UE ID includes SUPI. The event co-relation ID identifies a prior subscription that the consumer NF has made. Event filter identifies the events that the consumer NF is no longer interested with on the specific notify ID. Optionally, it indicates to unsubscribe the event per access type.

5.2.2.3.4	Namf_EventExposure_Notify service operation

Service operation name: Namf_EventExposure_Notify.

Service operation description: Provides the UE mobility event to the Consumer NF which has subscribed to the UE mobility event before. The mobility event includes: Location change (serving cell change when UE is in CM-CONNECTED state, serving AMF change), CM state change.

Editor's note:	The full list of the events that can be reported by Event Report service will be defined later.

Input: UE ID, Event filter

Output: The consumer has been notified the UE mobility event.

Known NF Consumers: NEF, PCF, SMF

Inputs, Required: AMF ID, UE ID, Event co-relation ID, Mobility information.

Inputs, Optional: Event specific parameter list, Access type(s).

Outputs, Required: None.

Outputs, Optional: None.

See step 4a of clause 4.2.3.2 and step 3a of clause 4.2.3.3 for details on this service operation invocation.

When the AMF detects the UE mobility event it sends the Event Notification ( mobility event container) message to the Consumer NF which has subscribed to the UE mobility event before. The event co-relation ID indicates to the NF the specific event notification subscription made by the consumer NF. The mobility event container indicates the type of mobility event and related information, e.g. Registration Area Update/new Registration Area. Optionally, it indicates the event notification per access type.

Editor's note:	The contents of the event filter and whether the event filter should have an optional parameter value are FFS.

* * * * End of Changes * * * *