SA WG2 Meeting #122	S2-175046

26 - 30 June 2017, San Jose Del Cabo, Mexico                   	(revision of S2-174667)

Source:	CATT

Title:	TS 23.501: access technology being used by UE being one of slice selection parameters

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution specifies that when AMF triggers slice selection procedure, access technology being used by UE is provided as one of parameters for discovery and selection for the set of slices.

- Discussion

It is concluded in S1-172404, SA1 Meeting #78 as follows:

The 5G system shall allow the operator to assign a UE to a network slice, to move a UE from one network slice to another, and to remove a UE from a network slice based on subscription, UE capabilities, the access technology being used by the UE, operator's policies and services provided by the network slice.

The access technology being used by the UE is also took into account when the operator assigns a UE to a network slice.

Therefore, to achieve finer granularity of discovery and selection for the set of slices, it is proposed that when a UE is registering over an access technology, the access technology is provided by AMF as one of parameters for discovery and selection for the set of slices to identify the allowed S-NSSAI.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.501.

*************** First change *********************

5.15.2.1	General

An S-NSSAI (Single Network Slice Selection Assistance information) identifies a Network Slice.

An S-NSSAI is comprised of:

-	A Slice/Service type (SST), which refers to the expected Network Slice behaviour in terms of features and services;

-	A Slice Differentiator (SD). which is optional information that complements the Slice/Service type(s) to allow further differentiation for selecting an Network Slice instance from the potentially multiple Network Slice instances that all comply with the indicated Slice/Service type. This information is referred to as SD.

The S-NSSAI can have standard values or PLMN-specific values. S-NSSAIs with PLMN-specific values are associated to the PLMN ID of the PLMN that assigns it. An S-NSSAI shall not be used by the UE in access stratum procedures in any PLMN other than the one to which the S-NSSAI is associated.

Editor's note:	Whether a single value which is a representation of a collection of the S-NSSAIs could also be used as NSSAI is FFS.

The NSSAI is a collection of S-NSSAIs (Single Network Slice Selection Assistance Information). There can be at most 8 S-NSSAIs in the NSSAI sent in signalling messages between the UE and the Network. Each S-NSSAI assists the network in selecting a particular Network Slice instance.

The same Network Slice instance may be selected by means of different S-NSSAIs.

Based on the operator’s operational or deployment needs multiple Network Slice instances of a Network Slice may be deployed in the same or in different registration areas for the same S-NSSAI. Whenever a UE is associated with an S-NSSAI, it will be served by only one instance at a time out of any of the corresponding deployed multiple Network Slice instances.

The CN part of a Network Slice instance(s) serving a UE is selected by NSSF based on Requested NSSAI, the list of subscribed S-NSSAI(s), UE location information, the access technology being used by the UE, local configuration in the NSSF, etc..

The (R)AN may use Requested NSSAI in access stratum signalling to handle the UE Control Plane connection before the 5GC informs the (R)AN of the Allowed NSSAI. The Requested NSSAI is not used by the RAN for routing when the UE provides also a Temporary User ID.

Editor's note:	Whether (R)AN uses Requested NSSAI is to be checked with RAN WGs.

When a UE is successfully registered, the CN informs the (R)AN by providing the whole Allowed NSSAI for the Control Plane aspects.

Editor's note:	The need for informing the (R)AN by providing the whole Allowed NSSAI is to be checked with RAN WGs.

When a PDU session for a specific slice instance is established, the CN provides to the (R)AN the S-NSSAI corresponding to the slice instance that this PDU session belongs to enable the RAN to perform access specific functions.

NOTE:	The details of how the RAN uses NSSAI information is described in TS 38.xxx[x].

*************** End of changes *********************