SA WG2 Meeting #123	S2-176965

October 23 – 27, 2017, Ljubljana, Slovenia

Source:	OPPO

Title:	Access Control for CM-Inactive state

Document for:	Approval

Agenda Item:	6.5.7.5

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution involves access control mechanism for CM-Inactive state.

- Discussion

For idle state, apart from the EUTRAN system in which multiple types of access control exist, e.g. ACB, EAB, ACDC, SSAC, etc, a uniform access control mechanism will be introduced. In this uniform access control mechanism, NAS will provide AS the access category when UE initiates access attempt. And then AS layer performs access barring checking based on the AC parameters.

For MO case, when the UE is in CM-CONNECTED with Inactive state, since the NAS layer in UE has the knowledge about UE Inactive state, NAS will be involved and provide some resume information, e.g. resume cause, to RRC layer every time UE attempts to resume the RRC connection to the connected state due to the specified cause. With this NAS-AS model for CM-CONNECTED with Inactive state, NAS layer will be notified when UL data arrives and can provide the access category to RRC layer. Therefore, the AC mechanism in RRC_IDLE state can be applicable to RRC_INACTIVE state for MO case.

For MT case, there is no AC mechanism for MT case in EUTRAN, if it is assumed the principle is also applied in NR, no issues need to be addressed.

Proposal 1: The AC mechanism in RRC_IDLE state can be applicable to RRC_INACTIVE state for MO case.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.501.

*************** Start of changes *********************

##### 5.3.3.2.5	CM-CONNECTED with RRC Inactive state

RRC Inactive state applies to NG-RAN.

The AMF, based on network configuration may provide assistance information to the NG-RAN, to assist the NG-RAN's decision whether the UE can be sent to RRC Inactive state.

Editor's note: It is FFS if the UE provides indication of support for RRC inactive state on NAS or AS layer.

The "RRC Inactive assistance information" includes:

-	UE specific DRX values.

-	the Registration Area provided to the UE;

-	Periodic Registration Update timer

-	If the AMF has enabled MICO mode for the UE, an indication that the UE is in MICO mode.

The RRC Inactive assistance information mentioned above is provided by the AMF during N2 activation with the (new) serving NG-RAN node (i.e. during Registration, service request, handover) to assist the NG RAN's decision whether the UE can be sent to RRC Inactive state. RRC Inactive state is part of RRC state machine, and it is up to the RAN to determine the conditions to enter RRC Inactive state. If any of the parameters included in the RRC Inactive Assistance Information changes as the result of NAS procedure, the AMF shall update the RRC Inactive Assistance Information to the NG-RAN node.

When the UE is in CM-CONNECTED state, if the AMF has provided RRC Inactive assistance information, the RAN node may decide to move a UE to CM-CONNECTED with RRC Inactive state.

The state of the N2 and N3 reference points are not changed by the UE entering CM-CONNECTED with RRC Inactive state. A UE in RRC inactive state is aware of the RAN Notification area.

The 5GC network is not aware of the UE transitions between CM-CONNECTED with RRC Connected and CM-CONNECTED with RRC Inactive state, unless the 5GC network is notified via N2 notification procedure in TS 23.502 [3] clause 4.8.3.

At transition into CM-CONNECTED with RRC Inactive state, the UE applies with a periodic RAN Notification Area Update and the timer is restarted in the UE and in the RAN as defined in TS 38.300 [27].

When assigning the Periodic RAN Notification Area Update timer, the NG-RAN shall take into account the value of the Periodic Registration Update timer value indicated in the RRC Inactive Assistance Information.

If the periodic RAN Notification Area Update timer expires in RAN, the RAN shall start an RRC release timer. If the RRC release timer expires, the RAN shall initiate UE context release in the AN procedure as specified in TS 23.502 [3].

When the UE is in CM- CONNECTED with RRC inactive state, the UE performs PLMN selection procedures as defined in TS 23.122 [17] for CM-IDLE.

When the UE is CM-CONNECTED with RRC Inactive state, the UE may resume the RRC connection due to:

-	Uplink data pending;

-	Mobile initiated NAS signalling procedure;

-	As a response to RAN paging;

-	Notifying the network that it has left the RAN Notification area;

-	Upon periodic RAN update timer expiration.

If the UE resumes the connection in a different NG-RAN node within the same PLMN, the UE AS context is retrieved from the old NG-RAN node and a procedure is triggered towards the CN (see TS 23.502 [3]).

If the RAN paging procedure, as defined in TS 38.300 [27], is not successful in establishing contact with the UE the procedure shall be handled by the network as follows:

-	If NG-RAN has a pending NAS PDU for transmission, the RAN node shall initiate the NAS signaling connection release procedure (see TS 23.502 [3]) to move the UE CM state in the AMF to CM-IDLE state and indicate to the AMF the NAS non-delivery.

-	If NG RAN has only pending user plane data for transmission, the NG-RAN node may keep the N2 connection active or initiate the NAS signaling connection release procedure (see TS 23.502 [3]) based on local configuration in NG-RAN.

The access control barring mechanism in CM-IDLE state can be applied in CM-CONNECTED with Inactive state.

If a UE in CM-CONNECTED with RRC Inactive state performs cell selection to GERAN/UTRAN/EPS, it shall follow idle mode procedures.

In addition, a UE in CM-CONNECTED state with RRC Inactive state shall enter CM-IDLE mode and follow relevant NAS procedure in the following cases:

-	If RRC resume procedure fails,

If the UE receives Core Network paging,

-	If the periodic RAN Notification Area Update timer expires and the UE cannot successfully resume the RRC connection.

-	in any other failure scenario that cannot be resolved in RRC Inactive mode and requires the UE to move to CM-IDLE mode.

When UE is in CM-CONNECTED with RRC Inactive state, if RAN has received Location Reporting Control message from AMF with the reporting type indicating single stand-alone report, the RAN shall perform RAN paging before reporting the location to AMF.

When UE is in CM-CONNECTED with RRC Inactive state, if RAN has received Location Reporting Control message from AMF with the reporting type indicating continuously reporting whenever the UE changes cell, the RAN shall send a Location Report message to AMF including UE's last known location with time stamp.

*************** End of changes *********************