SA WG2 Meeting #123	S2-176982

23 – 27 October 2017, Ljubljana, Slovenia	(revision of S2-17xxxx)

Source:	Ericsson

Title:	23.501 §5.15: Clarification of slice availability in a PLMN

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This paper relates to OI#4c and clarifies the availability of network slices within a PLMN and the relation to the Registration Area.

# 1 Discussion

Note: this pCR propose changes in 23.501. The companion pCR S2-176983 addresses the related changes in 23.502.

It has been discussed and agreed that not all network slices are available within the whole PLMN, but at least the RA shall be homogeneous with regards to support for network slices.

NOTE:	In this release of the specification it is assumed that the registration area allocated by the AMF to the UE shall have homogeneous support for network slices.

Observation 1: In a note in 5.15.5.2.2 it is stated that: It is possible to have not homogenous support of network slices in the RAs of a PLMN. In this Release at least, the network slice support is assumed to be homogenous within a RA.

Observation 2: The possibility of heterogenous support of network slices in a RA in a PLMN is based on the assumption that network slices may not be available across the PLMN, but this is not explicitly stated in 23.501.

Observation 3: No mechanisms has been discussed to handle the case of when a PLMN is heterogenous with regards to support of network slices.

Proposal 1: Clarify the heterogeneous support of network slices within a PLMN and make the existing note about homogenous support of network slicing within a RA normative text and move such basic statement from clause "5.15.5.2.2 Modification of the Set of Network Slice(s) for a UE".

To enable 5GC to ensure homogenous support of network slices within a RA, then 5GC needs to setup RA consisting of  one or more TA(s) with same support for network slices.

At least two conditions could lead to heterogeneous support of network slices:

1.	The operational needs lead to a certain deployment of the network slices, e.g. connectivity to a network slice is only available in a certain area or a functionality required for a network slice is not supported by a NF;

2.	The business logic enforces availability e.g. by SLA it is agreed to only allow access to a network slice in a certain area or time of day etc.

Observation 4: There is a need to enable awareness whether a network slice is available at a certain area, and the information should be possible to update based on operator policies.

Proposal 2: The NSSF should be the NF which knows when and where the network slices are available based on SLAs e.g. time of day etc.

As discussed in S2-176983, the AMF being able to allocate the RA is the selected target AMF.

Proposal 3: We propose an alignement of the normative text on the role of the NSSF.

# 2 Summary and Proposal

Below follow the changes that are proposed to TS 23.501 v.1.4

***** 1st Change *****

### 5.15.2	Identification and selection of a Network Slice: The S-NSSAI and the NSSAI

#### 5.15.2.1	General

An S-NSSAI identifies a Network Slice.

An S-NSSAI is comprised of:

-	A Slice/Service type (SST), which refers to the expected Network Slice behaviour in terms of features and services;

-	A Slice Differentiator (SD), which is optional information that complements the Slice/Service type(s) to differentiate amongst multiple Network Slices of the same Slice/Service type.

The S-NSSAI can have standard values or PLMN-specific values. S-NSSAIs with PLMN-specific values are associated to the PLMN ID of the PLMN that assigns it. An S-NSSAI shall not be used by the UE in access stratum procedures in any PLMN other than the one to which the S-NSSAI is associated.

The NSSAI is a collection of S-NSSAIs. There can be at most 8 S-NSSAIs in the NSSAI sent in signalling messages between the UE and the Network. Each S-NSSAI assists the network in selecting a particular Network Slice instance.

The same Network Slice instance may be selected by means of different S-NSSAIs.

Based on the operator's operational or deployment needs, multiple Network Slice instances of a given S-NSSAI may be deployed in the same or in different Tracking Areas. When multiple Network Slice instances of a given S-NSSAI are deployed in the same Tracking Area, the AMF instance serving the UE may logically belong to more than one Network Slice instances of that S-NSSAI, i.e. this AMF instance may be common to multiple Network Slice instances of that S-NSSAI. When a S-NSSAI is supported by more than one Network Slice instance in a PLMN, any of the Network Slice instances supporting the same S-NSSAI in a certain area may serve, as a result of the Network Slice instance selection procedure defined in clause 5.15.5, a UE which is allowed to use this S-NSSAI. Upon association with an S-NSSAI, the UE is served by the same Network Slice instance for that S-NSSAI until cases occur where e.g. Network Slice instance is no longer valid in a given registration area, or a change in UE's Allowed NSSAI occurs etc. In such cases, procedures mentioned in clause 5.15.5.2.2 or clause 5.15.5.2.3 applies.

The selection of a Network Slice instance(s) serving a UE and the Core Network Control Plane and user plane Network Functions corresponding to the Network Slice instance is the responsibility of 5GC.

The (R)AN may use Requested NSSAI in access stratum signalling to handle the UE Control Plane connection before the 5GC informs the (R)AN of the Allowed NSSAI. The Requested NSSAI is not used by the RAN for routing when the UE provides also a Temporary User ID.

When a UE is successfully registered, the CN informs the (R)AN by providing the whole Allowed NSSAI for the Control Plane aspects.

Editor's note:	The need for informing the (R)AN by providing the whole Allowed NSSAI is to be checked with RAN WGs.

When a PDU Session for a given S-NSSAI is established using a specific Network Slice instance, the CN provides to the (R)AN the S-NSSAI corresponding to this Network Slice instance to enable the RAN to perform access specific functions.

NOTE:	The details of how the RAN uses NSSAI information are described in TS 38.300 [27].

#### 5.15.2.2	Standardised SST values

Standardized SST values provide a way for establishing global interoperability for slicing so that PLMNs can support the roaming use case more efficiently for the most commonly used Slice/Service Types.

The SSTs which are standardised are in the following Table 5.15.2.2-1.

Table 5.15.2.2-1 - Standardised SST values

| Slice/Service type | SST value | Characteristics. |
| --- | --- | --- |
| eMBB (enhanced Mobile Broadband) | 1 | Slice suitable for the handling of 5G enhanced Mobile broadband, useful, but not limited to the general consumer space mobile broadband applications including streaming of High Quality Video, Fast large file transfers etc. It is expected this SST to aim at supporting High data rates and high traffic densities as outlined in Table 7.1-1 "Performance requirements for high data rate and traffic density scenarios" in TS 22.261 [2] |
| URLLC (ultra- reliable low latency communications) | 2 | Supporting ultra-reliable low latency communications for applications including, industrial automation, (remote) control systems. This SST is expected to aim at supporting the requirements in Table 7.2.2-1 "Performance requirements for low-latency and high-reliability services." in TS 22.261 [2] related to high reliability and low latency scenarios |
| MIoT (massive IoT) | 3 | Allowing the support of a large number and high density of IoT devices efficiently and cost effectively. |

NOTE:	The support of all standardised SST values is not required in a PLMN.

### 5.15.3	Subscription aspects

Subscription information may contain multiple S-NSSAIs. One or more of the Subscribed S-NSSAIs can be marked as default S-NSSAI. At most eight S-NSSAIs can be marked as default S-NSSAI. However, the UE may subscribe to more than eight S-NSSAIs. If an S-NSSAI is marked as default, then the network is expected to serve the UE with the related Network Slice when the UE does not send any valid S-NSSAI to the network in a Registration Request message.

Subscription Information for each S-NSSAI may contain multiple DNNs and one default DNN.

The NSSAI the UE provides in the Registration Request is verified against the user's subscription data.

### 5.15.4	UE NSSAI configuration and NSSAI storage aspects

A UE can be configured by the HPLMN with a Configured NSSAI per PLMN. A Configured NSSAI can be PLMN-specific and the HPLMN indicates to what PLMN(s) each Configured NSSAI applies, including whether the Configured NSSAI applies to all PLMNs, i. e. the Configured NSSAI conveys the same information regardless of the PLMN the UE is accessing (e.g. this could be possible for NSSAIs containing only standardized S-NSSAIs). When providing a Requested NSSAI to the network upon registration, the UE in a given PLMN shall only use S-NSSAIs belonging to the Configured NSSAI, if any, of that PLMN. Upon successful completion of a UE's Registration procedure, the UE may obtain from the AMF an Allowed NSSAI for this PLMN, which may include one or more S-NSSAIs. These S-NSSAIs are valid for the current Registration Area provided by the serving AMF the UE has registered with and can be used simultaneously by the UE (up to the maximum number of simultaneous Network Slices or PDU sessions). The UE may also obtain from the AMF one or more temporarily or permanently rejected S-NSSAIs.

The Allowed NSSAI shall take precedence over the Configured NSSAI for this PLMN. The UE shall use only the S-NSSAI(s) in the Allowed NSSAI corresponding to a Network Slice for the subsequent procedures in the serving PLMN, as described in clause 5.15.5.

The UE stores (S)NSSAIs as follows:

-	When the UE is provisioned with a Configured NSSAI for a PLMN in the UE, the Configured NSSAI shall be stored in the UE until a new Configured NSSAI for this PLMN is provisioned in the UE by the HPLMN:

-	When provisioned with a new Configured NSSAI for a PLMN, the UE shall both replace any stored Configured NSSAI for this PLMN with the new Configured NSSAI, and delete any stored Allowed NSSAI and rejected S-NSSAI for this PLMN;

-	If received, the Allowed NSSAI for a PLMN shall be stored in the UE, including when the UE is turned off, until a new Allowed NSSAI for this PLMN is received;

-	When a new Allowed NSSAI for a PLMN is received, the UE shall replace any stored Allowed NSSAI for this PLMN with this new Allowed NSSAI;

-	If received, a temporarily rejected S-NSSAI for a PLMN shall be stored in the UE while RM-REGISTERED;

-	If received, a permanently rejected S-NSSAI for a PLMN shall be stored in the UE while RM-REGISTERED.

Editor's note:	Additional conditions when rejected S-NSSAIs can be removed is FFS.

One or multiple of the S-NSSAIs in the Allowed NSSAI provided to the UE can have non-standardized values, which may not be a part of the UE's NSSAI configuration. In such cases, the Allowed NSSAI includes mapping information how the S-NSSAIs in the Allowed S-NSSAI correspond to S-NSSAI(s) in the Configured NSSAI in the UE. The UE uses this mapping information for its internal operation (e.g., finding an appropriate network slice for UE's services). Specifically, a UE application, which is associated with an S-NSSAI as per NSSP, is further associated with the corresponding S-NSSAI from the Allowed NSSAI.

### 5.15.x	Network Slice Configuration in PLMN

A Network Slice instance may be available in the whole PLMN or in one or more Tracking Areas of the PLMN. The Registration Area allocated by the AMF to a UE shall have homogeneous support for Network Slices.

The availability of a Network Slice per TA is derived by intersecting the S-NSSAI supported per TA in NG-RAN, the S-NSSAI supported in the AMF and the operator business logic per TA (based on SLAs, e.g. time of day, location) in the NSSF. This information is available per TA in NG-RAN, AMF and NSSF.

### 5.15.5	Detailed Operation Overview

#### 5.15.5.1	General

The establishment of User Plane connectivity to a Data Network via a Network Slice instance(s) comprises two steps:

-	performing a RM procedure to select an AMF that supports the required Network Slices.

-	establishing one or more PDU session to the required Data network via the Network Slice Instance(s).

#### 5.15.5.2	Selection of a Serving AMF supporting the Network Slices

##### 5.15.5.2.1	Registration to a set of Network Slices

When a UE registers with a PLMN, if the UE for this PLMN has a Configured NSSAI or an Allowed NSSAI, the UE shall provide to the network in RRC and NAS layer a Requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wishes to register, in addition to the Temporary User ID if one was assigned to the UE.

The Requested NSSAI may be either:

-	the Configured-NSSAI, or a subset thereof as described below, if the UE has no Allowed NSSAI for the serving PLMN; or

-	the Allowed-NSSAI, or a subset thereof as described below, if the UE has an Allowed NSSAI for the serving PLMN; or

-	the Allowed-NSSAI, or a subset thereof as described below, plus one or more S-NSSAIs from the Configured-NSSAI for which no corresponding S-NSSAI is present in the Allowed NSSAI and that were not previously permanently rejected (as defined below) by the network.

The subset of Configured-NSSAI provided in the Requested NSSAI consists of one or more S-NSSAI(s) in the Configured NSSAI applicable to this PLMN, if the S-NSSAI was not previously permanently rejected (as defined below) by the network, or was not previously added by the UE in a Requested NSSAI.

The subset of Allowed NSSAI provided in the Requested NSSAI consists of one or more S-NSSAI(s) in the last Allowed NSSAI for this PLMN.

The UE may provide in the Requested NSSAI an S-NSSAI from the Configured NSSAI that the UE previously provided to the serving PLMN in the present Registration Area if the S-NSSAI was not previously permanently rejected (as defined below) by the network.

The UE shall include the Requested NSSAI at RRC Connection Establishment and in NAS messages. The RAN shall route the NAS signalling between this UE and an AMF selected using the Requested NSSAI obtained during RRC Connection Establishment. If the RAN is unable to select an AMF based on the Requested NSSAI, it routes the NAS signalling to an AMF from a set of default AMFs.

When a UE registers with a PLMN, if for this PLMN the UE has no Configured NSSAI or Allowed NSSAI, the RAN shall route all NAS signalling from/to this UE to/from a default AMF. The UE shall not indicate any NSSAI in RRC Connection Establishment or Initial NAS message unless it has a Configured NSSAI or Allowed NSSAI for the corresponding PLMN. When receiving from the UE a Requested NSSAI and a 5G-S-TMSI in RRC, if the RAN can reach an AMF corresponding to the 5G-S-TMSI, then RAN forwards the request to this AMF. Otherwise, the RAN selects a suitable AMF based on the Requested NSSAI provided by the UE and forwards the request to the selected AMF. If the RAN is not able to select an AMF based on the Requested NSSAI, then the request is sent to a default AMF.

When the AMF selected by the AN receives the UE Initial Registration request:

-	As part of the registration procedure described in TS 23.502 [3], clause 4.2.2.2.2, the AMF may query the UDM to retrieve UE subscription information including the Subscribed S-NSSAIs.

-	The AMF verifies whether the S-NSSAI(s) in the Requested NSSAI are permitted based on the Subscribed S-NSSAIs.

-	When the UE context in the AMF does not yet include an Allowed NSSAI, the AMF queries the NSSF (see (B) below for subsequent handling), except in the case when, based on configuration in this AMF, the AMF is allowed to determine whether it can serve the UE (see (A) below for subsequent handling).

NOTE 1:	The configuration depends on operator's policy.

-	When the UE context in the AMF already includes an Allowed NSSAI, based on configuration for this AMF, the AMF may be allowed to determine whether it can serve the UE (see (A) below for subsequent handling).

NOTE 2:	The configuration depends on the operator's policy.

(A) Depending on fulfilling the configuration as described above, the AMF may be allowed to determine whether it can serve the UE, and the following is performed:

-	AMF checks whether it can serve all the S-NSSAI(s) from the Requested NSSAI present in the Subscribed S-NSSAIs, or all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs in case no Requested NSSAI was provided (see clause 5.15.3).

-	If this is the case, the AMF remains the serving AMF for the UE. The Allowed NSSAI is then composed of the list of S-NSSAI(s) in the Requested NSSAI permitted based on the Subscribed S-NSSAIs, or, if no Requested NSSAI was provided, all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs (see (C) below for subsequent handling).

-	If this is not the case, the AMF queries the NSSF (see (B) below).

(B) When required as described above, the AMF needs to query the NSSF, and the following is performed:

-	The AMF queries the NSSF, with Requested NSSAI, the Subscribed S-NSSAIs, PLMN ID of the SUPI, location information, and possibly access technology being used by the UE.

-	Based on this information, local configuration, and other locally available information including RAN capabilities in the current Tracking Area for the UE, the NSSF does the following:

-	It selects the Network Slice instance(s) to serve the UE. When multiple Network Slice instances in the registration area are able to serve a given S-NSSAI, based on operator's configuration, the NSSF may select one of them to serve the UE, or the NSSF may defer the selection of the Network Slice instance until a NF/service within the Network Slice instance needs to be selected.

-	It determines the target AMF Set to be used to serve the UE, or, based on configuration, the list of candidate AMF(s), possibly after querying the NRF.

-	It determines the S-NSSAI(s) allowed, taking also into account the availability of the Network Slice instances that are able to serve the S-NSSAI(s) allowedI in the current Tracking Area.

-	Based on operator configuration, the NSSF may determine the NRF(s) to be used to select NFs/services within the selected Network Slice instance(s).

-	Additional processing to determine the Allowed NSSAI in roaming scenarios, as described in clause 5.15.6.

-	The NSSF returns to the current AMF the Allowed NSSAI and the target AMF Set, or, based on configuration, the list of candidate AMF(s). The NSSF may return the NRF(s) to be used to select NFs/services within the selected Network Slice instance(s). It may return also information regarding rejection causes for S-NSSAI(s) not included in the Allowed NSSAI which were part of the Requested NSSAI.

-	Depending on the available information and based on configuration, the AMF may query the NRF with the target AMF Set. The NRF returns a list of candidate AMFs.

Editor's note:	If the current serving AMF is not part of the target AMF Set, which NRF is to be queried to obtain the list of candidate AMFs is FFS.

-	If rerouting to a target serving AMF is necessary, the current AMF reroutes the Registration Request to a target serving AMF as described in clause 5.15.5.2.3.

(C) The serving AMF shall return to the UE the Allowed NSSAI. It may also indicate to the UE for Requested S-NSSAI(s) not included in the Allowed NSSAI, whether the rejection is permanent (e.g. the S-NSSAI is not supported in the PLMN) or temporary (e.g. the S-NSSAI is not currently available in the Registration Area).

Upon successful Registration, the UE is provided with a 5G-S-TMSI by the serving AMF. The UE shall include this 5G-S-TMSI in any RRC Connection Establishment during subsequent initial accesses to enable the RAN to route the NAS signalling between the UE and the appropriate AMF.

If the UE receives an Allowed NSSAI from the serving AMF, it shall store this new Allowed NSSAI and override any previously stored Allowed NSSAI for this PLMN, as described in clause 5.15.4.

##### 5.15.5.2.2	Modification of the Set of Network Slice(s) for a UE

The set of Network Slices for a UE can be changed at any time while the UE is registered with a network, and may be initiated by the network, or the UE under certain conditions as described below.

The network, based on local policies, subscription changes and/or UE mobility, operational reasons (e.g. a Network Slice instance is no longer available), may change the set of Network Slice(s) to which the UE is registered and provides the UE new Allowed NSSAI. The network may perform such change during a Registration procedure or trigger a notification towards the UE of the change of the Network Slices using a Generic UE Configuration Update procedure as specified in TS 23.502 [3], clause 4.2.4. The new Allowed NSSAI is determined as described in clause 5.15.5.2.1 (an AMF Relocation may be needed). The AMF provides the UE with the new Allowed NSSAI and TAI list, and:

-	If the changes to the Allowed NSSAI do not require the UE to perform a registration procedure:

-	The AMF indicates that acknowledgement is required, but does not indicate the need to perform a registration procedure;

-	The UE responds with a UE configuration update complete message for the acknowledgement.

-	If the changes to the Allowed NSSAI require the UE to perform a registration procedure (e.g. the new S-NSSAIs require a separate AMF that cannot be determined by the current serving AMF):

-	The serving AMF indicates to the UE that current 5G-GUTI is invalid and the need for the UE to perform a registration procedure after entering CM-IDLE state. The AMF shall release the NAS signalling connection to the UE to allow to enter CM-IDLE based on local policies (e.g. immediately or delayed release);

-	The UE initiates a registration procedure after the UE enters CM-IDLE state. The UE shall include SUPI and new Allowed NSSAI in the registration in this case.

When a Network Slice used for a one or multiple PDU Sessions becomes no longer available for a UE, in addition to sending the new Allowed NSSAI to the UE, the following applies:

-	In the network, if the Network Slice becomes no longer available under the same AMF (e.g. due to UE subscription change), the AMF indicates to the SMF(s) corresponding to the relevant S-NSSAI to autonomously release the UE's SM context

-	In the network, if the Network Slice becomes no longer available with AMF relocation (e.g. due to Registration Area change), the new AMF indicates to the old AMF that the PDU Session(s) associated with the relevant S-NSSAI shall be released. The old AMF informs the corresponding SMF(s) to autonomously release the UE's SM context.

-	In the UE, the PDU session(s) context is implicitly released after receiving the Allowed NSSAI in the Registration Accept message.

The UE uses UE Configuration (e.g. NSSP) to determine whether ongoing traffic can be routed over existing PDU sessions belonging to other Network Slices or establish new PDU session(s) associated with same/other Network Slice.

In order to change the set of S-NSSAIs being used, the UE shall initiate a Registration procedure as specified in clause 5.15.5.2.1.1.

Change of set of S-NSSAIs to which the UE is registered (whether UE or Network initiated) may lead to AMF change subject to operator policy, as described in clause 5.15.5.2.1.

Editor's note:	The condition under which the UE is able to request the change of the Network Slices, and what it is able to request, are FFS.

##### 5.15.5.2.3	AMF Relocation due to Network Slice(s) Support

During a Registration procedure in a PLMN, in case the network decides that the UE should be served by a different AMF based on Network Slice(s) aspects, then the AMF that first received the Registration Request shall redirect the Registration request to another AMF via the RAN or via direct signalling between the initial AMF and the target AMF. The redirection message sent by the AMF via the RAN shall include information for selection of a new AMF to serve the UE.

For a UE that is already registered, the system shall support a redirection initiated by the network of a UE from its serving AMF to a target AMF due to Network Slice(s) considerations (e.g. the operator has changed the mapping between the Network Slice instances and their respective serving AMF(s)). Operator policy determines whether redirection between AMFs is allowed.

#### 5.15.5.3	Establishing connectivity PDU session to the required Network Slice Instance(s)

The establishment of a PDU session in a Network Slice to a DN allows data transmission in a Network Slice. A Data Network is associated to an S-NSSAI and a DNN.

The network operator (HPLMN) may provision the UE with Network Slice selection policy (NSSP). The NSSP includes one or more NSSP rules each one associating an application with a certain S-NSSAI. A default rule which matches all applications to a S-NSSAI may also be included. When a UE application associated with a specific S-NSSAI requests data transmission, then:

-	If the UE has one or more PDU sessions established corresponding to the specific S-NSSAI, the UE routes the user data of this application in one of these PDU sessions, unless other conditions in the UE prohibit the use of these PDU sessions. If the application provides a DNN, then the UE considers also this DNN to determine which PDU session to use.

The UE shall store the NSSP until a new NSSP is provided to the UE by the HPLMN.

If the UE does not have a PDU session established with this specific S-NSSAI, the UE requests a new PDU session corresponding to this S-NSSAI and with the DNN that may be provided by the application. In order for the RAN to select a proper resource for supporting network slicing in the RAN, RAN needs to be aware of the Network Slices used by the UE.

If a Network Slice instance was not selected during the Registration procedure for this specific S-NSSAI, the AMF may query the NSSF with this specific S-NSSAI, location information, PLMN ID of the SUPI to select the Network Slice instance to serve the UE and to determine the NRF to be used to select NFs/services within the selected Network Slice instance.

The AMF queries the NRF to select an SMF in a Network Slice instance based on S-NSSAI, DNN and other information e.g. UE subscription and local operator policies, when the UE triggers the establishment of a PDU session. The selected SMF establishes a PDU session based on S-NSSAI and DNN.

When the AMF belongs to multiple Network Slices, based on configuration, the AMF may use an NRF at the appropriate level for the SMF selection.

#### 5.15.5.4	Slice Privacy Considerations

Editor's note:	This clause is to be considered Void till SA3 state they explicitly require omission of NSSAI from RRC layer for privacy reasons and not just as their TR 33.899 agreement "The NSSAI shall be confidentiality protected whenever NAS security context is available (as far as regulation allows)."

In order to support network-controlled privacy of slice information for the slices the UE accesses, when the UE is aware or configured that privacy considerations apply to NSSAI:

-	The UE shall not include NSSAI in NAS signalling unless the UE has a NAS security context.

-	The UE shall not include NSSAI in unprotected RRC signalling.

Editor's note:	It is FFS how the UE is aware or configured that the network has privacy considerations for NSSAI information.

Editor's note:	It is FFS whether considering slice privacy has impact on the allocation and management of the 5G GUTI and whether security considerations need to be studied by SA WG3.

### 5.15.6	Network Slicing Support for Roaming

For roaming scenarios:

-	If the UE only uses Standard S-NSSAI values, then the same S-NSSAI values are used in V-PLMN and as in the HPLMN.

-	If the UE is allowed to use non-standard S-NSSAI values in the V-PLMN, the V-PLMN maps H-PLMN Subscribed S-NSSAIs values that can be used in the VPLMN to respective S-NSSAI values to be used in the VPLMN.

-	In RRC layer, the NSSAI, if provided, uses values valid in the VPLMN.

-	In Registration Request, the Requested NSSAI, if provided, shall include only the S-NSSAI values that are valid in the VPLMN.

-	When the NSSF in the VPLMN determines the S-NSSAI(s) allowed, it may take into account the availability only of the VPLMN part of the Network Slice instances that are able to serve the UE in the current Tracking Area.

-	The Allowed NSSAI in the Registration Accept includes S-NSSAI values that are valid in VPLMN.

-	In a Session management procedures, the UE includes the S-NSSAI value valid in the VPLMN as they are received in the Allowed NSSAI in the VPLMN. The VPLMN maps any VPLMN specific S-NSSAI values to HPLMN specific S-NSSAI values before forwarding SM messages to the HPLMN.

-	The Network Slice specific network functions in the VPLMN are selected by the VPLMN by using the VPLMN S-NSSAI values and querying the NRF either pre-configured or provided by the NSSF in the VPLMN. The Network Slice specific functions in the HPLMN (if applicable) are selected by using the HPLMN S-NSSAI via the support from the NRF in the HPLMN.

NOTE:	In this release of specification it is assumed that in any PLMN (home or visited), for all the S-NSSAIs that the PLMN can serve it is always possible to select an AMF that can serve a Requested NSSAI that contains only S-NSSAIs that can coexist each other.

Editor's note: The details of the procedure to select specific functions in the HPLMN minimizing the interactions between VPLMN and HPLMN and the exposure of HPLMN topology are FFS.

***** End of 1st Change *****