# SA WG2 Meeting #S2-123	S2-177118

# 23 - 27 October 2017, Ljubljana, Slovenia

3GPP TSG RAN WG3 Meeting #97bis	R3-174226

Prague, Czech Republic, 09 – 13 October 2017

Title:	LS on the need for EPS Bearer ID knowledge in NG-RAN for inter-system handover from 5GS to EPS

Release:	Rel-15

Work Item:	5GS_Ph1

Source:	RAN3

To:	SA2

Cc:	CT4

Contact person:

Name:			Lixiang Xu

E-mail Address:	lx.xu@samsung.com

Attachments:

# 1	Overall description

RAN3 has studied thoroughly the indirect data forwarding for inter system handover from 5GS to EPS and come to the following conclusions:

- Only indirect data forwarding is supported.

- PDU session information at the serving NG-RAN node contains mapping information per QoS Flow to a corresponding E-RAB.

- At handover preparation, the source NG-RAN node shall decide which mapped E-RABs are proposed to be subject to data forwarding and provide this information in the source-to-target container to the target eNB.

- The target eNB assigns forwarding TEID/TNL address(es) for the E-RAB(s) for which it accepts data forwarding.

- A single data forwarding tunnel is established between the source NG-RAN node and UPF per PDU session for which at least data for a single QoS Flow is subject to data forwarding. Then the UPF maps data received from the per PDU session data forwarding tunnel(s) to the mapped EPS bearer(s).

Therefore, RAN3 would like to kindly update SA2 that the NG-RAN needs to be aware of the mapped E-RAB IDs of the Qos flows to be configured before initiating the inter-system handover preparation procedure for handover to EPS.

# 2	Actions

To SA2:

ACTION: RAN3 kindly asks SA2 to take above into account and update stage 2 specifications if needed.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#98	27th November – 1st December 2017	Reno, USA

RAN3#Adhoc	22nd – 26th January 2018		Sophia Antipolis, France