SA WG2 Meeting #123	S2-177649

23-27 October 2017, Ljubljana 	revision of S2-177380

Source:	Huawei, HiSilicon

Title:	TS 23.502 Update of 5GS to EPS handover using N26 interface procedure

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to establish indirect data forwarding tunnel procedure during UE handover 5GS to EPS.

Discussion

Proposal

It is proposed to update TS 23.502 as follows:

*************** Start of the changes ***************

4.11	System interworking procedures with EPS

4.11.1	N26 based Interworking Procedures

4.11.1.1	General

N26 interface is used to provide seamless session continuity for single registration mode.

4.11.2	Handover procedures for single-registration mode

4.11.2.1	5GS to EPS handover using N26 interface

Figure 4.11.2.1-1 describes the handover procedure from 5GS to EPS when N26 is supported.

During the handover procedure, as specified in clause 4.9.1.2.1, the source AMF shall reject any SMF+PGW-C initiated N2 request received since handover procedure started and shall include an indication that the request has been temporarily rejected due to handover procedure in progress.

Upon reception of a rejection for a SMF+PGW-C initiated N2 request(s) with an indication that the request has been temporarily rejected due to handover procedure in progress, the SMF+PGW-C behaves as specified in clause 4.9.1.2.1.

# Figure 4.11.2.1-1: 5GS to EPS handover for single-registration mode with N26 interface

The procedure involves a handover to EPC and setup of default EPS bearer and dedicated bearers for GBR QoS flows in EPC in steps 1-16 and re-activation, if required, of dedicated EPS bearers for non-GBR QoS flows in step 17. This procedure can be triggered, for example, due to new radio conditions, load balancing or due to specific service e.g. in the presence of QoS Flow for voice, the source NG-RAN node may trigger handover to EPC.

UE has one or more ongoing PDU Sessions each including one or more QoS flows. During PDU Session establishment and GBR QoS flow establishment, PGW-C+SMF performs EPS QoS mappings and allocates TFT with the PCC rules obtained from the PCF+PCRF if PCC is deployed, otherwise EPS QoS mappings and TFT allocation are executed by the PGW-C+SMF locally. EPS Bearer IDs are allocated by the serving AMF requested by the SMF if the SMF determines that EPS bearer ID(s) needs to be assigned to the QoS flow(s). For each PDU Session, EPS bearer ID(s) are allocated to the default EPS bearer which non GRB flows are mapped to, and allocated to dedicated bearers which the GBR flows that are mapped to in EPC. The EPS Bearer ID(s) for these bearer are provided to the UE, NG RAN and PGW-C+SMF by AMF. The UE is also provided with the mapped QoS parameters. The mapped EPS QoS parameters may be provided to PGW-C+SMF by the PCF+PCRF, if PCC is deployed.

1.	NG-RAN decides that the UE should be handed over to the E-UTRAN. The NG-RAN sends a Handover Required (Target eNB ID, Source to Target Transparent Container) message to the AMF. NG-RAN indicates bearers for data forwarding in Source to Target Transparent Container.

2.	The AMF determines from the 'Target eNB Identifier' IE that the type of handover is Handover to E-UTRAN. The AMF requests the PGW-C+SMF to provide SM Context that also includes the mapped EPS Bearer Contexts. This step is performed with all PGW-C+SMFs allocated to the UE.

Editor's note:	This step should be aligned with intra 5GC inter-AMF handover.

NOTE:	In roaming scenario, the UE's SM EPS Contexts are obtained from the V-SMF.

3.	The AMF selects an MME and sends a Relocation Request (Target E-UTRAN Node ID, Source to Target Transparent Container, mapped MM and SM EPS UE Context (default and dedicated GBR bearers)) message. The SGW address and TEID for both the control-plane or EPS bearers in the message are such that target MME selects a new SGW.

4.	The MME selects the Serving GW and sends a Create Session Request message for each PDN connection to the Serving GW.

5.	The Serving GW allocates its local resources and returns them in a Create Session Response message to the MME.

6.	The MME requests the target eNodeB to establish the bearer(s) by sending the message Handover Request message. This message also contains a list of EPS Bearer IDs that need to be setup.

7.	The target eNB allocates the requested resources and returns the applicable parameters to the target MME in the message Handover Request Acknowledge (Target to Source Transparent Container, EPS Bearers setup list, EPS Bearers failed to setup list). The EPS Bearer Setup list includes a list of addresses and TEIDs allocated at the target eNB for downlink traffic on S1-U reference point (one TEID per bearer) and addresses and TEIDs for receiving forwarded data if necessary.

8.	If indirect forwarding applies, the MME sends a Create Indirect Data Forwarding Tunnel Request message (Target eNB Address, TEID(s) for DL data forwarding) to the Serving GW. The Serving GW returns a Create Indirect Data Forwarding Tunnel Response (Cause, Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding) message to the target MME.

9.	The MME sends the message Relocation Response (Cause, List of Set Up RABs, EPS Bearers setup list, MME Tunnel Endpoint Identifier for Control Plane, RAN Cause, MME Address for Control Plane, Target to Source Transparent Container, Address(es) and TEID(s) for Data Forwarding).

10a.	If indirect forwarding applies, the AMF sends the Create Indirect Data Forwarding Tunnel Request (Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding) to the PGW-C+SMF.

10b. The PGW-C+SMF sends Serving GW Address(es) and TEID(s) for data forwarding to the PGW-U+UPF. If CN Tunnel Info for Data Forwarding is allocated by the PGW-C+SMF, the CN Tunnel Info for Data Forwarding is provided to PGW-U+UPF in this step. The PGW-U+UPF acknowledges by sending a response. If CN Tunnel Info is allocated by the PGW-U+UPF, the CN Tunnel Info is provided to PGW-C+SMF in this response.

10c. The PGW-C+SMF maps the EPS bearer for Data forwarding to the 5G QoS flows based on the association between EPS bearer ID and QoS flow thenreturns a Create Indirect Data Forwarding Tunnel Response(Cause, CN tunnel Info for Data Forwarding,QoS flows for Data Forwarding).

11.	The AMF sends the Handover Command to the source NG-RAN. The source NG-RAN commands the UE to handover to the target access network by sending the HO Command (Transparent container (radio aspect parameters that the target eNB has set-up in the preparation phase), CN tunnel info for data forwarding per PDU session, QoS flows for Data Forwarding). The UE correlates the ongoing QoS flows with the indicated EPS Bearer IDs to be setup in the HO command. UE locally deletes the QoS flows that do not have an EPS bearer ID assigned.

For the QoS flows indicated in the “QoS flows for Data Forwarding”,NG-RAN initiate data forwarding to the PGW-U+UPF based on the CN tunnel Info for Data Forwarding per PDU session. Then the PGW-U+UPF maps data received from the data forwarding tunnel(s) to the corresponding EPS bearer(s), and send the data to the target eNodeB via Serving GW.

12.	When the UE has successfully accessed the target eNodeB, the target eNodeB informs the target MME by sending the message Handover Notify.

13.	The target MME informs the Serving GW that the MME is responsible for all the bearers the UE have established by sending the Modify Bearer Request message for each PDN connection.

The target MME releases the non-accepted EPS Bearer contexts by triggering the Bearer Context deactivation procedure. If the Serving GW receives a DL packet for a non-accepted bearer, the Serving GW drops the DL packet and does not send a Downlink Data Notification to the SGSN.

14.	The Serving GW informs the PGW-C+SMF of the relocation by sending the Modify Bearer Request message for each PDN connection. The PGW locally deletes the QoS flows that do not have an EPS bearer ID assigned. Due to the "match all" filter in the default QoS flow, the PGW maps the IP flows of the deleted QoS flows to the default QoS flow.

15.	The PGW-C+SMF acknowledges the Modify Bearer Request. At this stage the User Plane path is established for the default bearer and the dedicated GBR bearers between the UE, target eNodeB, Serving GW and the PGW+SMF.

16.	The Serving GW acknowledges the User Plane switch to the MME via the message Modify Bearer Response.

17.	The PGW-C+SMF initiates dedicated bearer activation procedure for non-GBR QoS flows by mapping the parameters of the non-GBR flows to EPC QoS parameters. This setup may be triggered by the PCRF+PCF which may also provide the mapped QoS parameters, if PCC is deployed. This procedure is specified in TS 23.401 [13], clause 5.4.1.

*************** End of the changes****************