SA WG2 Meeting #S2-123	S2-177657

23 - 27 October 2017, Ljubljana, Slovenia

Title:	[draft] Reply LS to RAN 2 on QCIs for EPC based ULLC

Response to:	S2-176871 = R2-1709976 (and R1-1715089)

Release:	Rel-15

Work Item:	NR_newRAT-Core, LTE_HRLLC, LTE_sTTIandPT, EDCE5

Source:	Vodafone [SA2]

To:	RAN 2, RAN 3

Cc:	RAN 1, SA 1

Contact Person:

#### Name:	Chris Pudney

E-mail Address:	chris dot pudney at vodafone dot com

1. Overall Description:

SA2 thanks RAN 2 and RAN 1 for their replies in R2-1709976 and R1-1715089.

SA2 notes the statements that it is expected that both LTE and NR will support the transfer of 32 bytes within 1 ms or less, and, that work on “ultra reliability” has not yet started within RAN 1 and RAN 2.

The SA2 work under the EDCE5 work item has concentrated on what new QCIs are needed for ‘low latency with normal reliability’, taking into account TS 22.261’s requirements in this area (see the yellow highlighted lines in Annex A) and lower latency Mobile BroadBand.

Future SA2 work will investigate the “ultra-reliable” aspects under the EPS_URLLC WID. Note that SA2’s work on the QoS mechanisms for 5G Core may provide a more flexible description for URLLC data flows when the RAN is connected to the 5G Core Network.

SA2 have considered RAN2’s request for information on packet size, packet arrival rate, activity factor, and maximum data rates, and SA2 can see the value of having this knowledge within the RAN. However, SA2 see significant problems in the HPLMN’s core network entities and/or applications servers and/or VPLMN core network being able to determine these values accurately (or even approximately) and simply.

An example of these problems is where the same type of cellular modem is built into different types of machinery (e.g. wheeled robot vs lathe) by different firms, and then those machines are sold and used in different types of factories/environments, with the original builder of e.g. the lathe’s application changing its software (and the packet size and arrival rate) without the knowledge of the HPMLN operator or the factory owner or the cellular modem builder.

While considering the issue, SA2 came up with the concept that the eNB could learn (e.g. over a period of some seconds or even less) the characteristics of most devices’ arrival rate/activity factor and packet size, and that these characteristics could be transferred at X2/S1 handover and stored in the MME in idle mode (using vendor labelled containers) and used to assist in admission control for these devices.

As a consequence, for ‘low latency with normal reliability’, SA2 are currently planning to add just four new QCIs into their specifications. In addition, it is expected that the ‘example services’ for the existing QCI 3 are expanded to include the Electricity distribution – medium voltage’ and ‘Process automation ‒ monitoring’ scenarios from TS 22.261 clause 7.2.2, table 1. Annex B shows the characteristics of QCI 3 and the other existing QCIs.

The characteristics of these 4 new proposed QCIs are shown below. The characteristics/components are somewhat different to the characteristics/components of the legacy QCIs in TS 23.203.

| QCI value | Resource Type | Priority Level | One way PDB between SGi and UE application | Proportion of unsuccessful packets | Comments | Example services |
| --- | --- | --- | --- | --- | --- | --- |
| A | GBR | 2 | 10 ms for 99.99% of packets. Assumes 1ms from SGi to eNB physical layer, and, | 10-4 Including delayed packets As with normal GBR traffic, data above GBR is best effort | Activity factor and data burst size is to be learnt by the RAN, and stored as “unreadable RAN context” in the MME in idle state For initial admission control, a data rate averaging window of 20ms is assumed | Discrete Automation (TS 22.261, table 7.2.2-1, row 2.) |
| B | Non-GBR (but Guaranteed service level) | 2 | 10 ms for 99.99% of packets. Assumes 1ms from SGi to eNB physical layer, and, | 10-4 Including delayed packets Packets outside of the “low latency envelope” should be delivered as “best effort” and not systematically discarded. | Maximum packet size and arrival rate for which RAN “guarantees” latency and reliability are achieved is: Max packet size: 255 bytes Arrival rate < once per 50 ms Activity factor and data burst size is learnt by the RAN, and stored as “unreadable RAN context” in the MME in idle state | Discrete Automation (TS 22.261, table 7.2.2-1, row 2, “small packets”) |
| C | Non-GBR (but Guaranteed service level) | 2 | 10 ms for 99.99% of packets. Assumes 1ms from SGi to eNB physical layer, and, | 10-4 Including delayed packets Packets outside of the “low latency envelope” should be delivered as “best effort” and not systematically discarded. | Maximum packet size and arrival rate for which RAN “guarantees” latency and reliability are achieved is Max packet size: 1500 bytes Arrival rate < once per 50 ms Activity factor and data burst size is learnt by the RAN and stored as “unreadable RAN context” in the MME in idle state | Discrete Automation (TS 22.261, table 7.2.2-1, row 2, “big packets”) |

| E | Non-GBR | 6.2 | 10 ms For 98% of packets (as existing TS 23.203) Assumes 2 ms from SGi to eNB physical layer | 10-6 Excluding delayed packets. This error rate is intended for the configuration of the RAN data link. | This is a “best effort” bearer | Low latency eMBB applications (TCP-IP based); Augmented Reality |
| --- | --- | --- | --- | --- | --- | --- |

Discussion on the background to these values and characteristics is welcomed, e.g. via conference call or email prior to SA2 #124 in Reno.

2. Actions:

To RAN 2, RAN 3.

ACTION: 	SA2 politely asks RAN 2 and RAN 3 to note that SA2 is intending to pursue the above way forward, and hence to provide rapid feedback if they wish SA2 to adjust this way forward.

To RAN 3:

ACTION: 	SA2 politely asks RAN 3 to develop S1 and X2 interface CRs for the above mentioned container mechanism.

3. Date of Next SA2 Meetings:

SA2 #124	27 Nov – 1 Dec 2017	Reno, Nevada, USA

SA2 #125	22 – 26 Jan 2018	Gothenberg, Sweden

# Annex A: extracted from TS 22.261 v15.1.0

Table 7.2.2-1 Performance requirements for low-latency and high-reliability scenarios.

| Scenario | End-to-end latency (note 3) | Jitter | Survival time | Communication service availability (note 4) | Reliability (note 4) | User experienced data rate | Payload size (note 5) | Traffic density (note 6) | Connection density (note 7) | Service area dimension (note 8) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discrete automation – motion control (note 1) | 1 ms | 1 µs | 0 ms | 99,9999% | 99,9999% | 1 Mbps up to 10 Mbps | Small | 1 Tbps/km2 | 100 000/km2 | 100 x 100 x 30 m |
| Discrete automation | 10 ms | 100 µs | 0 ms | 99,99% | 99,99% | 10 Mbps | Small to big | 1 Tbps/km2 | 100 000/km2 | 1000 x 1000 x 30 m |
| Process automation – remote control | 50 ms | 20 ms | 100 ms | 99,9999% | 99,9999% | 1 Mbps up to 100 Mbps | Small to big | 100 Gbps/km2 | 1 000/km2 | 300 x 300 x 50 m |
| Process automation ‒ monitoring | 50 ms | 20 ms | 100 ms | 99,9% | 99,9% | 1 Mbps | Small | 10 Gbps/km2 | 10 000/km2 | 300 x 300 x 50 |
| Electricity distribution – medium voltage | 25 ms | 25 ms | 25 ms | 99,9% | 99,9% | 10 Mbps | Small to big | 10 Gbps/km2 | 1 000/km2 | 100 km along power line |
| Electricity distribution – high voltage (note 2) | 5 ms | 1 ms | 10 ms | 99,9999% | 99,9999% | 10 Mbps | Small | 100 Gbps/km2 | 1 000/km2 (note 9) | 200 km along power line |
| Intelligent transport systems – infrastructure backhaul | 10 ms | 20 ms | 100 ms | 99,9999% | 99,9999% | 10 Mbps | Small to big | 10 Gbps/km2 | 1 000/km2 | 2 km along a road |
| Tactile interaction (note 1) | 0,5 ms | TBC | TBC | [99,999%] | [99,999%] | [Low] | [Small] | [Low] | [Low] | TBC |
| Remote control | [5 ms] | TBC | TBC | [99,999%] | [99,999%] | [From low to 10 Mbps] | [Small to big] | [Low] | [Low] | TBC |
| NOTE 1: Traffic prioritization and hosting services close to the end-user may be helpful in reaching the lowest latency values. NOTE 2: Currently realised via wired communication lines. NOTE 3: This is the end-to-end latency the service requires. The end-to-end latency is not completely allocated to the 5G system in case other networks are in the communication path. NOTE 4: Communication service availability relates to the service interfaces, reliability relates to a given node. Reliability should be equal or higher than communication service availability. NOTE 5: Small: payload typically ≤ 256 bytes NOTE 6: Based on the assumption that all connected applications within the service volume require the user experienced data rate. NOTE 7: Under the assumption of 100% 5G penetration. NOTE 8 Estimates of maximum dimensions; the last figure is the vertical dimension. NOTE 9: In dense urban areas. NOTE 10: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |  |  |

# Annex B: Existing QCIs from 3GPP TS23.203

Table 6.1.7: Standardized QCI characteristics

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE 13) | Packet Error Loss Rate (NOTE 2) | Example Services |
| --- | --- | --- | --- | --- | --- |
| 1 (NOTE 3) |  | 2 | 100 ms (NOTE 1, NOTE 11) | 10-2 | Conversational Voice |
| 2 (NOTE 3) | GBR | 4 | 150 ms (NOTE 1, NOTE 11) | 10-3 | Conversational Video (Live Streaming) |
| 3 (NOTE 3), NOTE 14 |  | 3 | 50 ms (NOTE 1, NOTE 11) | 10-3 | Real Time Gaming, V2X messages |
| 4 (NOTE 3) |  | 5 | 300 ms (NOTE 1, NOTE 11) | 10-6 | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 3, NOTE 9, NOTE 12) |  | 0.7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 (NOTE 3, NOTE 12) |  | 2 | 100 ms (NOTE 1, NOTE 10) | 10-2 | Non-Mission-Critical user plane Push To Talk voice |
| 75 (NOTE 14) |  | 2.5 | 50 ms (NOTE 1) | 10-2 | V2X messages |
| 5 (NOTE 3) |  | 1 | 100 ms (NOTE 1, NOTE 10) | 10-6 | IMS Signalling |
| 6 (NOTE 4) |  | 6 | 300 ms (NOTE 1, NOTE 10) | 10-6 | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 (NOTE 3) | Non-GBR | 7 | 100 ms (NOTE 1, NOTE 10) | 10-3 | Voice, Video (Live Streaming) Interactive Gaming |
| 8 (NOTE 5) |  | 8 | 300 ms (NOTE 1) | 10-6 | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file |
| 9 (NOTE 6) |  | 9 |  |  | sharing, progressive video, etc.) |
| 69 (NOTE 3, NOTE 9, NOTE 12) |  | 0.5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 (NOTE 4, NOTE 12) |  | 5.5 | 200 ms (NOTE 7, NOTE 10) | 10-6 | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 (NOTE 14) |  | 6.5 | 50 ms (NOTE 1, NOTE 10) | 10-2 | V2X messages |
| NOTE 1: A delay of 20 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. This delay is the average between the case where the PCEF is located "close" to the radio base station (roughly 10 ms) and the case where the PCEF is located "far" from the radio base station, e.g. in case of roaming with home routed traffic (the one-way packet delay between Europe and the US west coast is roughly 50 ms). The average takes into account that roaming is a less typical scenario. It is expected that subtracting this average delay of 20 ms from a given PDB will lead to desired end-to-end performance in most typical cases. Also, note that the PDB defines an upper bound. Actual packet delays - in particular for GBR traffic - should typically be lower than the PDB specified for a QCI as long as the UE has sufficient radio channel quality. NOTE 2: The rate of non congestion related packet losses that may occur between a radio base station and a PCEF should be regarded to be negligible. A PELR value specified for a standardized QCI therefore applies completely to the radio interface between a UE and radio base station. NOTE 3: This QCI is typically associated with an operator controlled service, i.e., a service where the SDF aggregate's uplink / downlink packet filters are known at the point in time when the SDF aggregate is authorized. In case of E-UTRAN this is the point in time when a corresponding dedicated EPS bearer is established / modified. NOTE 4: If the network supports Multimedia Priority Services (MPS) then this QCI could be used for the prioritization of non real-time data (i.e. most typically TCP-based services/applications) of MPS subscribers. NOTE 5: This QCI could be used for a dedicated "premium bearer" (e.g. associated with premium content) for any subscriber / subscriber group. Also in this case, the SDF aggregate's uplink / downlink packet filters are known at the point in time when the SDF aggregate is authorized. Alternatively, this QCI could be used for the default bearer of a UE/PDN for "premium subscribers". NOTE 6: This QCI is typically used for the default bearer of a UE/PDN for non privileged subscribers. Note that AMBR can be used as a "tool" to provide subscriber differentiation between subscriber groups connected to the same PDN with the same QCI on the default bearer. NOTE 7: For Mission Critical services, it may be assumed that the PCEF is located "close" to the radio base station (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence delay of 10 ms for the delay between a PCEF and a radio base station should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In both RRC Idle and RRC Connected mode, the PDB requirement for these QCIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that QCI-65 and QCI-69 are used together to provide Mission Critical Push to Talk service (e.g., QCI-5 is not used for signalling for the bearer that utilizes QCI-65 as user plane bearer). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In both RRC Idle and RRC Connected mode, the PDB requirement for these QCIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC Idle mode, the PDB requirement for these QCIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This QCI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this QCI value. NOTE 13: Packet delay budget is not applicable on NB-IoT or when Enhanced Coverage is used for WB-E-UTRAN (see TS 36.300 [19]). NOTE 14: This QCI could be used for transmission of V2X messages as defined in TS 23.285 [48]. |  |  |  |  |  |