SA WG2 Meeting #123	S2-177697

23 – 27 October, 2017, Ljubljana, Slovenia	(revision of S2-176903)

Source:	Ericsson

Title:	Event triggers

Document for:	Approval

Agenda Item:	6.5.6

Work Item / Release:	5GS_ph1

Abstract of the contribution: -

# Introduction

This contribution proposes to introduce Event triggers for events provided by the SMF to the PCF.

# Proposal

It is proposed to update 23.503 as described below.

* * * * Start of Changes * * * *

### 8.3.1.5	Event Triggers

The event triggers define the conditions when the SMF shall interact again with PCF after a PDU session establishment.

The event triggers are listed in table 8.3.1.5a.

# Table 8.3.1.5a: Access independent Event triggers

| Event trigger | Description | Difference compared with table 6.2 in TS 23.203 [4] | Motivation |
| --- | --- | --- | --- |
| PLMN change | The UE has moved to another operators' domain. | None |  |
| QoS change | The QoS parameters of the QoS flow has changed | Removed | Only applicable when binding of bearers was done in PCRF. |
| QoS change exceeding authorization | The QoS parameters of the QoS flow has changed and exceeds the authorized QoS | Removed | Only applicable when binding of bearers was done in PCRF. |
| Traffic mapping information change | The traffic mapping information of the QoS profile has changed | Removed | Only applicable when binding of bearers was done in PCRF. |
| Resource modification request | A request for resource modification has been received by the SMF (note 6). | Removed | Not in 5GS yet. |
| Routing information change | The IP flow mobility routing information has changed (when IP flow mobility as specified in TS 23.261 [23] applies) or the PCEF has received Routing Rules from the UE (when NBIFOM as specified in TS 23.161 [43] applies) | Removed | Not in 5GS yet. |
| Change in type of Access Type | The access type of the PDU session has changed. | None |  |
| Loss/recovery of transmission resources | The Access type transmission resources are no longer usable/again usable. | Removed | Not in 5GS yet. |
| Location change (serving cell) | The serving cell of the UE has changed. | Remove |  |
| Location change (serving area) | The serving area of the UE has changed. | None |  |
| Location change (serving CN node) | The serving core network node of the UE has changed. | None |  |
| Change of UE presence in Presence Reporting Area | The UE is entering/leaving a Presence Reporting Area | None |  |
| Out of credit | Credit is no longer available. | None | May need validation with SA5. |
| Enforced PCC rule request | SMF is performing a PCC rules request as instructed by the PCF. | None | PCF |
| Enforced ADC rule request | TDF is performing an ADC rules request as instructed by the PCRF. | Removed | ADC Rules are not applicable. |
| UE IP address change | A UE IP address has been allocated/released | None | Always set |
| Access Network Charging Correlation Information | Access Network Charging Correlation Information has been assigned. | None | PCF |
| Usage report | The PDU session or the Monitoring key specific resources consumed by a UE either reached the threshold or needs to be reported for other reasons. | None | PCF |
| Start of application traffic detection and Stop of application traffic detection | The start or the stop of application traffic has been detected. | None | PCF |
| SRVCC CS to PS handover | A CS to PS handover has been detected | removed | No support in 5GS yet |
| Access Network Information report | Access information as specified in the Access Network Information Reporting part of a PCC rule. | None | PCF |
| Credit management session failure | Transient/Permanent Failure as specified by the OCS | None | PCF |
| Addition / removal of an access to an IP-CAN session | The PCEF reports when an access is added or removed | removed | No support in 5GS yet |

The enforced PCC rule request event trigger shall trigger a SMF interaction to request PCC rules from the PCF for an established PDU session. This SMF interaction shall take place within the Revalidation time limit set by the PCF in the PDU session related policy information.

The UE IP address change event trigger shall trigger a SMF interaction with the PCF in case a UE IP address is allocated or released during the lifetime of the IP-CAN session.

The Access Network Charging Correlation Information event shall trigger the SMF to report the assigned access network charging identifier for the PCC rules that are accompanied with a request for this event at activation.

If the Usage report event trigger is set and the volume or the time thresholds, earlier provided by the PCF, are reached, the SMF shall report this event to the PCF. If both volume and time thresholds were provided and the thresholds, for one of the measurements, are reached, the SMF shall report this event to the PCF and the accumulated usage since last report shall be reported for both measurements.

The Start of application traffic detection and Stop of application traffic detection events shall trigger an interaction with PCF once the requested application traffic is detected (i.e. Start of application traffic detection) or the end of the requested application traffic is detected (i.e. Stop of application traffic detection) unless it is requested within a specific PCC Rule to mute such a notification for solicited application reporting or unconditionally in case of unsolicited application reporting. The application identifier and service data flow descriptions, if deducible, shall also be included in the report. An application instance identifier shall be included in the report both for Start and for Stop of application traffic detection when service data flow descriptions are deducible. This is done to unambiguously match the Start and the Stop events.

At PCC rule activation, modification and deactivation the SMF shall send, as specified in the PCC rule, the User Location Report and/or UE Timezone Report to the PCF.

NOTE 4:	At PCC rule deactivation the User Location Report includes information on when the UE was last known to be in that location.

The PCF shall send the User Location Report and/or UE Timezone Report to the AF upon receiving an Access Network Information report corresponding to the AF session from the SMF.

If the event trigger for Access Network Information reporting is set, the SMF shall check the need for access network information reporting after successful installation/modification or removal of a PCC rule or upon termination of the PDU session. The SMF shall check the Access Network Information report parameters (User Location Report, UE Timezone Report) of the PCC rules and report the access network information to the PCF. The SMF shall not report any subsequent access network information updates received from the PDU session without any previous updates of related PCC rule unless the associated QoS flow or PDU session has been released.

If the SMF receives a request to install/modify or remove a PCC rule with Access Network Information report parameters (User Location Report, UE Timezone Report) set the SMF shall initiate a PDU session modification to retrieve the current access network information of the UE and forward it to the PCF afterwards.

If the Access Network Information report parameter for the User Location Report is set and the user location (e.g. cell) is not available to the SMF, the SMF shall provide the serving PLMN identifier to the PCF which shall forward it to the AF.

The Credit management session failure event trigger shall trigger a SMF interaction with the PCF to inform about a credit management session failure and to indicate the failure reason, and the affected PCC rules.

Editor´s note: Whether following supplementary events are supported:  the UE has moved between EPC and 5GC: The PCF takes this information into account to determine which features can or can no more be supported by the PDU session is FFS

* * * * End of Changes * * * *