SA WG2 Meeting #123		S2-178077

23 – 27 October, 2017, Ljubljana, Slovenia	(revision of S2-177696)

Source:	Ericsson

Title:	TS 23.503: Policy and Charging Control Rule in 5G

Document for:	Approval

Agenda Item:	6.5.6

Work Item / Release:	5GS_ph1

Abstract of the contribution: This contribution proposes to move the PCC Rule into TS 23.503.

# Introduction

This document proposes to move to the PCC Rule into TS 23.503.  The differences with table 6.3. in TS 23.203 are listed in a new column.

| Information name | Description | Difference compared to table 6.3 in TS 23.203 |
| --- | --- | --- |
| Rule identifier | Uniquely identifies the PCC rule, within an IP-CAN session. It is used between PCF and SMF for referencing PCC rules. | None |
| Service data flow detection | This part defines the method for detecting packets belonging to a service data flow. |  |
| Precedence | Determines the order, in which the service data flow templates are applied at service data flow detection, enforcement and charging. | None |
| Service data flow template | For IP PDU traffic: Either a list of service data flow filters or an application identifier that references the corresponding application detection filter for the detection of the service data flow. For Ethernet PDU traffic: Combination of traffic patterns of the Ethernet PDU traffic. | Modified (packet filters for Ethernet PDU traffic added) |
| Mute for notification | Defines whether application's start or stop notification is to be muted. | None |
| Charging | This part defines identities and instructions for charging and accounting that is required for an access point where flow based charging is configured |  |
| Charging key | The charging system (OCS or OFCS) uses the charging key to determine the tariff to apply to the service data flow. | None |
| Service identifier | The identity of the service or service component the service data flow in a rule relates to. | None |
| Sponsor Identifier | An identifier, provided from the AF which identifies the Sponsor, used for sponsored flows to correlate measurements from different users for accounting purposes. | None |
| Application Service Provider Identifier | An identifier, provided from the AF which identifies the Application Service Provider, used for sponsored flows to correlate measurements from different users for accounting purposes. | None |
| Charging method | Indicates the required charging method for the PCC rule. Values: online, offline or neither. | None |
| Measurement method | Indicates whether the service data flow data volume, duration, combined volume/duration or event shall be measured. This is applicable to reporting, if the charging method is online or offline. Note: Event based charging is only applicable to predefined PCC rules and PCC rules used for application detection filter (i.e. with an application identifier). | None |
| Application Function Record Information | An identifier, provided from the AF, correlating the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. | None |
| Service identifier level reporting | Indicates that separate usage reports shall be generated for this Service identifier. Values: mandated or not required | None |
| Policy control | This part defines how the PCEF shall apply policy control for the service data flow. |  |
| Gate status | The gate status indicates whether the service data flow, detected by the service data flow template, may pass (Gate is open) or shall be discarded (Gate is closed). | None |
| 5G QoS identifier | Identifier for the authorized QoS parameters for the service data flow. Values: | modified (5G QoS Indicators (5QIs) as values) |
| QoS Notification Control (QNC) | Indicates a request for notification from RAN for the SDF when the QoS targets for a GBR 5QI cannot be fulfilled for a QoS flow during the lifetime of the QoS flow. | Added |
| Reflective QoS Indication | Indicates to apply reflective QoS for the service data flow. | Added |
| UL-maximum bitrate | The uplink maximum bitrate authorized for the service data flow | None |
| DL-maximum bitrate | The downlink maximum bitrate authorized for the service data flow | None |
| UL-guaranteed bitrate | The uplink guaranteed bitrate authorized for the service data flow | None |
| DL-guaranteed bitrate | The downlink guaranteed bitrate authorized for the service data flow | None |
| UL sharing indication | Indicates resource sharing in uplink direction with service data flows having the same value in their PCC rule | None |
| DL sharing indication | Indicates resource sharing in downlink direction with service data flows having the same value in their PCC rule | None |
| Redirect | Redirect state of the service data flow (enabled/disabled) | None |
| Redirect Destination | Controlled Address to which the service data flow is redirected when redirect is enabled | None |
| ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability | None |
| Bind to Default QoS flow | Indicates that the dynamic PCC rule shall always have its binding with the default QoS flow | None |
| PS to CS session continuity | Indicates whether the service data flow is a candidate for vSRVCC. | Removed |
| Access Network Information Reporting | This part describes access network information to be reported for the PCC rule when the corresponding bearer is established, modified or terminated. |  |
| User Location Report | The serving cell of the UE is to be reported. When the corresponding bearer is deactivated, and if available, information on when the UE was last known to be in that location is also to be reported. | None |
| UE Timezone Report | The time zone of the UE is to be reported. | None |
| Usage Monitoring Control | This part describes identities required for Usage Monitoring Control. |  |
| Monitoring key | The PCRF uses the monitoring key to group services that share a common allowed usage. | None |
| Indication of exclusion from session level monitoring | Indicates that the service data flow shall be excluded from PDU session usage monitoring | None |
| Traffic Steering Enforcement Control | This part describes identities required for Traffic Steering Enforcement Control. |  |
| Traffic steering policy identifier(s) | Reference to a pre-configured traffic steering policy at the SMF (NOTE 13). | None |
| Data Network Access Identifier | Identifier of the target Data Network Access. It is defined in 3GPP TS 23.501, clause 5.6.7. | Added |
| Data Network Access Change report | Indicates whether a notification in case of change of DNAI at addition/change/removal of the UPF is requested, as well as the destination(s) for where to provide the notification. The notification information includes the target DNAI and an indication of early and/or late notification. It is defined in 3GPP TS 23.501, clause 5.6.7 | Added |
| NBIFOM related control Information | This part describes PCC rule information related with NBIFOM (defined in TS 23.161 [43]. Refer also to clause 6.1.18. |  |
| Allowed Access Type | The access to be used for traffic identified by the PCC rule | Removed |
| Routing Rule Identifier | The Routing Rule identifier to be used in NBIFOM routing rule | Removed |

# Proposal

It is proposed to update 23.503 as described below.

* * * First Change * * *

## 8.3.3	Policy and Charging Control Rule

## 8.3.3.1	General

To enable the enforcement in the 5GC of the policy decisions made by the PCF for the policy and charging control of a service data flow, the 5GC shall provide 5G Policy and Charging Control information from the PCF to the SMF as described in table 8.3.3.1. The description of the new or modified IE compared with table 6.1 is listed below.

The differences with table 6.3 in TS 23.203[4] are shown, either “none” means that the IE applies in 5GS or “removed” meaning that the IE does not apply in 5GS, this is due to the lack of support in the 5GS for this feature or “modified” meaning that the IE applies with some modifications defined in the IE.

Table 8.3.3.1: The PCC rule information in 5GC

| Information name | Description | Category | PCF permitted to modify for a dynamic PCC rule in the SMF | Differences compared with table 6.3. in TS 23.203 [4] |
| --- | --- | --- | --- | --- |
| Rule identifier | Uniquely identifies the PCC rule, within a PDU session. It is used between PCF and SMF for referencing PCC rules. | Mandatory | no | None |
| Service data flow detection | This part defines the method for detecting packets belonging to a service data flow. |  |  |  |
| Precedence | Determines the order, in which the service data flow templates are applied at service data flow detection, enforcement and charging. (NOTE 1). | Conditional (NOTE 2) | yes | None |
| Service data flow template | For IP PDU traffic: Either a list of service data flow filters or an application identifier that references the corresponding application detection filter for the detection of the service data flow. For Ethernet PDU traffic: Combination of traffic patterns of the Ethernet PDU traffic. It is defined in 3GPP TS 23.501, clause 5.7.6.3 | Mandatory (NOTE 3) | Conditional (NOTE 4) | Modified (packet filters for Ethernet PDU traffic added) |
| Mute for notification | Defines whether application's start or stop notification is to be muted. | Conditional (NOTE 5) | No | None |
| Charging | This part defines identities and instructions for charging and accounting that is required for an access point where flow based charging is configured |  |  |  |
| Charging key | The charging system (OCS or OFCS) uses the charging key to determine the tariff to apply to the service data flow. |  | yes | None |
| Service identifier | The identity of the service or service component the service data flow in a rule relates to. |  | yes | None |
| Sponsor Identifier | An identifier, provided from the AF which identifies the Sponsor, used for sponsored flows to correlate measurements from different users for accounting purposes. | Conditional (NOTE 6) | yes | None |
| Application Service Provider Identifier | An identifier, provided from the AF which identifies the Application Service Provider, used for sponsored flows to correlate measurements from different users for accounting purposes. | Conditional (NOTE 6) | yes | None |
| Charging method | Indicates the required charging method for the PCC rule. Values: online, offline or neither. | Conditional (NOTE 7) | no | None |
| Measurement method | Indicates whether the service data flow data volume, duration, combined volume/duration or event shall be measured. This is applicable to reporting, if the charging method is online or offline. Note: Event based charging is only applicable to predefined PCC rules and PCC rules used for application detection filter (i.e. with an application identifier). |  | yes | None |
| Application Function Record Information | An identifier, provided from the AF, correlating the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. |  | no | None |
| Service identifier level reporting | Indicates that separate usage reports shall be generated for this Service identifier. Values: mandated or not required |  | yes | None |
| Policy control | This part defines how to apply policy control for the service data flow. |  |  |  |
| Gate status | The gate status indicates whether the service data flow, detected by the service data flow template, may pass (Gate is open) or shall be discarded (Gate is closed). |  | yes | None |
| 5G QoS identifier | Identifier for the authorized QoS parameters for the service data flow. | Conditional | yes | Modified (corresponds to QCI in 23.203) |
| QoS Notification Control (QNC) | Indicates a request for notification from ®AN for the SDF when the QoS targets for a GBR 5QI cannot be fulfilled for a QoS flow during the lifetime of the QoS flow. | Conditional | yes | Added |
| Reflective QoS Indication | Indicates to apply reflective QoS for the service data flow. |  | yes | Added |
| UL-maximum bitrate | The uplink maximum bitrate authorized for the service data flow |  | yes | None |
| DL-maximum bitrate | The downlink maximum bitrate authorized for the service data flow |  | yes | None |
| UL-guaranteed bitrate | The uplink guaranteed bitrate authorized for the service data flow |  | yes | None |
| DL-guaranteed bitrate | The downlink guaranteed bitrate authorized for the service data flow |  | yes | None |
| UL sharing indication | Indicates resource sharing in uplink direction with service data flows having the same value in their PCC rule |  | no | None |
| DL sharing indication | Indicates resource sharing in downlink direction with service data flows having the same value in their PCC rule |  | no | None |
| Redirect | Redirect state of the service data flow (enabled/disabled) | Conditional (NOTE 8) | yes | None |
| Redirect Destination | Controlled Address to which the service data flow is redirected when redirect is enabled | Conditional (NOTE 9) | yes | None |
| ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability | Conditional (NOTE 10) | yes | None |
| Bind to QoS flow of the default QoS rule | Indicates that the dynamic PCC rule shall always have its binding with the QoS flow of the default QoS rule. | Conditional (NOTE 11) | yes | Modified (corresponds to bind to the default bearer in 23.203) |
| PS to CS session continuity | Indicates whether the service data flow is a candidate for vSRVCC. |  |  | Removed |
| Access Network Information Reporting | This part describes access network information to be reported for the PCC rule when the corresponding bearer is established, modified or terminated. |  |  |  |
| User Location Report | The serving cell of the UE is to be reported. When the corresponding bearer is deactivated, and if available, information on when the UE was last known to be in that location is also to be reported. |  | yes | None |
| UE Timezone Report | The time zone of the UE is to be reported. |  | yes | None |
| Usage Monitoring Control | This part describes identities required for Usage Monitoring Control. |  |  | None |
| Monitoring key | The PCF uses the monitoring key to group services that share a common allowed usage. |  | Yes | None |
| Indication of exclusion from session level monitoring | Indicates that the service data flow shall be excluded from PDU session usage monitoring |  | Yes | None |
| Traffic Steering Enforcement Control | This part describes identities required for Traffic Steering Enforcement Control. |  |  |  |
| Traffic steering policy identifier(s) | Reference to a pre-configured traffic steering policy at the SMF (NOTE 12). |  | Yes | None |
| Data Network Access Identifier | Identifier of the target Data Network Access. It is defined in 3GPP TS 23.501, clause 5.6.7. |  | Yes | Added |
| Data Network Access Change report | Indicates whether a notification in case of change of DNAI at addition/change/removal of the UPF is requested, as well as the destination(s) for where to provide the notification. The notification information includes the target DNAI and an indication of early and/or late notification. It is defined in 3GPP TS 23.501, clause 5.6.7 |  | Yes | Added |
| NBIFOM related control Information | This part describes PCC rule information related with NBIFOM |  |  |  |
| Allowed Access Type | The access to be used for traffic identified by the PCC rule |  |  | Removed |
| NOTE 1: For PCC rules based on an application detection filter, the precedence is only relevant for the enforcement, i.e. when multiple PCC rules overlap, only the enforcement, reporting of application starts and stops, monitoring, and charging actions of the PCC rule with the highest precedence shall be applied. NOTE 2: The Precedence is mandatory for PCC rules with SDF template containing SDF filter(s). For dynamic PCC rules with SDF template containing an application identifier, the precedence is either preconfigured in SMF or provided in the PCC rule from PCF. NOTE 3: Either service data flow filter(s) or application identifier shall be defined per each rule. NOTE 4: YES, in case the service data flow template consists of a set of service data flow filters. NO in case the service data flow template consists of an application identifier NOTE 5: Optional and applicable only if application identifier exists within the rule. NOTE 6: Applicable to sponsored data connectivity. NOTE 7: Mandatory if there is no default charging method for the PDU session. NOTE 8: Optional and applicable only if application identifier exists within the rule. NOTE 9: If Redirect is enabled. NOTE 10: Mandatory when policy control on SDF level applies. NOTE 11: The presence of this attribute causes the 5QI/ARP/QNC of the rule to be ignored. NOTE 12: The Traffic steering policy identifier can be different for uplink and downlink direction. If two Traffic steering policy identifiers are provided, then one is for uplink direction, while the other one is for downlink direction. |  |  |  |  |

Editor's note:	It is FFS how it is indicated in the PCC rule the AF subscription to SMF notifications.

Editor's note:	The details of charging are specified by SA5, which may require later amendments to the Charging information in the PCC Rule.

Editor´s note: How interworking with E-UTRAN is not defined yet.

The Service data flow template may comprise any number of Service data flow filters or an application identifier for IP PDU traffic as is defined in table 6.3 Additionally, it may also comprise any combination of traffic patterns of the Ethernet PDU traffic.

The 5G QoS Indicator, 5QI, represents the QoS parameters for the service data flow. The 5G QoS indictor is scalar and accommodates the need for differentiating QoS in both 3GPP and non-3GPP access type.

The Bind to QoS Flow of the default QoS rule indicates that the SDF shall be bound to the QoS Flow of the default QoS rule. The presence of  this parameter attribute causes the 5QI/ARP/QNC of the rule to be ignored by the SMF during the QoS Flow binding.

The QoS Notification Control, QNC, indicates a request for notification to the access network (i.e. 3GPP RAN) when the QoS targets for the SDF cannot be fulfilled. If it is set and QoS targets cannot be fulfilled, the access network (i.e 3GPP RAN) sends a notification towards SMF, which notifies to PCF.

The Reflective QoS Indication indicates to apply reflective QoS for the service data flow. Reflective QoS is defined in TS 23.501 [2] clause 5.7.5.

The Traffic Steering Enforcement Control contains:

-	The Target DNAI is a reference to the DNAI the SMF needs to consider for UPF selection/reselection.

-	The Data Network Access Change report parameters (Target DNAI and Indication of early and/or late notification) instruct the SMF about what information to forward to the PCF when DNAI changes at change of the UPF and where to provide the indication.

#### 8.3.3.2	Policy and Charging Rule Control Operations

Policy and Charging Rule operations consist of activation, modification and de-activation of PCC rules performed by PCF to SMF.

An active PCC rule means that:

-	the service data flow template shall be used for service data flow detection;

-	the service data flow template shall be used for mapping of downlink packets to the QoS flow determined by the QoS flow binding;

-	the service data flow template shall be used for service data flow detection of uplink packets on the PDU session;

-	usage data for the service data flow shall be recorded;

-	policies associated with the PCC rule, if any, shall be invoked;

-	for service data flow detection with an application detection filter, the start or the stop of the application traffic is reported to the PCF, if applicable and requested by the PCF. In that case, the notification for start may include service data flow filters, (if possible to provide) and the application instance identifier associated with the service data flow filters.

A predefined PCC rule is known at least, within the scope of one PDU session.

NOTE 1:	The same predefined PCC rule can be activated for multiple QoS flows in multiple PDU sessions.

A predefined PCC rule is bound to one and only one QoS flow per PDU session. For a predefined PCC rule whose service data flow cannot be fully reflected for the uplink direction in terms of traffic mapping information sent to the UE, the SMF may rquest the UPF to apply the uplink service data flow detection at additional QoS flows with non-GBR 5QI of the same PDU session. The deactivation of such a predefined PCC rule ceases its service data flow detection for the whole PDU session.

The PCF may, at any time, deactivate an active PCC rule in the SMF. At QoS flow termination all active PCC rules on that QoS flow are deactivated without explicit instructions from the PCF to do so.

Policy and charging control rule operations can be also performed in a deferred mode. A PCC rule may have either a single deferred activation time, or a single deferred deactivation time or both.  How activation and deactivation works is described in clause 6.3.2.

Deferred activation and deactivation of PCC rules can only be used for PCC rules that belong to the QoS flow without traffic mapping information.

NOTE 2:	This limitation prevents dependencies on the signalling of changed traffic mapping information towards the UE.

* * * End of Changes * * *