SA WG2 Meeting #123	S2- 178173

23 - 27 October 2017, Ljubljana, SIovenia	(revision of S2-176995, 7596, 7967, 8121, 8149)

Source:	Ericsson

Title:	Finalizing the UE Reachability Procedure with RRC Inactive handling (OI#13)

Document for:	Approval

Agenda Item:	6.5.7.5

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: Completion of RRC Inactive in the UE Reachability procedure

# Discussion

In TS 23.502 v1.2.0, the following issues with relation to RRC Inactive state remains:

- The UE Reachability Notification Request procedure in clause 4.2.5.2 is not fully completed. Step 4,5,6 are missing and several ENs/FFSes remain.

- The UE Activity Notification procedure in clause 4.2.5.3 is not fully completed. Steps 2 & 3 needs clarification.

- There is a lack of procedure support for AMF to provide the service defined in Namf_MT_EnableUEReachability service operation in clause 5.2.2.4.2.

Proposals:

- Steps 4, 5, 6 in the UE Reachability Notification Request Procedure are finalized using the N2 UE Notification for reachability check and two options for the Namf_EventExposure_Notify message, with and without UDM involvement (i.e. directly AMFNF and AMFUDMNF) similar to the direct notification from the AMF to the NEF in clause 4.15.2.3.

- Steps 2 & 3 in the UE Activity Notification procedure in clause 4.2.5.3 are clarified with option for direct notification same as above.

- Update the N2 UE notification procedure to fulfill the need of UE reachability Notification Request Procedure and the Namf_MT_EnableUEReachability service:

  - Add report type for single notification when the UE moves to RRC-Connected state (as needed by the UE Reachability Notification Request procedure in clause 4.2.5.2).

  - Possibility to page and move UE to RRC-Connected state (as required by the MT_EnableUEReachability service in clause 5.2.2.4.2).

# Proposal

***** 1st Change *****

### 4.2.5	Reachability procedures

Editor's note:	Procedures for reachability e.g. procedures equivalent to URRP-MME request and notification of UE NAS activity.

#### 4.2.5.1	General

Elements of this procedure are used by both "SMS over NAS" and UDM initiated UE Reachability Notification requests.

The procedure applies to UEs that are in RRC-Idle, RRC-Inactive and RRC-Connected states.

There are two procedures necessary for any service related entity that would need to be notified by the reachability of the UE:

-	UE Reachability Notification Request procedure; and

-	UE Activity Notification procedure.

#### 4.2.5.2	UE Reachability Notification Request procedure

The UE Reachability Notification Request procedure is illustrated in figure 4.2.5.2-1.

Figure 4.2.5.2-1: UE Reachability Notification Request Procedure

0.	During the Registration or subscription update procedure, the UDM informs the AMF of the identities (e.g. FQDNs) of the Network Functions that are authorised to request notifications on this UE's reachability via Nudm_UEContextManagement_Registration or Nudm_SubscriberData_Update service operation. By default, the UDM is always authorised.

1.	If a service-related entity requests the UDM to provide an indication regarding UE reachability, the UDM checks that that entity is authorised to perform this request on this subscriber.

If the entity is not authorised, the request may be rejected (e.g. if the requesting entity is recognised as being a valid entity, but not authorised for that subscriber) or silently discarded (e.g. if the requesting entity is not recognised). Appropriate O&M reports are generated.

2.	The UDM stores the identity of the service-related entity and sets the URRP-AMF parameter to indicate that such request is received. If the value of URRP-AMF parameter has changed from "not set" to "set", the UDM initiates Namf_EventExposure_Subscribe_service operation (URRP-AMF) towards the AMF. The UDM may indicate if direct notification to NF shall be used.

Editor's note:	The UDM may also need to send this message to an MME and/or to an entity handling non-3GPP access.

3.	The AMF checks that the requesting entity is authorised to perform this request on this subscriber.

If the entity is not authorised, the request may be rejected (e.g. if the requesting entity is recognised as being a valid entity, but not authorised for that subscriber) or silently discarded (e.g. if the requesting entity is not recognised). Appropriate O&M reports are generated.

If the AMF has a MM Context for that user, the AMF sets URRP-AMF to indicate the need to report to the UDM information regarding changes in UE reachability, e.g. when the next NAS activity with that UE is detected.

4.	If the UE state in AMF is in CM-CONNECTED state, the AMF initiates N2 Notification procedure (see clause 4.8.3) with reporting type set to Single RRC-Connected state notification.

5.	When UE reachability exists, the AMF initiates a UE reachability notification using Namf_EventExposure_Notify (SUPI, UE-Reachable) towards either UDM or using direct notification to NF if provided in step 2. The AMF clears the corresponding URRP-AMF for that UE

For a UE in CM-IDLE state, "UE reachability exists" at the next NAS signalling UE activity.

For a UE in CM-CONNECTED state, "UE reachability exists" either when AMF receives the UE Notification from the RAN node informing that UE is reachable (see clause 4.8.3), or when AMF receives a Path Switch Request from a new RAN node.

#### 4.2.5.3	UE Activity Notification procedure

The UE Activity Notification procedure is illustrated in figure 4.2.5.3-1.

Figure 4.2.5.3-1: UE Activity Procedure

1a.	The AMF receives (N1) NAS signalling implying UE reachability, e.g. a Registration Request or Service Request message from the UE, or;

1b.	The AMF receives a (N2) UE Notification (see clause 8.4.3) or a (N2) Path Switch Request (see clause 4.9.1.2) from the RAN.

2.	If the AMF has an MM context for the UE and the URRP-AMF is set to report once that the UE is reachable, the AMF initiates the Namf_EventExposure_Notify service operation (SUPI, UE-Reachable) message to the UDM or directly to the NF (if previously indicated to AMF in step 2 in clause 4.2.5.2). AMF clear the corresponding URRP-AMF for the UE.

2a.	When the UDM receives the Namf_EventExposure_Notify service operation (SUPI, UE-Reachable) message or Nudm_UEContextManagement_Registration service for a UE that has URRP-AMF set, it triggers appropriate notifications to the NFs (e.g. SMSF or SMS-GMSC) that have subscribed to the UDM for this notification. UDM clears the URRP-AMF for the UE.

***** End of 1st Change *****

***** 2nd Change *****

### 4.8.3	N2 Notification procedure

This procedure is used by an AMF to request the RAN to report RRC state information, when the target UE is in CM-CONNECTED state. When AMF has requested reporting of subsequent state changes, the need for the RAN to continue reporting ceases when the UE transitions to CM-IDLE or the AMF sends a cancel indication. This procedure may be used for services that require RRC state information (e.g. 5GC MT control and paging assistance, O&M and collection of statistics), or for subscription to the service by other NFs.

Reporting of RRC state transitions can be requested per UE by AMF. Continuous reporting of all RRC state transitions can be enabled by operator local configuration.

Figure 4.8.3-1: RRC state transition notification

1.	The AMF sends a UE State Transition Notification Request message to the RAN. The UE State Transition Notification Request message shall identify the UE for which notification(s) are requested, and may contain a reporting type. The reporting type either indicates subsequent state transitions shall be notified at every RRC state transition, or it indicates Single RRC-Connected state notification.

Editor's note:	This request can either be sent as an indication when the UE context is established towards RAN, or after the UE context has been established. Details concerning N2 messages and the information contained in these messages will be aligned with RAN WG3.

2.	The RAN sends the UE Notification message to report the current RRC state for the UE (i.e. RRC Inactive state or RRC Connected state). The current UE location information (i.e. TAI + Cell Identity) is always included when RRC state information is reported.

2b.	When the AMF has requested reporting about subsequent state transitions, the RAN sends subsequent UE Notification messages to the AMF at every RRC state transition until the UE transitions to CM-IDLE or RAN receives a Cancel UE State Notification message from the AMF.

When the AMF has requested reporting for Single RRC-Connected state notification and UE is in RRC-Connected state, the RAN sends one UE Notification message but no subsequent messages. If UE is in RRC-Inactive state, the RAN sends one UE Notification message plus one subsequent UE Notification message when RRC state transits to RRC-Connected.

3.	The AMF can send a Cancel UE State Notification message to inform the RAN that it should terminate notifications for a given UE. This message should only be used when notification(s) about subsequent state transitions was requested at every RRC state transition.

Editor's note:	Details concerning RRC and N2 messages and the information contained in these messages will be aligned with RAN WG2 and RAN WG3.

***** End of 2nd Change *****