SA WG2 Meeting #123	S2-178303

Nov. 27 – Dec.1, 2017, Reno, USA

Source:	OPPO

Title:	Handover Cancel Procedure between 5GS and EPS

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution adds the handover cancel procedures between 5GS and EPS.

- Discussion

This contribution provides the handover cancel procedure between 5GS and EPS.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.502.

*************** Start of changes *********************

4.11.1.2.x   Handover cancel using N26 interface

4.11.1.2.x.y1  General

Instead of completing the handover procedure, the source AN node (NR-RAN, RAN) may at any time during the handover procedure, up to the time when a handover command message is sent to the UE cancel the handover. The reason for cancelling may be e.g. due to a timer expiration or due to other events within the source RAN node and is initiated by sending a handover cancel message to the source CN node (AMF or MME).

A handover cancel message shall also be sent by the source AN node after a handover command message is sent to the UE for the case where the handover fails and the UE returns to the old cell or radio contact with the UE is lost. This is done in order to release the resources reserved for the Handover in the target system.

4.11.1.2.x-y1 and Figure 4.11.1.2.x-y2 descripts the handover cancel procedure between 5GS to EPS when N26 is supported.

4.11.1.2.x.y2  Handover cancel from 5GS to EPS

This sub-clause describes the handover cancel procedure from 5GS to EPS.

Figure 4.11.1.2.x-y1: Handover Cancel procedure from 5GS to EPS

1. When the source NG-RAN decides to cancel the handover to the target EPS system, the source NG-RAN initiates Handover Cancel message to the AMF, optionally, including the cancel reason in the message.

2. After receiving the Handover Cancel message from the source NG-RAN, the AMF initiates Relocation Cancel message to the target MME requesting cancel the relocation procedure.

3. The target MME deletes the session established during handover preparation phase to the serving GW.

4. The target MME initiates handover cancel procedure towards the target E-UTRAN, and release the S1 connection established during the handover preparation phase. The target E-UTRAN releases the AN resources allocated for the handover.

5. The target MME responses the relocation cancels towards the source AMF.

6. The AMF responses the handover cancels towards the source NG-RAN.

7. If indirect forwarding tunnel is setup during handover preparation then cancellation of handover triggers the target MME to release the temporary resources used for indirect forwarding in the serving GW.

8. If indirect forwarding tunnel is setup during handover preparation then cancellation of handover triggers the source AMF to release the temporary resources used for indirect forwarding in the SMF and UPF.

4.11.1.2.x.y3  Handover cancel from EPS to 5GS

This sub-clause describes the handover cancel procedure from EPS to 5GS.

Figure 4.11.1.2.x-y2: Handover Cancel procedure from EPS to 5GS

1. When the source E-UTRAN node decides to cancel the handover to the target 5GS system, the source E-UTRAN node initiates Handover Cancel message to the source MME, optionally, including the cancel reason in the message.

2. After receiving the Handover Cancel message from the source E-UTRAN node, the MME initiates Relocation Cancel message to the target AMF requesting cancel the relocation procedure.

3. The target AMF deletes the session established during handover preparation phase to the SMF.

4. The target AMF initiates handover cancel procedure towards the target NG-RAN, and release the N2 connection established during the handover preparation phase. The target NG-RAN releases the AN resource allocated for the handover.

5. The target AMF responses the relocation cancels towards the source MME.

6. The source MME responses the handover cancels towards the source E-UTRAN.

7. If indirect forwarding tunnel is setup during handover preparation then cancellation of handover triggers the source MME to release the temporary resources used for indirect forwarding in the serving GW.

8. If indirect forwarding tunnel is setup during handover preparation then cancellation of handover triggers the target AMF to release the temporary resources used for indirect forwarding in the SMF and UPF.

*************** End of changes *********************