SA WG2 Meeting #124	S2-178383

Nov 27 – Dec 1, 2017, Reno, USA

Source:	China Mobile

Title:	Proposal for Network slicing and Interworking with EPS

Document for:	Discussion / Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes how to select the target MME by the AMF when network slice interworks with the EPS DCN in TS 23.501.

- Discussion

The NSSF entity can store the mapping information between network slice and DCN, when a UE is CM-CONNECTED in 5GC and a handover from network slice to EPS DCN occur, the AMF can select the target MME based on the mapping information between network slice and DCN stored in the NSSF or local configuration. So the related Editor’s note can be removed.

2.	Proposal

It is propose to include the following changes into the TS 23.501.

* * * * First Change * * * *

5.15.7.3	CONNECTED Mode aspects

In addition to the interworking principles documented in clause 5.17.2 the following applies for interworking with N26:

-	When a UE is CM-CONNECTED in 5GC and a handover to EPS occur, the AMF selects the target MME based on the mapping information between network slice and DCN stored in the NSSF or local configuration and forwards the UE context to the selected MME over the N26 Interface. The handover procedure is executed as documented in TS 23.502 [3]. When the Handover completes the UE performs a Tracking Area Update. This completes the UE registration in the target EPS. As part of this the UE obtains a DCN-ID if the target EPS uses it.

-	When a UE is ECM-CONNECTED in EPC, and performs a handover to 5GS, the MME selects the target AMF based on target location information, e.g. TAI and any other available local information (including the UE Usage Type if one is available for the UE in the subscription data) and forwards the UE context to the selected AMF over the N26 interface. The handover procedure is executed as documented in TS 23.502 [3]. When the Handover completes the UE performs a Registration procedure. This completes the UE registration in the target 5GS and as part of this the UE obtains an Allowed NSSAI.

Editor's note:	How to determine the S-NSSAIs for the sessions moved from EPC is FFS.

Editor's note:	Interworking with (e)DECOR-enabled EPC networks without N26 is FFS when slicing is supported.

* * * * Next Change * * * *

6.2.14	NSSF

The Network Slice Selection Function (NSSF) supports the following functionality:

-	Selecting the set of network slice instances serving the UE,

-	Determining the Allowed NSSAI and, if needed, the mapping to the Subscribed S-NSSAIs,

-     Storing the mapping information between network slice and DCN,

-	Determining the AMF Set to be used to serve the UE, or, based on configuration, a list of candidate AMF(s), possibly by querying the NRF.

* * * * End of Changes * * * *