SA WG2 Meeting #123	S2-179456

Nov. 27 – Dec.1, 2017, Reno, USA

Source:	OPPO

Title:	Correction and Clarification on QoS

Document for:	Approval

Agenda Item:	6.5.5

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution corrects the description on QoS part.

- Discussion

Change 2

In section 5.7.1.4, there is duplication description, it is proposed to improve the wording.

A default QoS rule is required for every PDU Session and associated with the QoS Flow of the default QoS rule. The default QoS rule is the only QoS rule of a PDU Session that may contain no Packet Filter Set in which case, the highest precedence value shall be used). If the default QoS rule does not contain a Packet Filter Set, the default QoS rule defines the treatment of packets that do not match any other QoS rule in a PDU Session. If the default QoS rule does not contain a packet filter, the reflective QoS shall not be applied to the QoS Flow of the default QoS rule.

As long as the default QoS rule does not contain any packet filter, reflective QoS should not be applied for the QoS Flow belonging to the default QoS rule and the RQA shall not be sent for this QoS Flow.

Change 3

The GFBR and MFBR should be calculated in UE, UE should be removed in the sentence.

The Averaging window is defined only for GBR QoS Flows. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. (R)AN, UPF, UE). The averaging window may be signalled with 5QIs to the (R)AN and UPF, and if it is not received a standardized value applies (for standardized 5QIs the value in the QoS characteristics Table 5.7.4-1 applies).

Change 4

In section 5.8.2.7, the PDU Session AMBR should also be provided to the UE, the current texture misses this part.

The SMF shall provide the PDU Session AMBR value together with a QoS Enforcement Rule correlation ID to the UPF so that the UPF can enforce the PDU Session AMBR across all non GBR QoS Flows of the PDU Session.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.501.

******************* 4th of changes *********************

#### 5.8.2.7	PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall use the 5QI and optionally, the ARP priority level, to determine the transport level packet marking and provide the transport level packet marking to the UPF.

The SMF shall provide the PDU Session AMBR value to the UE and together with a QoS Enforcement Rule correlation ID to the UPF, so that the UPF and the UE can enforce the PDU Session AMBR across all nonGBR QoS Flows of the PDU Session.

SMF shall provide the GFBR and MFBR value for each QoS Flow of the PDU Session to the UPF.

*************** End of changes *********************