SA WG2 Meeting #124	S2-179582

Nov 27 – Dec 1, 2017, Reno, USA	(revision of S2-179086)

Source:	China Mobile, CATR, Huawei, ZTE, CATT, KDDI

Title:	Proposals for RRC connection release with redirection for Connected Mobility for IWK

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1/ Rel15

Abstract of the contribution:

Proposes to add RRC connection release with redirection for Connected Mobility between 5GC and EPC with/without N26 support.

- Discussion

This paper is to clarify:

For connected-mode mobility from 5GC to EPC, either inter-system handover or RRC connection release with redirection to E-UTRAN may be performed. While for connected-mode mobility from EPC to 5GC, either inter-system handover or RRC connection release with redirection to NG-RAN may be performed.

# 2	 Proposal

It is proposed to adopt the following in TS 23.501.

* * * * Begin First Change * * * *

5.17.2.2	Interworking Procedures with N26 interface

5.17.2.2.1	General

Interworking procedures using the N26 interface, enables the exchange of MM and SM states between the source and target network. When interworking procedures with N26 is used, the UE operates in single-registration mode. The network keeps only one valid MM state for the UE, either in the AMF or MME. Either the AMF or the MME is registered in the HSS+UDM.

The support for N26 interface between AMF in 5GC and MME in EPC is required to enable seamless session continuity (e.g. for voice services) for inter-system change.

NOTE:	When applying the AMF planned removal procedure or the procedure to handle AMF failures (see clause 5.21.2) implementations are expected to update the DNS configuration to enable MMEs to discover alternative AMFs if the MME tries to retrieve a UE context from an AMF that has been taken out of service or has failed. This addresses the scenario of UEs performing 5GC to EPC Idle mode mobility and presenting a mapped GUTI pointing to an AMF that has been taken out of service or has failed.

5.17.2.2.2	Mobility for UEs in single-registration mode

When the UE supports single-registration mode and network supports interworking procedure with the N26 interface:

-	For idle-mode mobility from 5GC to EPC, the UE performs TAU procedure with EPS GUTI mapped from 5G-GUTI sent as old Native GUTI. The MME retrieves the UE's MM and SM context from 5GC if the UE has a PDU session established or if the UE or the EPC support "attach without PDN connectivity". The UE performs an attach procedure if the UE is registered without PDU session in 5GC and the UE or the EPC does not support attach without PDN connectivity. For connected-mode mobility from 5GC to EPC, either inter-system handover or RRC connection release with redirection to E-UTRAN is performed. During the TAU or Attach procedure the HSS+UDM cancels any AMF registration.

-	For idle-mode mobility from EPC to 5GC, the UE performs registration procedure with the EPS GUTI sent as the old GUTI. The AMF and SMF retrieve the UE's MM and SM context from EPC. For connected-mode mobility from EPC to 5GC, either inter-system handover or RRC connection release with redirection to NG-RAN is performed. During the Registration procedure, the HSS+UDM cancels any MME registration.

* * * * End First Change * * * *

* * * * Begin Second Change * * * *

5.17.2.3	Interworking Procedures without N26 interface

…. snip…

5.17.2.3.4 Redirection for UEs in connected mode

When the UE supports single-registration mode or dual-registration mode without N26 interface:

-	If the UE is in CM-CONNECTED status in 5GC, the NG-RAN may perform RRC connection release with redirection to E-UTRAN based on certain criteria (e.g. based on local configuration in NG-RAN, or triggered by the AMF upon receiving handover request from NG-RAN).

-	If the UE is in ECM-CONNECTED status in EPC, the E-UTRAN may perform RRC connection release with redirection to NG-RAN based on certain criteria (e.g. based on local configuration in E-UTRAN, or triggered by the MME upon receiving handover request from E-UTRAN).

* * * * End Second Change * * * *