SA WG2 Meeting #124	S2- 179583

November 27 – December 1, 2017, Reno, USA

Source:	CATT, China Mobile, ZTE

Title:	OI#6 TS 23.502 Update EPS bearer ID allocation procedure to support EBI re-assignment

Document for:	Discussion / Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to update the procedure of allocating EPS bearer ID.

- Discussion

As agreed in the last SA2 meeting, EBI re-assignment needs to be supported, we updated the procedure of EPS bearer ID allocation to support EBI exhaustion handling.

2.	Proposal

It is propose to include the following changes into the TS 23.502.

* * * * First Change * * * *

4.11.1.x	Procedures for EPS bear ID allocation

Following procedures are updated to allocate EPS bearer ID(s) towards EPS bearer(s) mapped from QoS flow(s):

-	Non-roaming and Roaming with Local Breakout (clause 4.3.2.2.1)

-	Home-routed Roaming (clause 4.3.2.2.2)

-	UE or network requested PDU session modification (non-roaming and roaming with local breakout) (clause 4.3.3.2)

-	UE or network requested PDU session modification (home-routed roaming) (clause 4.3.3.3)

Figure 4.11.3-1: Procedures for EPS bear ID allocation

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2. 	If the SMF determines that EPS bearer ID(s) needs to be assigned to the QoS flow(s) in the PDU session, SMF invokes Namf_Communication_EBIAssignment Request (PDU session ID, ARP list).

3.	[Conditional] If the AMF is requested to assign an EBI for QoS flow(s) for high priority service, i.e. with high ARP, but has no available EBIs, the AMF may revoke an EBI that was assigned to QoS flow(s) based on the ARP(s) received in step 2, EBIs information in the UE context and local policies. If an assigned EBI is to be revoked, the AMF invokes Nsmf_PDUSession_Update SM Context (EBI(s) to be revoked) to request the related SMF to release the mapped EPS QoS parameters corresponding to the EBI to be revoked.

4.	The SMF that receives the request in step 3 shall invoke Namf_Communication_N1N2Message Transfer (N2 SM information (PDU Session ID, EBI(s) to be revoked), N1 SM container (PDU Session Modification Command (PDU Session ID, EBI(s) to be revoked))) to inform the (R)AN and the UE to remove the mapped EPS QoS parameters corresponding to the EBI(s) to be revoked. In home routed roaming case, the Namf_Communication_N1N2Message Transfer is invoked by the V-SMF based on the message received from H-SMF.

5.	If the UE is in CM-CONNECTED state, the AMF sends N2 PDU Session Request (N2 SM information received from SMF, NAS message (PDU Session ID, N1 SM container (PDU Session Modification Command))) Message to the (R)AN.

If the UE is in CM-IDLE state and an ATC is activated, the AMF updates and stores the UE context based on the Namf_Communication_N1N2MessageTransfer and step 5-6 are skipped. When the UE is reachable, e.g. when the UE enters CM-CONNECTED state, the AMF forwards the N1 message to synchronize the UE context with the UE.

6.	The rest steps of the procedure are executed as specified in the figure above.

7	If the AMF successfully assigns EBI(s), it responds with the assigned EBI(s). Otherwise, it responds with a cause indicating EBI assignment failure.

8.	[Conditional] If the SMF that invoked Namf_Communication_EBIAssignment Request receives any EBI(s) from the AMF, it needs to include the received EBI(s) into the mapped EPS QoS parameters to be sent to the UE.

9.	The relevant steps of the procedure as specified in the figure above are executed.

Editor’s Note: It is FFS in which step EBI is provided to NG-RAN

* * * * 2nd Change * * * *

* * * * 3rd Change * * * *

Nsmf_PDUSession_Update SM Context service operation

Service operation name: Nsmf_PDUSession_Update SM Context.

Description: It allows to update the AMF-SMF association to support a PDU Session and/or to provide SMF with N1/N2 SM information received from the UE or from the AN.

Input, Required: SUPI, PDU Session ID.

Input, Optional: N1 SM container received from the UE, N2 SM information received from the AN (e.g. N3 addressing information, notification indicating that the QoS targets cannot be fulfilled for a QFI), Operation type (e.g. activate, deactivate), UE location information, AN type, UE Time Zone, EBI(s) to be revoked.

Output, Required: Result Indication.

Output, Optional: Cause, EBI list, N2 SM information (e.g. QFI, UE location information, notification indication indicating that the QoS targets cannot be fulfilled), N1 SM container to be transferred to the AN/UE.

See clause 4.3.3.2 and clause 4.3.3.3 for an example usage of this service operation.

If the consumer NF is AMF and the SMF determines that some EBIs are not needed, the SMF will put the EBIs back.

* * * * End of Changes * * * *