SA WG2 Meeting #124	S2-179180

November 27 – December 1, 2017, Reno, NV, USA	(was S2-178852)

Source:	ITRI

Title:	Update to 23.502: Removal of FFS in Section 4.12.3 Deregistration procedure for untrusted non-3gpp access

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5GS_Ph1/ Rel15

Abstract of the contribution: .This contribution proposes to remove Editor’s note in Section 4.12.3 in TS 23.502.

1	Discussion

This contribution aims to remove the following Editor’s note in Section 4.12.3:

- It is FFS whether the N3IWF can initiate the deregistration procedure by sending UE Release (UE identity) message to the AMF via N2 signalling when the IKEv2 tunnel is release.

In section 5.5.3.2 of TS 23.501, the description of UE reachability in CM-CONNECTED,

- N3IWF releases the N2 connection when UE becomes unreachable from N3IWF point of view, i.e. upon NWu release.

The above implies that UE is disconnected from the network and the DL data cannot be transmitted from the network via N3IWF. Therefore, N3IWF release the N2 connection with AMF. Based on N2 connection, it can imply AMF that the AMF can trigger Deregistration procedure. Thus, the above editor note that N3IWF can initiate the deregistration is not necessary. This editor’s note can be directly deleted without adding new text.

2	Proposal

It is proposed to adopt the following content in TS 23.502.

* * * * (First Change) * * * *

4.12.3	Deregistration procedure for untrusted non-3gpp access

Figure 4.12.3-1: Deregistration procedure for untrusted non-3gpp access

1.	The Deregistration procedure is triggered by one of the events:

1a.	For UE initiated deregistration as in steps from A to B of Figures 4.2.2.3.1-G.

1b.	For AMF initiated deregistration as in steps from C to D of Figures 4.2.2.3.2-H.

1c.	For UDM initiated deregistration as in steps from E to F of Figures 4.2.2.3.3-I.

2.	The AMF sends a N2 Context UE Release Command message to the N3IWF as defined in step J of clause 4.7.K.

3.	The N3IWFsends INFORMATIONAL EXCHANGE (Delete payload) message to the UE. Delete payload is included to indicate the release of the IKE SA.

4.	The UE sends an empty INFORMATIONAL EXCHANGE message to acknowledge the release of the IKE SA as described in RFC 7296 [3]. Non-3GPP access specific resources are released including the IKEv2 tunnel (and the associated IPSec resources) and the local UE contexts in N3IWF (N3 tunnel Id).

5.	The N3IWF acknowledges the N2 UE Context Release Command message by sending N2 UE Context Release Complete message to the AMF.

Editor's note: The details of the messages (e.g. Deregistration Request, Deregistration Response, Deregistration Indication, etc.) including message name and parameters are FFS and should be align with Deregistration procedure via 3GPP access.

* * * * End of First Change * * * *