# SA WG2 Meeting # SA WG2 Meeting #124	S2-179627

# 27 November - 01 December, 2017,Reno, Nevada, USA	(e-mail revision 1 of S2-179556)

Source:	China Mobile, KDDI, Verizon, Deutsche Telekom

Title:	23.501: Resolving Editor's Note on per-session binding information storage problem of PCF BSF

Document for:	Approval

Agenda Item:	6.5.6

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution:  This proposal is to solve the Editor's Note on per-session binding information storage problem of PCF BSF in 6.3.7.3 of TS 23.501.

# Introduction

Based on the discussion of last meeting on the issue mentioned in S2-177477, the alternative solution proposal is moved into the editor’s note in 6.3.7.3 shown in S2-178164. That is because some companies need more time to analyse this proposal.

In this meeting, based on analysis of discussion paper S2-177477 last meeting, we suggest to move the proposal into the main text.

# Conclusion and Proposal

It is proposed to adopt the changes in TS 23.501.

* * * First Change * * *

#### 6.3.7.1	PCF selection for a UE or a PDU Session

The AMF selects the PCF for a UE, the SMF selects the PCF for a PDU Session. The selected PCF may be the same or may be a different one, following one of the alternatives below:

-	The AMF utilizes the NRF to discover the PCF instance(s) for a UE unless PCF information is available by other means, e.g. locally configured on AMF based on operator policies.

-	The SMF utilizes the NRF to discover the PCF instance(s) for a PDU Session unless PCF information is available by other means, e.g. locally configured on SMF or received from the AMF. The following factors may be considered during the PCF selection by the SMF:

a)	Local operator policies.

b)	Selected Data Network Name (DNN).

c)	PCF selected by the AMF. This is to select the same PCF for the AMF and the SMF.

#### 6.3.7.2	Providing policy requirements that apply to multiple UE and hence to multiple PCF

An authorized Application Function may, via the NEF, provide policy requirements that apply to multiple UE (which, for example, belong to group of UE(s) defined by subscription or to any UE) and hence may apply to multiple PCF.

NOTE:	Application Function influence on traffic routing described in clause 5.6.7 is an example of such requirement.

The NEF, after relevant validation of the application request (and possible parameter mapping) stores the request from the application. When the UDR serving the NEF is deployed, NEF stores the request from the application info into the UDR. The possible parameter mapping includes mapping UE (group) identifiers provided by the AF to identifiers used within the 5GC, e.g. from GPSI to SUPI, from External Group Identifier to IMSI-Group Identifier.

PCF(s) that need to receive application requests that targets a DNN (and slice) and / or a group of UE subscribe to receive notifications from the NEF about such group information.

Editor's note:	It is FFS whether the PCF(s) can subscribe to/receive notification of such application request information from the UDR.

Editor's note:	Slicing aspects are FFS.

The NEF associates the request with information allowing to later modify and delete the application request; it associates the AF request with:

-	When the request targets PDU Sessions established by "any UE": the DNN and slicing information target of the application request,

-	When the request targets PDU Sessions established by UE within a predefined/subscribed group of UE: the DNN, the slicing information and the group of UE target of the application request.

All PCF that serve the DNN and slicing information are triggered by this policy modification induced by the AF request and take it into account for existing and future PDU Sessions that match the policy change. In case of existing PDU Sessions the change of policy may trigger an N7 rule change from the PCF to the SMF.

#### 6.3.7.3	Binding an AF request targeting an IP address to the relevant PCF

The BSF is used for binding an AF request to the relevant PCF as described in TS 23.503 [45].

* * * End of Change * * *