SA WG2 Meeting #125	S2-180293

January 22 – 26, 2018, Gothenburg, Sweden

Source:	OPPO

Title:	Emergency Services Support indication per RAT

Document for:	Approval

Agenda Item:	6.5.8

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution discusses whether Emergency Service Support indication per RAT is needed.

- Discussion

In current TS 23.501, it is FFS whether the Emergency Services Support indicator is per RAT.

When the UE is camped normally in the cell, during Registration procedure described in TS 23.502 [3] clause 4.2.2.2, the serving AMF includes an indication for Emergency Services Support within the Registration Accept to the UE. The Emergency Services Support indication is valid within the current Registration Area per RAT (i.e. this is to cover cases when the same registration area supports multiple RATs and they have different capability).

Editor's note: need for per RAT indicated will need to be confirmed.

Different RAT, e.g. NR RAN and E-UTRA connected to 5GC may be deployed in one RA, while different RAT may have different capability to support the emergency services. In this case, the UE behaviour when UE camps on the different RAT could be different when initiating an emergency service. The following table elaborates the UE behaviour considering the emergency support capability per RAT.

| NR support | E-UTRA support | UE camping | UE behaviour |
| --- | --- | --- | --- |
| Y | Y | NR | Initiating PDU session establishment for emergency |
|  |  | E-UTRA | Initiating PDU session establishment for emergency |
| Y | N | NR | Initiating PDU session establishment for emergency |
|  |  | E-UTRA | Service Request with fallback or emergency session domain selection |
| N | Y | NR | Service Request with fallback or emergency session domain selection |
|  |  | E-UTRA | Initiating PDU session establishment for emergency |
| N | N | NR | Service Request with fallback or emergency session domain selection |
|  |  | E-UTRA | Initiating PDU session establishment for emergency |

From the above table, it can be seen that UE behaviour is different when NR or E-UTRA connected to 5GC have different capability to support emergency services, therefore, the Emergency Services Support indication per RAT is confirmed to be needed.

Proposal: The Emergency Services Support indication per RAT is confirmed to be needed.

2	Proposal

In this contribution, whether the Emergency Services Support indication per RAT is needed is discussed and provides the proposal.

Proposal: The Emergency Services Support indication per RAT is confirmed to be needed.

3	Reference

[1] TS 23.501 vf00.