SA WG2 Meeting #S2-125	S2-180455

22 - 26 January 2018, Gothenburg, Sweden	(revision of S2-17xxxx)

Title:	[DRAFT] LS reply on Mapping of Conversational Services to 5G System

Response to:	S4-171278 / S2-180010

Release:	Rel-15

Work Item:	5GS_Ph1 / FS_5G_MEDIA_MTSI

Source:	SA2

To:	SA4

Cc:	SA1

Contact person:	Sašo Stojanovski

(saso stojanovski intel com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

SA2 would like to thank SA4 for their LS on mapping of conversational services to 5G System in S4-171278 / S2-180010.

With respect to the SA4 question:

3GPP SA4 kindly asks SA2 to confirm SA4’s assessment that there are no 5G architecture implications on 3GPP conversational services beyond the AF-grouping mentioned above.

SA2 would like to point out that the use of 5G System to access IMS in Rel-15 has been specified in TS 23.228 Annex Y. In addition, PCC aspects are described in TS 23.503.

As described in the specifications referenced above, for Rel-15 it is assumed that the IMS can be accessed via 5G System without any changes to the IMS. The IMS subsystem connects to the underlying 5G System using the Sh/Cx interface towards the UDM/HSS and the N5 or Rx interface to the PCF.

SA2 would like to point out that the N5 reference point is a point-to-point representation of the service-based (HTTP-based) interface Npcf/Naf. While both N5 and Rx can be used for communication between the IMS and the PCF, it is noted that in Rel-15 the two reference points are functionally equivalent.

Any functional grouping within the IMS as implied by TR 26.919 are transparent to the 5G System as long as the reference points N/Rx, Sh and Cx remain unchanged.

SA2 would also like to point out that for Rel-16 work is just starting on a new study SP-171052 on enhanced 5GC to IMS integration (FS_eIMS5G). It is too early to say whether this new study will have any impact on the FS_5G_MEDIA_MTSI study.

# 2	Actions

To SA4:

ACTION: SA2 kindly asks TSG SA4 to take the information above into account.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 126	26 Feb – 2 Mar 2018	Montreal, CA

TSG SA WG2 Meeting 127	16-20 Apr 2018	CN