3GPP TSG-SA2 Meeting #126	S2-181588

# Montreal, Canada, 26th Feb 2018 - 2nd Mar 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0042 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on non-Standardized 5QI provided over N2 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A 5QI is included as a mandatory parameter of a QoS profile. The QoS profile is used to provide QoS flow level QoS parameters to AN. However, for a QoS Flow with a dynamically assigned 5QI, the 5G QoS characteristics are signalled in the QoS profile. The dynamically assigned 5QI value is not used at AN, so it is also not needed to be signalled to AN in the QoS Profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the 5QI is a mandatory parameter of a QoS profile only when the standardized or pre-configured 5QI is used. For a QoS Flow with a dynamically assigned 5QI, the dynamically assigned 5QI value is not used at AN, so it is also not needed to be signalled to AN in the QoS Profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The dynamically assigned 5QI value is not used at AN but is mandatory provided in the QoS profile. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************Start of 1st change*****************************************

5.7.1.2	QoS Profile

A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS Flow contains QoS parameters as described below (details of QoS parameters are described in clause 5.7.2):

-	For each QoS Flow, the QoS profile shall include QoS parameters:

-	A 5G QoS Identifier (5QI) if a standardized or pre-configured 5QI is used; and.

-	An Allocation and Retention Priority (ARP).

-	For each Non-GBR QoS Flow, the QoS profile may also include the QoS parameter:

-	Reflective QoS Attribute (RQA).

-	For each GBR QoS Flow, the QoS profile shall also include the QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL; and

-	Maximum Flow Bit Rate (MFBR) - UL and DL; and

-	In case of a GBR QoS Flow only, the QoS parameters may also include:

-	Notification control.

-	Maximum Packet Loss Rate - UL and DL.

NOTE:	In this Release, the Maximum Packet Loss Rate (UL, DL) is only provided for a GBR QoS flow belonging to voice media.

Each QoS profile has one corresponding QoS Flow identifier (QFI) which is not included in the QoS profile itself.

For a QoS Flow with a dynamically assigned 5QI, the 5G QoS characteristics are signalled as part of the QoS profile. Details of QoS characteristics are described in clause 5.7.3. The dynamically assigned 5QI value is not included in the QoS profile as it is not used at AN.

**************************************End of changes*******************************************