3GPP TSG-SA2 Meeting #126	S2-181590

# Montreal, Canada, 26th Feb 2018 - 2nd Mar 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0009 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS rule generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It’s not clear how SMF generates QoS rule, as we have agreed “the QoS rule precedence value is set to the precedence value of the PCC rule from which the QoS rule is generated”, for our understanding, the QoS rule provided by SMF is SDF level. The default QoS rule is generated from a PCC rule which the 5QI and ARP equal to the authorized default 5QI and ARP.We also need to clarify, for a PDU session of unstructured type, only one PCC Rule is provided to SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify each QoS rule provided by SMF is generated from a PCC rule. The default QoS rule is generated by SMF based on a PCC rule which the 5QI and ARP equal to the authorized default 5QI and ARP. Clarify for a PDU session of unstructured type, only one PCC Rule is provided to SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | it’s not clear how SMF generates the QoS rule and default QoS rule. For a PDU session of unstructured type, it’s not clear whether PCF needs to provide a PCC rule to SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Re-submit the agreed draft CR of S2-181056. |  |  |  |  |  |  |  |  |

**************************************Start of 1st change*****************************************

6.2.2.4	QoS control

The SMF receives the authorized QoS for a service data flow in the PCC rule. The SMF calculates the QoS parameters for a QoS Flow as described in TS 23.501 [2] clause 5.7.1. In addition, for GBR QoS Flows, the SMF should set the QoS Flow GBR to the sum of the GBRs of all PCC rules that are bound to that QoS Flow and the QoS Flow MBR to the sum of the MBRs of all PCC rules that are active and bound to that GBR QoS Flow.

The SMF receives the authorized Session AMBR and the default 5QI and default ARP in the PDU Session information. The SMF ensures that the authorized Session AMBR for a PDU Session is enforced for bandwidth policing at the UPF(s) as described in TS 23.501 [2] clause 5.7.1. For PDU Sessions of unstructured type the default 5QI and default ARP is the authorized QoS for all traffic in the PDU Session as described in TS 23.501 [2] clause 5.7.1.

The SMF generates each QoS rule based on a PCC rule which may be pre-defined or dynamic as described in TS 23.501 [2].During the PDU Session establishment, the SMF generates the default QoS rule based on the PCC rule in which the authorized default 5QI and ARP are used.

For a PDU session of unstructured type, only one PCC Rule allowing all packets is to be activated in the SMF.

**************************************End of changes*******************************************