3GPP TSG-SA2 Meeting #126	S2-181700

# Montreal, Canada, 26th Feb 2018 - 2nd Mar 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0103 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Npcf_Policy Authorization Delete Service Description Correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The response to the Npcf_PolicyAuthorization-Delete service operation does not require any optional or mandatory outputs. This prevents the PCF from including in the response information elements, such User Location Data, that is deemed essential in the 4G PCC specifications. Reference: 3GPP 29.214 Message Format: <ST-Answer> ::= < Diameter Header: 275, PXY > < Session-Id > { Origin-Host } { Origin-Realm } [ Result-Code ] [ Error-Message ] [ Error-Reporting-Host ] [ OC-Supported-Features ] [ OC-OLR ] [ Failed-AVP ] [ Sponsored-Connectivity-Data ] [ Origin-State-Id ] [ 3GPP-User-Location-Info ] [ User-Location-Info-Time ] [ 3GPP-MS-TimeZone ] *[ RAN-NAS-Release-Cause ] [ 3GPP-SGSN-MCC-MNC ] [ TWAN-Identifier ] [ Netloc-Access-Support ] *[ Class ] *[ Redirect-Host ] [ Redirect-Host-Usage ] [ Redirect-Max-Cache-Time ] *[ Proxy-Info ] *[ AVP ] |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of the Npcf_PolicyAuthorization_Delete service operation to include a list of information elements in the optional output section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# * * * First change* * * *

##### 5.2.5.3.4	Npcf_PolicyAuthorization_Delete service operation

Service operation name: Npcf_PolicyAuthorization_Delete

Description: Provides means for the NF Consumer to delete the context of application level session information.

Inputs, Required: Identification of the application session context.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: - Sponsored Connectivity Data, 3GPP User Location-Info, User Location Info Time, 3GPP MS Time Zone, Netloc Access Support

Provides means for the NF Consumer to delete the context of application session information

* * * End of the changes* * * *