SA WG2 Meeting #126	S2- 182237

26 February – 2 March 2018, Montreal, Canada	(revision of S2-18xxxx)

Source:	Ericsson

Title:	PCR 23.724: Infrequent Small Data user plane transmission for CIoT

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_CIoT_5G / Rel-16

Abstract: The solution presented below describes how small data can be transmitted using the user plane without overhead of signalling caused by CIoT UE idle-connected mode transitions. This solution can handle infrequent small data transmissions as of the traffic model defined in [1].

# Discussion

This solution addresses the Key Issue 1: Support for infrequent small data transmission. The proposition is based on setting up a special PDU session tailored for small data transmission. The UE used for CIoT is, during PDU session establishment, provided with information from the SMF about the end-point of the UPF connection and relevant QoS flow(s) to be used for the connection. The UE can then append this information to small data that is sent UL to the RAN. The RAN uses the information and passes the small data packet via N3 to the UPF. The UPF then verifies the received packet using a security context setup during PDU session establishment and forwards it as normal. To allow subsequent DL small data, the UPF uses the information received in the UL data over N3 from RAN.

The solution could also allow storing only the UE context in the SMF and rebuilding the UPF context on demand.

***** First Change *****

# 6	Solution

## 6.X	Small Data Fast Path communication

### 6.X.1	Introduction

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

This solution addresses the Key Issue 1: Support for infrequent small data transmission.

The Small Data Fast Path (SDFP) solution targets to optimize small data transmission for CIoT UEs in CM-IDLE mode. It describes how small data can be passed in a fast path of the user plane without the overhead amount of signalling caused by CM-IDLE / CM-CONNECTED mode transitions.

The SDFP solution has the following specifics:

- UP security is between UE and UPF. The security information is configured by NAS during UE registration or PDU session setup.

- The UE passes information required for the RAN to forward the small data to the UPF. The RAN derives the UPF information from the information provided by the UE.

Editor’s note: It is FFS how the N3 related information needed to pass the data to UPF is provided to RAN.

- One or several QoS flows can be enabled for SDFP. UE will indicate QoS information of the packet over radio interface, the RAN will forward the QoS information for verification by the UPF (i.e. that the UE is authorized to use a certain QoS flow).

- The solution support DL data using CN paging. (e)DRX parameters etc. is configured during UE registration. The solution can also support PSM if defined.

### 6.X.2	Functional Description

Editor's Note:	This clause outlines solution principles and documents any assumptions made.

#### 6.X.2.1	General

The Small data fast path is intended as an add-on function handled in a separate PDU session. It does not replace any existing system function, but it can be used if both the network and UE support it. This should also keep the impact on the system lower and simplify the introduction in Low Power Wide Area Networks (LPWAN).

Figure 1: E2E SDFP transmission

The SDFP is realized by providing relevant UPF or PDU session related information to the UE from the SMF, which the UE would later provide to the RAN. The UPF or PDU session relevant information enables the RAN to derive the path over N3 to the UPF. At UL data arrival the UE passes the data together with the UPF or PDU session relevant information to the RAN. The RAN forwards the data on the N3 interface. Since all info required to forward the data is received from the UE, the RAN does not need to signal to the AMF or have any UE context information stored.

Editor’s note: Potential impacts to N3 are FFS.

The UPF or PDU session relevant information is provided to the UE as part of the Session Management (SM) procedures. The SMF may enable one (or more) of the QoS flows for SDFP.

The AMF derives small data security information and provides it to the SMF. The SMF stores the security information for the PDU session, enables indicated QoS flows and acknowledges to the AMF that they have been SDFP enabled.

#### 6.X.2.2	UE requested SDFP PDU session establishment

Fig 3: UE-requested PDU Session Establishment for SDFP

The procedure assumes that the UE has already registered on the AMF. Below are some modifications for SDFP compared to the existing PDU Session Establishment procedure.

Editor’s note: More work is needed on the details of the modifications below. Alternative solutions to what is shown could also be considered.

- UE should provide an indication it wants to establish a SDFP PDU session. Additionally, we assume the UE could provide NSSAI indicating a specific network slice for SDFP which could be used by the RAN to select an AMF that supports SDFP.

- AMF selects an SMF that supports SDFP.

- The AMF will generate a SDFP security context, which will be forwarded to the SMF.

Editor’s note: The details on the security context is FFS.

…

8.	SMF selects a UPF that suppose SDFP.

…

10.	SMF performs N4 Session Establishment including setting up the SDFP security context in the UPF. During this procedure the UE UPF and PDU Session information will be generated.

Editor’s note: The details on the security context handling is FFS.

11.	The SMF will include the UE UPF and PDU Session information in an N1 SM message to the UE.

#### 6.X.2.2	SDFP transfer initiated by UL data

The following figure shows an example of how MO small data packet is passed from the UE in UL. The term "SDFP session" is used in the text below meaning the time from a fast path is established (e.g. DRB is created) until it is released (e.g. inactivity timeout)

Figure 3: SDFP transfer initiated by UL data

- The UE establishes an RRC for SDFP transfer. Parameters for selection of UPF for the PDU Session for the UE are passed to the RAN.

Editor’s note: The detailed solution for this part should be discussed in RAN WGs. An alternative would be to transfer the parameters as part of the UL data PDU.

2.	The UE encrypts, and integrity protects an UL data PDU and passes it to RAN.

3.	RAN forwards the UL data PDU to the selected UPF. RAN will also provide the UPF with RAN N3 DL Tunnel Info for the SDFP session.

4.	The UPF checks integrity protection and decrypts the UL data PDU. If passed the check, the UPF forwards the UL data on the N6/N9 interface. In addition, the UPF enables subsequent DL data transmissions to the RAN node it received the UP data PDU from.

5.	A DL data packet may arrive on N6/N9 shortly after, e.g. an acknowledgement.

6.	The UPF encrypt and integrity protects the DL data PDU and passes it to the RAN node, which was enabled in step 3-4.

7.	RAN forwards the DL data PDU to the UE.

8.	The SDFP information contexts are released in UE, RAN and DL data transmission is set in disabled state in the UPF.

Editor’s note: Further details are FFS.

#### 6.X.2.3	SDFP transfer initiated by DL data

Small data initiated in DL on a QoS flow where fast path is enabled but not active (i.e. the RAN N3 DL Tunnel Info is unknown), is handled as described in the figure below. It should be noted that compared to when small data is initiated UL, an additional paging of the UE is required.

Figure 3: SDFP transfer initiated by DL data

At arrival of DL data a network initiated service request procedure could be used to reach the UE. Two options can be considered for providing the UPF with the RAN N3 DL Tunnel Info, either it is carried from the RAN as part of a Service Request procedure towards the UPF or as part of a Dummy UL data procedure.

Editor’s note: More work is needed on the details of the SDFP transfer initiated by DL data.

### 6.X.3	Mobility

Mobility is support as for normal UEs in IDLE mode. When the UE enters a new TA currently not part of the UE registration area it will initiate a UE registration update. During the UE registration update the UPF or PDU session information could be updated towards the UE by the SMF e.g. if the UE context is re-allocated to a new UPF. SSC mode 2 mobility is probably appropriate for infrequent small data.

Editor's Note: There may be dependencies related to the ETSUN study item in case of change of SMF and/or UPF.

### 6. X.5	Security

Editor's Note: This clause describes impacts to existing entities and interfaces.

In the SDFP the UE sends traffic without having set up the regular AS security.

In the described solution the security protection is done by security contexts in the UE and the AMF/SMF/UPF. These are established as part of existing signalling, hence keeping the signalling overhead for small data to a minimum. The security information for the SDFP is stored in the SMF and UPF and UE as part of the PDU Session Resource Request information. It is kept regardless if the fast path is active or not, i.e. the security information is kept as long as the fast path remains enabled. When the security information is updated for the UE, the AMF updates the SMF with new security information for small data transfer.

The security protocol between the UE and the UPF consists of protocols similar to E-UTRA PDCP layer that reuses the integrity protection and ciphering functionality.

Editor's Note: Further details are needed related to the security context handling.

### 6. X.6	Evaluation

Editor's Note: This clause provides an evaluation of the solution.

The solution addresses infrequent small data transfer.

Benefits

-	Can be used for SDT over the user plane with a minimum signalling overhead, e.g. primarily for messaging applications, but also some applications using keep-alive signalling (M2M and Smartphones).

-	Existing UP functions is reused, e.g. passing data, charging, LI, IP address & IP routing, firewalls/NATs/VPNs, DPI, etc.

- 	No context is needed to be stored in the RAN for UEs when there is not active SDFP session. UE is in CM-IDLE state and can utilize existing IDLE mechanism (incl. Mobility).

***** End of Changes *****

# References

[1]	Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, 3GPP TS 45.820 v1.3.1 Release 13, June 2015.