SA WG2 Meeting #127bis	S2-185117

May 28 –June 1, 2018, Newport Beach, USA

Source:	Huawei, Hisilicon

Title:	UL-CL insertion when ULCL is in different region comparing to A-SMF

Document for:	Discussion/Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: This contribution discusses how the UL-CL is inserted when ULCL is in different region comparing to A-SMF.

- Discussion

When the UE is located in service area of I-SMF, and if the local service is available, an UL-CL and local PDU session anchor may be inserted to support the UE access the local service. There are 2 possibilities scenario for UL-CL insertion:

- The UL-CL and local PDU session anchor are controlled by I-SMF

- The UL-CL and local PDU session anchor are controlled by local SMF

The architecture that support I-SMF controls UL-CL and local PDU session anchor is as following:

Figure 1 UL-CL/BP insertion when UL-CL/BP is controlled by I-SMF

In this architecture, the UL-CL and local A-UPF that accesses the local DN are controlled by I-SMF.

The architecture that support local SMF controls UL-CL and local PDU session anchor is as following:

Figure 1 UL-CL/BP insertion when UL-CL/BP is controlled by Local SMF

In this architecture, the UL-CL and local A-UPF are controlled by Local SMF which is within the service area of I-SMF.

- Proposal

It is proposed to add the following text into TR 23.726.

/*************************** Start of the first change ************************/

6.7.2.3	UL-CL/BP in the different region as A-SMF

6.7.2.3.x1	UL-CL/BP controlled by I-SMF

For this scenario, the UPF that acts as UL-CL/BP is controlled by I-SMF.

The UL-CL/BP may be inserted during or after the PDU session establishment, e.g.:

- When UE moves to a location that supports to connect a local DN via UL-CL/BP; Or,

- When UE starts an application that has local deployment via the UL-CL/BP

Before the UL-CL/BP insertion, the I-UPF controlled by I-SMF is already inserted into the PDU Session path. After the UL-CL/BP insertion, the architecture is as in figure 6.7.x.1-1

Figure 6.7.2.3.x1.1-1 UL-CL/BP insertion when UL-CL/BP is controlled by I-SMF

In this architecture, the UL-CL/BP and local PSA are controlled by I-SMF. The I-SMF determines whether the UL-CL/BP needs to be inserted and determines the DNAI the UL-CL/BP connects to. The I-SMF selects the UL-CL/BP based on DNAI and UE location information.

6.7.2.3.x2	Procedures

6.7.2.3.x2.1	Insertion of UL-CL/BP controlled by I-SMF

The procedure in figure 6.7.x.2.1-1 is to insert a UL-CL/BP for a PDU session when the UE is located in service area of I-SMF and UL-CL/BP can be controlled by the I-SMF.

Figure 6.7.2.3.x2.1-1: Procedure to support UL-CL/BP insertion when UE is in the region of A-SMF

- Before the procedure, a PDU session anchored in A-SMF has been established. An I-UPF controlled by I-SMF has been inserted.

- The PCC rule related to AF request is provisioned to I-SMF. Two alternatives can be used to provision PCC rules to I-SMF as described in clause 6.13 or 6.15.

- From PCC rule the I-SMF may determine to insert a UL-CL/BP for the PDU session based on the received triggers. The triggers includes:

-	The UE moves to a new location based on the area of interest subscription (from AMF); or

-	Received trigger from PCF or from I-UPF that a new application has been detected (from PCF or I-UPF).

If UL-CL/BP is to be inserted, the I-SMF determines the DNAI that the UL-CL/BP will connect to.

The I-SMF selects a UL-CL/BP based on the DNAI and UE location information, and selects local PSA based on DNAI.

4~5. The I-SMF sends N4 Session Establishment request to UL-CL/BP and local PSA, to establish the PDU session.

6~8. I-SMF invokes Nsmf_PDUSession_Update to send tunnel info of UL-CL/BP to A-SMF, and the A-SMF updates the corresponding N4 session in A-UPF.

In case the I-SMF does not have direct interface with PCF, the I-SMF provides information related to UL-CL/BP to A-SMF, e.g. the DNAI selected, the local IP address in case of BP, etc. the A-SMF may provide such information to PCF.

In case the I-SMF has direct interface with PCF, this information can be provided to PCF directly, no need via A-SMF.

9.	I-SMF send SM N2 request to AN via AMF to update AN with the UL-CL/BP tunnel info (N3).

10. I-SMF releases PDU session in I-UPF.

6.7.2.3.y1	UL-CL/BP controlled by local SMF in I-SMF service area

For this scenario, the UE is in service area of I-SMF, but the UPF that acts as UL-CL/BP is controlled by a local SMF within the service area of I-SMF.

The UL-CL/BP may be inserted during or after the PDU session establishment, e.g.:

- When UE moves to a location that supports to connect a local DN via UL-CL/BP; Or,

- When UE starts an application that has local deployment via the UL-CL/BP

Before the UL-CL/BP insertion, the I-UPF controlled by I-SMF is already inserted into the PDU Session path. After the UL-CL/BP insertion, the architecture is as in figure 6.7.y.1-1

Figure 6.7.2.3.y1-1 UL-CL/BP insertion when UL-CL/BP is controlled by local SMF within I-SMF service area

In this architecture, the UL-CL/BP and local PSA are controlled by local SMF that is located within I-SMF service area. The I-SMF determines whether the UL-CL/BP needs to be inserted and determines the DNAI the UL-CL/BP connects to. The I-SMF selects the local SMF based on DNAI and UE location information.

6.7.2.3.y2	Procedures

6.7.2.3.y2.1	Insertion of UL-CL/BP controlled by local SMF within I-SMF service area

Editor’s Notes: the procedure to support insertion of UL-CL/BP controlled by local SMF within I-SMF service area is FFS.

******************************* End of Changes *********************************