SA WG2 Meeting #127-bis	S2-185275

May 28 – June 1, 2018, Newport Beach, USA	(was S2-18----)

Source:	CATT

Title:	UDM based Positioning Access selection for LCS service

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	FS_eLCS / Rel-16

Abstract of the contribution: This paper proposes a solution for KI#14 of Positioning Access selection for LCS service.

1.	Discussion

When UE accesses 5GC via 3GPP access and non-3GPP access simultaneously, the serving AMF may be same or different in the following scenarios:

- When served by the same PLMN for 3GPP and non-3GPP accesses, the UE is served by the same AMF. For example, the UE is registered via a 3GPP access and selects a N3IWF located in the same PLMN.

- When served by different PLMNs for 3GPP and non-3GPP accesses, the UE is served by two AMFs belonging to two different PLMNs. For example, the UE is registered to a 3GPP access and selects a N3IWF located in PLMN A which is different from the PLMN B of the 3GPP access, or the UE is registered over N3GPP access and registers to a 3GPP access in a PLMN different form the PLMN of the N3IWF.

For both scenarios, the UDM manages separate/independent UE Registration procedures for each access. For example, when UE initiates Registration procedure over 3GPP access, the AMF registers with the UDM by providing, e.g. AMF ID and Access Type. The UDM stores the AMF ID associated to the Access Type and does not remove the AMF ID associated to the other Access Type. So the UDM has the full view of accesses connected by UE and also records the serving AMF for each access.

It is proposed that UDM selects access used for positioning and a new solution is added into TR 23.731.

2.	Proposal

It is proposed to add a new solution for key issue#14 as following: (all new text)

*** Start of changes ***all new text

6.X	Solution #X: UDM based Positioning Access selection for LCS service

6.X.1	Introduction

This solution addresses the Key Issue 14: Positioning Access selection for LCS service.

6.X.2	Functional Description

When UE accesses 5GC via 3GPP access and non-3GPP access simultaneously, the UDM is responsible to manage separate/independent UE Registration procedures for each access. For example, when UE initiates Registration procedure over 3GPP access, the AMF registers with the UDM by providing, e.g. AMF ID and Access Type. The UDM stores the AMF ID associated to the Access Type and does not remove the AMF ID associated to the other Access Type.

When UDM receives request from GMLC, it is proposed that UDM decides access type to transmit the location information (e.g. positioning message) based on, e.g. access type and associated serving AMF which is provided by AMF during Registration procedure and the UE state change in the access type (e.g. connectivity state changes) which is obtained by subscribing UE access and mobility information event to AMF. The UDM provides Access Type to the GMLC.

6.X.3	Procedures

# Figure 6.X.3-1: Access Type selection for LCS service

1.	An external LCS client sends a location request to an HGMLC in the HPLMN for the target UE. The HGMLC verifies the authorization of the LCS Client to locate the target UE and UE privacy requirements.

2.	The HGMLC invokes the Nudm_UE ContextManagement_Get service operation towards the home UDM of the target UE with the GPSI or SUPI of the UE.

3.	The UDM returns the serving AMF address and possibly a VGMLC address and/or an LMF address in the VPLMN. The UDM also selects and returns Access Type to transmit the location information based on, e.g. Access Type, associated serving AMF and the UE state change (e.g. connectivity state changes) in the access. If the serving AMF for different Access Type is different, the UDM only returns the AMF address related to the selected Access Type and will not return the Access Type.

4.	The HGMLC forwards the location request to the VGMLC and includes the AMF address, the target UE identity (e.g. SUPI), any LMF address received in step 3 and Access Type.

5a.	[Conditional] If solution in clause 6.2 in TR 23.731 (i.e. LMF Based Location Solution) is performed, the VGMLC invokes the Nlmf_ProvideLocation Request service operation to forward the location request and Access Type to the LMF.

6a.	[Conditional] If step 5a is performed, the LMF invokes the Namf_Communication_N1N2Message Transfer service operation to send the Positioning Message and Access Type to the AMF.

5b.	[Conditional] If solution in clause 6.14 in TR 23.731 (i.e. Reuse R15 existing location service architecture for commercial use cases) is performed, the VGMLC invokes the Namf_Location_ProvideLocation_Request service operation to forward the Access Type to the AMF.

7.	[Conditional] If the Access Type is 3GPP access, the step is performed. The step can be the steps 2-6 in clause 4.13.5.4 in TS 23.502, steps 2-5 in clause 4.13.5.5 in TS 23.502, steps 2-4 in clause 4.13.5.6 in TS 23.502 or other new positioning procedures defined for 3GPP access.

8.	[Conditional] If the Access Type is N3GPP access, the step is performed. The step can be the steps 5-9 in clause 6.12.3.1 in TS 23.731 or other new positioning procedures defined for N3GPP access.

9a.	[Conditional] If step 6a is performed, the AMF invokes the Namf_Communication_N1N2Message Transfer service operation to forward the Positioning Message to the LMF.

10a.	[Conditional] If step 9a is performed, the LMF invokes the Nlmf_ProvideLocation Response service operation to forward the location information to the VGMLC.

9b.	[Conditional] If step 5b is performed, the VGMLC invokes the Namf_Location_ProvideLocation_Response service operation to forward the location information to the VGMLC.

11-12.	VGMLC returns the location estimate to the LCS client via HGMLC.

6.X.4	Impacts on existing entities and interfaces

# Impacts are as follows:

-	UDM: decide the Access Type to transmit the location information (e.g. positioning message) and provide the Access Type to the GMLC.

-	GMLC: send the Access Type received from UDM to AMF or LMF

-	LMF: send the Access Type received from GMLC to the AMF.

-	AMF: triggers the procedure to obtain UE location in the specific access based on the Access Type received from GMLC or LMF.

6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

*** End of changes ***