3GPP TSG-SA2 Meeting #127bis 	S2-186112

# Newport Beach, USA, May 28 - 01 Jun 2018												was  5451

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0502 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying UE Reachability Notification Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-01-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Text from 23.501 states: Reachability management is responsible for detecting whether the UE is reachable and providing UE location (i.e. access node) for the network to reach the UE. However, the procedure in 23.502 is NOT rechability reporting, but, either a. reporting on next "AMF Detectable" activity from UE: (a) Next NAS activity from UE (in CM-IDLE or CM-CONNECTED) or (b) S1AP message from the RAN that the UE is reachable (in CM-CONNECTED), or b. reporting of UE's state change from reachable to un-reachable as described in 4.15.4.2: - During Network Triggered Service Request procedure, if the UE does not respond to paging, when the AMF considers the UE as unreachable the AMF notifies the NF consumers that have subscribed for UE reachability event, that the UE is not reachable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarified the event reporting from reachability procedure. (a) Added the case for reporting on state change from reachable to unreachable. (b) Added the case of UE in CM-CONNECTED and AMF does not use the N2 notification procedure. 2. Event subscription by UDM does not have URRP-AMF as a paramter. So deleted that. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion that the Reachability procedure is only providing reachability (un-reachable to reachable transition) events. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5.1, 4.2.5.2, 4.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************ Next Change ***********************

### 4.2.5	Reachability procedures

#### 4.2.5.1	General

Elements of this procedure are used for UDM/NF initiated UE Reachability Notification requests, eg. for "SMS over NAS".

The procedure applies to UEs that are in RRC-Idle, RRC-Inactive and RRC-Connected states.

There are two procedures necessary for any service related entity that would need to be notified by the reachability of the UE:

-	UE Reachability Notification Request procedure; and

-	UE Activity Notification procedure.

#### 4.2.5.2	UE Reachability Notification Request procedure

The UE Reachability Notification Request procedure is illustrated in figure 4.2.5.2-1.

Figure 4.2.5.2-1: UE Reachability Notification Request Procedure

0.	During the Registration or subscription update procedure, the UDM informs the AMF of the identities (e.g. FQDNs) of the Network Functions that are authorized to request notifications on this UE's reachability via Nudm_UECM_Registration or Nudm_SubscriberData_Update service operation. By default, the UDM is always authorized.

1.	If a service-related entity requests the UDM to provide an indication regarding UE reachability, the UDM checks that that entity is authorized to perform this request on this subscriber.

If the entity is not authorized, the request may be rejected (e.g. if the requesting entity is recognized as being a valid entity, but not authorized for that subscriber) or silently discarded (e.g. if the requesting entity is not recognized). Appropriate O&M reports are generated.

2a.	The UDM stores the identity of the service-related entity and sets the URRP-AMF parameter to indicate that such request is received. If the value of URRP-AMF parameter has changed from "not set" to "set", the UDM initiates Namf_EventExposure_Subscribe_service operation  towards the AMF. The UDM may indicate if direct notification to NF shall be used.

2b.	Other NFs request notifications on this UE's reachability via Namf_EventExposure_Subscribe_service operation towards the AMF.

NOTE:	The UDM may trigger UE Reachability Notification Request procedure with two different AMFs for a UE which is connected to 5G Core Network over 3GPP access and non-3GPP access simultaneously. Also, the UDM may trigger UE Reachability Notification Request procedure with MME as described in TS 23.401 [13].

3	The AMF checks that the requesting entity is authorized to perform this request on this subscriber.

If the entity is not authorized, the request may be rejected (e.g. if the requesting entity is recognized as being a valid entity, but not authorized for that subscriber) or silently discarded (e.g. if the requesting entity is not recognized). Appropriate O&M reports are generated.

If the AMF has a MM Context for that user, the AMF sets URRP-AMF to indicate the need to report to the UDM or other NF with a UE Activity Notification (see clause 4.2.5.3).

4.	[Conditional] If the UE state in AMF is in CM-CONNECTED state and the Access Type is 3GPP access, the AMF initiates N2 Notification procedure (see clause 4.8.3) with reporting type set to Single RRC-Connected state notification.

#### 4.2.5.3	UE Activity Notification procedure

The UE Activity Notification procedure is illustrated in figure 4.2.5.3-1.

Figure 4.2.5.3-1: UE Activity Procedure

1a.	For a UE in CM-IDLE, the AMF receives (N1) NAS signalling implying UE reachability, e.g. a Registration Request or Service Request message from the UE or the UE's reachability state changes from reachable to unreachable, or

1b.	For a UE in CM-CONNECTED, if the AMF has initiated the N2 Notification procedure in Step 4 of clause 4.2.5.2 and the AMF receives a (N2) UE Notification (see clause 4.8.3) or a (N2) Path Switch Request (see clause 4.9.1.2) from the NG-RAN. Otherwise (i.e. UE is in CM-CONNECTED and AMF has not intiaited N2 Notification procedure), AMF performs Step 2.

2.	If the AMF has an MM context for the UE and the URRP-AMF is set to report once that the UE is reachable, the AMF initiates the Namf_EventExposure_Notify service operation (SUPI, UE-Reachable) message to the UDM or directly to the NF (if previously indicated to AMF in step 2 in clause 4.2.5.2). The AMF clear the corresponding URRP-AMF for the UE. If the UE was in CM-IDLE and the UE's reachability state changes from reachable to unreachable, the AMF initiates the Namf_EventExposure_Notify service operation (SUPI, UE-UnReachable).

2a.	When the UDM receives the Namf_EventExposure_Notify service operation (SUPI, UE-Reachable) message or Nudm_UECM_Registration service for a UE that has URRP-AMF set, it triggers appropriate notifications to the NFs (e.g. SMSF or SMS-GMSC) that have subscribed to the UDM for this notification. UDM clears the URRP-AMF for the UE.

2b.	When the AMF receives Namf_EventExposure_Subscribe_service operation directly from the NF, or the UDM indicates that the notification needs to be sent to the NF, the AMF initiates the Namf_EventExposure_Notify service operation (SUPI, UE-Reachable) message directly to the NF.