SA WG2 Meeting #127	S2-183580

Sanya, China, 16 - 20 April	(revision of S2-18xxxx)

Source:	LG Electronics

Title:	ATSSS Solution - Differentiation of 3GPP RATs in ATSSS Rule

Document for:	Approval

Agenda Item:	6.8 Study on Access Traffic Steering, Switch and Splitting support in the 5G system architecture

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution proposes structure of ATSSS rule to solve Key Issue 6: Differentiation of 3GPP RATs in ATSSS rule.

Discussion

In 5G system, two RATs (i.e. eLTE and NR) are supported in 3GPP access. Depending on operator deployment, each RAT may have different data rate and/or coverage. As a consequence, operators may have preferred RAT for specific services. For example, operators may prefer to use NR RAT to provide 4K video streaming because it supports high data rate. In order to provide such operator preference and to address Key Issue#6, we propose to differentiate 3GPP RATs in ATSSS Rule.

5.6 Key Issue #6: Key issue on differentiation of 3GPP RATs in ATSSS rule 5.6.1 Descriptions In EPS, when a UE has multi-access PDN connection, NBIFOM routing rule is used to select an access network to send traffic. In the routing rule access type is defined 3GPP access or non-3GPP access. This restricts the ability for the operator to provide policies that favour a specific 3GPP RAT over another one with respect to the non-3GPP preference. This key issue studies whether and how to provide relative priorities for 3GPP RATs with respect to the non-3GPP access in ATSSS rule.

Regarding Access Technology composing ATSSS Rule, we refer to the AccessTechnology leaf and the related description specified in TS 24.312 as below.

5.7.23AA ANDSF/ISRP/<X>/ForFlowBased/<X>/RoutingRule/<X> This interior node acts as a placeholder for one or more prioritized accesses. - Occurrence: OneOrMore - Format: node - Access Types: Get, Replace - Values: N/A This node and its descendants are the same as defined in ANDSF/Policy/<X>/PrioritizedAccess/<X> unless explicitly specified. The UE selects between 3GPP access technologies (i.e. E-UTRA, UTRA, GERAN) using the mechanisms implemented by lower layers (e.g. cell selection, cell reselection, handover, etc). If a 3GPP access technology corresponding to the value configured in the AccessTechnology leaf under this node is not selected by the UE then the UE shall ignore this node until the 3GPP access technology corresponding to the value configured in the AccessTechnology leaf becomes selected. 5.7.23A ANDSF/ISRP/<X>/ForFlowBased/<X>/RoutingRule/<X>/ AccessTechnology The AccessTechnology leaf indicates a prioritized access technology. - Occurrence: One - Format: int - Access Types: Get, Replace - Values: <Access technology> Possible values for the Access technology are specified in table 5.7.23A.1. Table 5.7.23A.1: Possible values for the AccessTechnology leaf Value; Description. 0; Reserved. 1; Any 3GPP access technology, e.g. E-UTRA, UTRA, GERAN. 2; Reserved. 3; WLAN. 4; E-UTRA. 5; UTRA. 6; GERAN. 7-255; Reserved.

Proposal

* * * * Start of 1st Change * * * *

6.X	Solution X: Structure of an ATSSS Rule

This solution applies to Key Issue #6: Key issue on differentiation of 3GPP RATs in ATSSS rule and also applicable to all other Key Issues.

An ATSSS Rule contains the following parameters:

-	ATSSS Rule Identifier

-	ATSSS Rule Priority

-	Packet Filter Set

-	Access Technology

Editor's note: Any other parameters are FFS.

Editor's note: It is FFS whether ATSSS Rule is part of URSP.

The ATSSS Rule Identifier uniquely identifies an ATSSS Rule in the UE. The ATSSS Rule identifier is allocated by the entity creating the ATSSS Rule, i.e. by the UE in UE mode and by the PCF in Network mode.

For the purpose of matching user traffic against ATSSS Rules, the Packet Filter Set is applied in the order of the ATSSS Rule Priority.

There are two types of Packet Filter Set, i.e. IP Packet Filter Set, and Ethernet Packet Filter Set, corresponding to those PDU Session Types. The Packet Filter Set may contain multiple packet filters. The Packet Filter Set is defined in TS 23.501 [5] clause 5.7.6.

The Access Technology indicates a prioritized access technology for routing of the traffic flows, and possible values for the Access Technology are described in Table 6.X-1. If a 3GPP access technology corresponding to the value configured for the Access Technology is not selected by the UE then the UE shall ignore this configuration until the 3GPP access technology corresponding to the value configured for the Access Technology becomes selected.

NOTE:	If a 3GPP access technology corresponding to the value configured for the Access Technology is E-UTRA or NR and the UE is using more than two access technologies at the same time (i.e. Dual Connectivity), the UE shall ignore this configuration.

Table 6.X-1: Possible values for the Access Technology

| Value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Any 3GPP access technology e.g. NR, E-UTRA |
| 2 | WLAN |
| 3 | NR |
| 4 | E-UTRA |
| 5-255 | Reserved |

* * * End of Changes * * * *