SA WG2 Meeting #127	S2- 184629

16 - 20 April 2018, Sanya, PR China	(e-mail revision 2 of S2-184418)

Source:	Ericsson

Title:	PCR 23.724: eDRX

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	Fs_CIOT_5G / Rel-16

Abstract of the contribution: Proposal for a solution to KI#4 for eDRX (Track 1)

# Discussion

In EPS the UE and the network may negotiate over non-access stratum signalling the use of extended idle mode DRX for reducing its power consumption, while being available for mobile terminating data and/or network originated procedures within a certain delay dependent on the DRX cycle value.

The proposal is to enable a eDRX equivalent feature from EPS in 5GS.

As the 5GC has a different architecture and some new functionality compared to EPC some adaptions are needed compared to eDRX in EPS. The following adoptions are needed:

-	eDRX sleep cycle negotiation
In EPC the UE requests extended idle mode DRX parameters during attach procedure and RAU/TAU procedure and the SGSN/MME may reject or accept the UE request for enabling extended idle mode DRX.
In case the SGSN/MME accepts the extended idle mode DRX, the SGSN/MME based on operator policies and, if available, the extended idle mode DRX cycle length value in the subscription data from the HSS, may also provide different values of the extended idle mode DRX parameters than what was requested by the UE. If the SGSN/MME accepts the use of extended idle mode DRX, the UE applies extended idle mode DRX based on the received extended idle mode DRX parameters. 
The same principles can be used on 5GC but the procedures used by the UE to request extended idle mode DRX parameters will be Registration and Registration Update.
In EPS the UE and the SGSN/MME handles the request in 5GS the UE handling shall be the same but in 5GC it will be the AMF instead of SGSN/MME.

-	Subscription information	
Extended idle mode DRX cycle length value needs to be added in the subscription data from the UDM using the same principles as in EPS.

-	Hyper SFN, Paging Hyperframe and Paging Time Window
Same handling as in EPC but in 5GC the AMF will assign the Paging Time Window. The AMF will include the Paging Time Window in the extended DRX parameters to the UE and in the paging eDRX information to the (R)AN.

# Proposal

It is proposed to add the solution below for power saving to the 5G CIoT TR 23.724.

***** First Change *****

# 6	Solutions

## 6.X	Solution #X: eDRX in 5GS

### 6.X.1	Introduction

The proposal is to enable an eDRX equivalent feature from EPS in 5GS. It addresses Key Issue 4, Power saving functions.

### 6.X.2	Functional Description

.

The proposal is to enable an eDRX equivalent feature from EPS in 5GS.

As the 5GC has a different architecture and some new functionality compared to EPC some adaptions are needed compared to eDRX in EPS. The following adoptions are needed:

-	eDRX sleep cycle negotiation:

In EPC the UE requests extended idle mode DRX parameters during attach procedure and RAU/TAU procedure and the SGSN/MME may reject or accept the UE request for enabling extended idle mode DRX.
In case the SGSN/MME accepts the extended idle mode DRX, the SGSN/MME based on operator policies and, if available, the extended idle mode DRX cycle length value in the subscription data from the HSS, may also provide different values of the extended idle mode DRX parameters than what was requested by the UE. If the SGSN/MME accepts the use of extended idle mode DRX, the UE applies extended idle mode DRX parameters either as received or modified by SGSN/MME.

The same principles can be used on 5GC but the procedures used by the UE to request extended idle mode DRX parameters will be Registration and Registration Update.
In EPS the UE and the SGSN/MME handles the request in 5GS the UE handling shall be the same but in 5GC it will be the AMF instead of SGSN/MME.

-	Subscription information	:

Extended idle mode DRX cycle length value needs to be added in the subscription data from the UDM using the same principles as in EPS.

-	Hyper SFN, Paging Hyperframe and Paging Time Window.

Same handling as in EPC but in 5GC the AMF will assign the Paging Time Window. The AMF will include the Paging Time Window in the extended DRX parameters to the UE and in the paging eDRX information to the (R)AN.

NOTE:	Whether the solution for Hyper SFN and Paging Hyperframe in NB-IoT and WB-E-UTRAN in the 5GS Rel-16 will look the same as extended idle mode DRX (eDRX) in E-UTRAN depends on RAN further work.

Editor's note:	Whether RRC inactive can be used in combination with long DRX cycles above those already supported in connected DRX for NB-IOT/WB-E-UTRAN with RAN buffering is FFS.

### 6.X.3	Support of EPC interworking

Editor's Note:	This clause describes if and how EPC-5GC interworking is supported by this solution.

TBD

### 6.X.4	Procedures

The existing procedures in 5GC will be reused to handle the eDRX signalling between AMF, (R)AN, UDM and UE.

### 6.X.5	Impacts on existing entities and interfaces

The UE shall send request for extended idle mode DRX parameters to the AMF. The UE will also receive the extended DRX parameters and utilize this information for extended DRX handling.

The AMF shall handle the request from the UE and reject, modify or accept the request. If accepted (as is or with modification) the AMF shall set the extended idle mode DRX parameters and supply relevant parameters to UE and (R)AN.

The UDM may include the extended idle mode DRX cycle length value in the subscription data to the AMF.

Editor's note:	It is FFS if there are any dependencies with the eNA study.

The (R)AN will receive the paging eDRX information from the AMF.

### 6.X.6	Evaluation

Editor's Note: This clause provides an evaluation of the solution.

TBD

***** End of Changes *****