3GPP TSG-SA2 Meeting #128BIS	S2-187690

# Sophia-Antipolis, France, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0334 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct conditions for using valid local emergency number list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Ltd., ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEW2 |  |  |  |  |  | Date: |  |  | 03/08/2018 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a UE receives emergency numbers/types via mobility management procedures as described in TS 24.301 and TS 24.008, it uses them while in the same country in accordance with stage 1: The serving network may download emergency call numbers to the UE in order to ensure that local emergency call numbers are known to the UE. The UE shall regard these emergency numbers as valid in that country only (as identified by the MCC) and shall discard them when a new country is entered. Likewise, stage 3, as described in TS 24.301 and TS 24.008, does not compare the PLMN id of the current registered PLMN against the PLMN from which a valid list was received, prior to using these local emergency call numbers. The requirement to only use the emergency numbers/types, when received from the last PLMN, is not aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align J.2 and L.2 with TS 24.301 and TS 24.008. Mention GERAN's mobility management procedures: TS 24.008 does not prevent mobility management procedures used over GERAN from downloading local emergency numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 2 and stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | J.2, L.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

# J.2	UE specific behaviour

Procedure for determining the list of emergency numbers:

-	In addition to the emergency numbers/types stored either on the USIM or on the user equipment specified in TS 22.101 [8] clause 10.1.1, the UE shall use the stored Local Emergency Numbers List received from the 3GPP network to detect that the number dialled is an emergency number.

-	If a UE has a SIM/USIM and has received a list of emergency numbers/types via mobility management procedures as described in TS 24.301 [33] and TS 24.008 [13] when connected to the PLMN using GERAN, UTRAN or E-UTRAN radio access network, the UE shall use this list to determine if the dialled digits correspond to an emergency call as long as the UE has determined that the list is valid (i.e. UE has determined that it is in the same country when the list of emergency numbers was received from PLMN).

-	The UE may also determine that the dialled digits/types correspond to an emergency call based on the list of emergency numbers retrieved by one of the following means:

NOTE:	How the UE uses two lists (a list of emergency numbers/types received via mobility management procedures and a list of emergency numbers retrieved via the WLAN) is a stage 3 decision.

a)	via Access Network Query Protocol (ANQP) procedures defined in IEEE Std 802.11 2012 [47] from the associated WLAN AP upon completing authentication with the trusted WLAN, with management frame protection enabled.

b)	via a previous query to a DNS only when that DNS is in the same country as the UE and is an internal DNS server in the 3GPP network whose address is acquired from the ePDG; or

c)	via IKEv2 procedures from ePDG.

-	The list of downloaded emergency numbers shall be deleted at country change.

For the specific case where the UE has selected to make an emergency call over WLAN access to EPC (hence the UE has detected that the target URI or dialled digits correspond to an emergency sessions), the UE shall use the following procedures:

-	The UE shall establish an emergency PDN connection over WLAN and shall perform an IMS emergency registration before sending an IMS emergency session request.

-	The UE shall include any available location information in the IMS emergency session request. This is further detailed in TS 23.228 [1].

-	For the media supported during IMS emergency sessions, media codec and format support is specified in TS 26.114 [34].

-	For the support of DRVCC on emergency call on WLAN, see TS 23.237 [32].

A UE shall not establish an emergency PDN connection over WLAN if the UE initiated a normal (i.e. non-emergency) session and is subsequently informed by the network that the ongoing session is accepted by the network even though it is an emergency session.

***** Next change *****

# L.2	UE specific behaviour

Procedure for determining the list of emergency numbers:

-	In addition to the emergency numbers/types stored either on the USIM or on the user equipment specified in TS 22.101 [8] clause 10.1.1, the UE shall use the stored Local Emergency Numbers List received from the 3GPP network to detect that the number dialled is an emergency number.

-	If a UE has a SIM/USIM and has received a list of emergency numbers/types via mobility management procedures as described in TS 24.301 [33] and TS 24.008 [13] when connected to the PLMN using GERAN, UTRAN or E-UTRAN or NG-RAN radio access network, the UE shall use this list to determine if the dialled digits correspond to an emergency call as long as the UE has determined that the list is valid (i.e. UE has determined that it is in the same country when the list of emergency numbers was received from PLMN).

-	The UE may also determine that the dialled digits/types correspond to an emergency call based on the list of emergency numbers retrieved via a previous query to a DNS only when that DNS is in the same country as the UE and is an internal DNS server in the 3GPP network whose address is acquired from the AMF;

NOTE:	How the UE uses two lists (a list of emergency numbers/types received via mobility management procedures and a list of emergency numbers retrieved via the untrusted non-3GPP access) is a stage 3 decision.

-	The list of downloaded emergency numbers shall be deleted at country change.

For the specific case where the UE has selected to make an emergency call over untrusted non-3GPP access to 5GC (hence the UE has detected that the target URI or dialled digits correspond to an emergency sessions), the UE shall use the following procedures:

-	The UE shall establish an emergency PDU session over untrusted non-3GPP and shall perform an IMS emergency registration before sending an IMS emergency session request.

-	The UE shall include any available location information in the IMS emergency session request. This is further detailed in TS 23.228 [1].

-	For the media supported during IMS emergency sessions, media codec and format support is specified in TS 26.114 [34].

A UE shall not establish an emergency PDU session over untrusted non-3GPP access if the UE initiated a normal (i.e. non-emergency) session and is subsequently informed by the network that the ongoing session is accepted by the network even though it is an emergency session.

***** No more changes *****