SA WG2 Meeting #128-bis	S2-187746

Sophia Antipolis, France, 20 - 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0120 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AMF requests for the PCF policy update of the URSP rules in the UE after the Allowed and/or Configured NSSAI update. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Wistron |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-08-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 23.501, clause 5.15.4.2, it describes that, once the UE received the Allowed and/or Configured NSSAI update, it should also receive the URSP update according TS 23.503. However, when referring to TS 23.503, clause 6.6.2.2, there is no mention how and when the PCF is triggered to update the URSP rule in the UE. Furthermore, clause 5.15.4.2 in TS 23.501states that, without the URSP rule, the UE is not allowed to indicate the target S-NSSAI in the PDU Session Establishment procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This is the 23.503 part of the CR correction to have the AMF to request the UE’s serving PCF in the HPLMN for the policy update of the URSP rule in the UE once the Network Slicing subscription change has been successfully updated in the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not able to initiate the PDU Session Establishment until the UE’s serving PCF in the HPLMN updates the URSP in the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The corresponding changes to TS 23.501 and TS 23.502 are in CR0554 and CR0632. |  |  |  |  |  |  |  |  |

#### ***** First Change *****

#### 6.6.2.2	Configuration and Provision of URSP

The UE may be provisioned with URSP rules by PCF of the HPLMN. When the UE is roaming, the PCF in the HPLMN may update the URSP rule in the UE. In addition, the UE may be also pre-configured with URSP rules (e.g. by the operator).

The PCF provisions the URSP rules in the UE may be during or after the UE’s successful registration, configuration update or Network Slicing subscription change.

Only the URSP rules provisioned by the PCF is used by the UE, if both URSP rules provisioned by the PCF and pre-configured URSP rules are present.

#### ***** End of Change *****