3GPP TSG-SA WG2 Meeting #128BIS	S2-187943

Sophia Antipolis, France, 20 - 24 August, 2018

Source:	ORANGE, HUAWEI

Title:	Updates of Solution #2

Document for:	Approval

Agenda Item:	6. 11

Work Item / Release:	FS_eNA

Abstract of the contribution: The proposal addresses updates of Solution #2:” Network Data Analytics Feedback”.

# Discussion

The modifications address the following topics:

| Topic | Reason | Proposal |
| --- | --- | --- |
| Editor’s notes about the convergence with network exposure framework (§6.2.1, §6.2.1.3) | The vocabulary (EventID, matching filters) has been harmonized with: - The NWDAF vocabulary stated in 23.502 §4.19 - The NEF 23.502 §4.15 vocabulary (which is similar, with some parameters more detailed) | - Removal of Editor’s notes Convergence of vocabulary - Explicit references to 23.502 §4.19 and §4.15 |
| Editor’s note about non-UE targets to be defined in conjunction with SA5 (§6.2.1.2, §6.2.1.3) | The corresponding level “NF ID” is now stated with a NOTE in the Table 6.2.1.3-1. The text of Editor’s note is in the §Impact | - Removal of Editor’s note - Addition of a NOTE on NF ID - Addition of text “Usage of non UE targets (NFs or groups of NFs) is optional. It needs to be defined in conjunction with SA WG5. Non UE targets (e.g. NFs) may typically be requested to help optimizing NF selection.” |
| Editor’s notes about parameters, KPIs leaving them for further study (§6.2.1.1, §6.2.1.3) | All Editor’s notes denoting a “further study in the TR” are changed into a NOTE describing the topic as to be addressed in the normative phase | - Removal of Editor’s notes (KPI, parameters, levels) - Addition of text “NOTE: The exact definition of … is left for the normative phase.” |
| §Impacts and §Evaluation | 3GPP default Editor’s note signalling the missing parts | - Addition of the §Impact and §Evaluation chapters |
| Various changes | Editor’s note on the possibility to provides arrays of KPIs in mode then one dimension (§6.2.2) Possibility to have periodical notification (§6.2.1.1) Possibility to express the time validity of analytics | - Removal of Editor’s note - Addition of text to be align with NEF - Addition of timestamp |

# Proposal

It is proposed to modify the following clause in TR 23.791.

**** Start of Change ****

## 6.2	Solution 2: Network Data Analytics Feedback

### 6.2.1	Description

This is a solution to Key Issue#1: Analytic Information Exposure to 5GS NF

The procedure for Network data analytics Subscribe/Unsubscribe and Network data analytics Request are defined in clause 6.1.1.1 and clause 6.1.1.2, respectively.

The solution extends the features of NWDAF as exposed in clause 4.19 of TS 23.502 by enabling statistics in the past and predictions in the future. It also integrates some input parameters (e.g. network area, UE groups) similar to those of the network exposure framework as defined in clause 4.15 of TS 23.502 or clause 5.3 of TS 29.518.

#### 6.2.1.1	Content of Analytics requests/subscribe

In addition to basic SBI parameters defined in TS 29.520 [10], the requests or the subscription to analytics should contain:

-	Event ID : identification of the desired analytics categories, possibly fit to use cases (connectivity, registration, mobility, session management, user plane, QoS management, roaming, etc)

-	Event Filter Information (as defined in TS 23.502 §4.15), this set of parameter types and values enables to select which type of analytics information are requested (e.g. subset of all available KPIs for the given Event ID value)

-	Event parameters for periodic notification, similar to those defined in TS 23.502 §4.15: event reporting granularity (time period), maximum number of reports, maximum duration of reporting

-	Optional preferences on analytics (e.g. computation constraints).

NOTE:	 the exact definition of preferences on analytics/predictions is left for the normative phase are provided and the parameters regarding to the preferences on analytics/predictions will be for further study.

-	Observation period: time interval [start..end] covered by the computation, either in the past or in the future, expressed as positive or negative offsets to the present. An interval in the past is a request for statistics. An interval in the future is a request for predictions. A default value means statistics in the recent past, with a time range defined by the NWDAF.

- 	Event reporting target (as defined in TS 23.502 §4.15):

-	Entities concerned by analytics (e.g. UEs, slice, NFs/NF sets).

-	Optional location filters e.g. network Area Of Interest that specifies a geographical area within 3GPP system, geographical area, area type (urban, rural).

-	Optional additional activity filters, which restrict the nature of provided KPIs (e.g. targeted users, type of users, type of service (voice, etc), IP filter, QFI).

#### 6.2.1.2	Content of Analytics responses/notifications

In addition to basic SBI parameters defined in TS 29.520 [10], the requests or the subscription to analytics should contain:

-	Identification of the request/subscription.

-	Target Object Values, which contains values associated with the requested analytics target object.

-	Related

KPIs (or arrays of KPIs) on the requested observation period,

NOTE: The exact definition of KPIs is left for the normative phase.

-	Optional additional information:

-	Timestamp of analytics generation, which allows consumers of the feedback to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp.

-

Activity warnings (e.g. overload, attack).

-	Probability assertion: level of certainty, degree of confidence in statistics/prediction.

#### 6.2.1.3	Event reporting target

The event reporting target is a set of value assertions which determine which entities are concerned for the production of relevant analytics, and a way to produce aggregated or more detailed results on these objects.

Each level of parameters may or may not be support wildcards. In case of wildcards, it shall be possible to specify either an empty wildcard meaning "all values", or a constrained wildcard by denoting a range of values.

The parameters may be organized as a set of additional value assertions (equality assertions or more improved) which acts as constraints for the NWDAF, for the production of analytics with a finer grain.

The use of wildcards enables the discovery of new values (e.g. the set of values of UE group ID).

Table 6.2.1.3-1

| Level | Level role | Example of parameters | Value assertion (examples) |
| --- | --- | --- | --- |
| 0 | slice ID | S-NSSAI | = « MBB » |
| 1 | NF ID | NF, NF set, NF region | ="all NFs" |
| 2 | user ID | SUPI/PEI/GPSI, group ID | = "All UE-groups", explicit list |
| 3 | Area/location | TAI, Cell ID, geographical area | ="Cell-3" |
| 4 | Application | DNN, AFid, | = « MMS » |
| 5 | Activity | Additional filters FFS | / |

NOTE: The exact definition of parameters is left for the normative phase.
NOTE: the need and usage of NF ID (level 1) is to be defined in conjunction with SA WG5

In complement, the explicit mode may be used to determine whether the provided KPIs are aggregated or explicitly listed. For at most one single level and one single parameter within a subscription, it should be possible to specify if it is explicitly listed (e.g. Level 2 "explicit list"). This permits to collect an array of KPIs on one single dimension, in order for the consumer NF to compare KPIs and facilitate its decisions.

When level aggregation is chosen, the cumulative or mean value of the measurement shall be provided, depending on the type of KPI. For instance, if the KPI "registration counter" is requested aggregated, only one aggregated counter is provided which totalizes counters on the various. Aggregation rules should be specified per KPI type.

When explicitly listed, all values of the scope parameter corresponding to the wildcard are requested. For instance, if the KPI "number of registrations" is requested as an explicit list at level UserGroup-ID, an array of counters is provided, one per each value of UE-groups (e.g. list of <UE-group-counter>=<number of registrations>).

### 6.2.2	Impacts on Existing Nodes and Functionality

The solution is compatible with the existing NWDAF interface as exposed in clause 4.19 of TS 23.502.

The solution extends the features of NWDAF as exposed in TS 23.502, by enabling both statistics in the past and predictions in the future.

It integrates some additional input parameters (e.g. network area, UE groups) similar to those of the network exposure framework as defined in clause 4.15 of 23.502 or clause 5.3 of 29.518.

Usage of non UE targets (NFs or groups of NFs) is optional. It needs to be defined in conjunction with SA WG5. Non UE targets (e.g. NFs) may typically be requested to help optimizing NF selection.

### 6.2.3	Solution Evaluation

The solution provides application- agnostic analytics.

It enables to perform comparisons of statistics in the past and predictions in the future related to various domains, leaving all decisions to the consumer NF.

**** End of Changes ****