SA WG2 Meeting #128BIS	S2-187956

20 - 24 August 2018, Sophia Antipolis, France	(revision of S2-18xxxx)

Source:	Huawei, HiSilicon

Title:	Update of solution#5

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC

Abstract of the contribution: This paper intends to update solution #5 for KI#2.

# Discussion

1. Update for the Xn based handover in Solution #5:

This contribution proposes to update the Xn based “enhanced handover” as depicted in Figure 1.

Figure 1: updated Xn based handover for “enhanced handover” procedure

The proposed changes to Xn-based HO are:

Change 1:

Change the message name in step 2a from “PDU session update” to “N2 message for handover”, because the source RAN node can not initiate a PDU session update request.

Change 2:

Split “Handover in RAN side” into “Handover preparation in RAN side” and “handover execution in RAN side”. Step 2 should happen between “Handover preparation in RAN side” and “handover execution in RAN side”.

Change 3:

Resolve the EN:

Editor’s note: it is FFS how to guarantee UE receiving all the packets sent via source tunnel before handover to target RAN node when UE works in single radio mode.

When the secondary tunnel is finished (after step-2b), the UPF begin to send duplicate the downlink data and send to both source and target RAN node simultaneously. An indication is sent to the source RAN to indicate the start of redundant transmission. The indication can be included in the GTP-U header of the first redundant packet to the source RAN or sent to the source RAN before the transmission of the first redundant packet.

The source RAN will trigger Uu handover once it receives the indication for the start of redundant transmission and finishes the transmission of the packets received before the indication.

2. Similar changes can be implemented for the N2 based handover.

# Proposal

It is proposed to update TR 23.725 as follows:

*******FIRST CHANGE*******

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 23.502: "Procedures for the 5G System".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6]	802.1CB-2017 - IEEE Standard for Local and metropolitan area networks--Frame Replication and Elimination for Reliability, https://ieeexplore.ieee.org/document/8091139/.

[7]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity".

[8]	ETSI GS MEC 003: "Mobile Edge Computing (MEC); Framework and Reference Architecture".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

*******SECOND CHANGE*******

## 6.5	Solution #5: Duplication of user plane tunnelling during HO

### 6.5.1	Functional Description

This solution addresses Key Issue#2 (supporting low latency and low jitter during HO). The basic idea is that the user plane tunnel will be established and used to transmit data as long as the DRB is established during handover procedure, which is called “enhanced handover”, so as to avoid the additional latency and jitter brought by data forwarding and/or data path switch on CN side.

The "enhanced handover" is defined per [S-NSSAI, DNN]. It can be dynamically provisioned to SMF from UDM or PCF during PDU session procedure, or statically preconfigured in SMF. Then SMF determines if apply this "enhanced handover" for each PDU session and store the association of the "enhanced handover" and PDU session id.

When handover is triggered by source RAN node, the SMF will duplicate the tunnel for the PDU session which is associated to "enhanced handover", and send the downlink data to both source and target RAN node until the handover is completed.

### 6.5.2	Procedures

#### 6.5.2.1	PDU session establishment or modification

Figure 6.5.2.1-1: PDU session establishment or modification for "enhanced handover" provisioning

In step3, based on the "enhanced handover" policy provided from PCF or static policy stored in SMF, the SMF determines if the PDU session should be applied for "enhanced handover" and stored the association between PDU session id and "enhanced handover" indication.

NOTE:	The SMF may add an indicator to the PDU Session create/update message sent from SMF to RAN to indicate if the PDU session data needs data forwarding when handover happens.

#### 6.5.2.2	N2 based Handover

Figure 6.5.2.2-1: N2 based handover for "enhanced handover"

The procedure reuse existing N2 based handover, with the following characteristics:

3.	During session modification, the SMF establishes the secondary tunnel for PDU sessions marked as "enhanced handover" which sharing the same IP address with the original tunnel within the same PDU session.

11.	When the secondary tunnel is finished (after step-8), the UPF begins to send the duplicate downlink data to both source and target RAN node simultaneously. An indication is sent to the source RAN via user plane to indicate the start of redundant transmission. The indication can be included in the GTP-U header of the first redundant packet to the source RAN or sent to the source RAN before the transmission of the first redundant packet.

12a. The source RAN will trigger Uu handover once it receives the indication for the start of redundant transmission and finishes the transmission of the packets received before the indication .

12b.	Before the target DRBs are established, the target RAN node caches the downlink data from UPF. As long as the target DRBs are established, the UL and DL data transmission will be performed.

13.	As long as the DRBs in target RAN node are established, UE will transmit the UL and DL data via target RAN node before the handover procedure is totally finished (UE sending handover complete). In dual radio based handover case, the UE will transmit the data via source and target RAN nodes simultaneously before the handover procedure is totally finished.

Editor's note:	how to realize this reordering is FFS.

15-16:	When receiving handover notify, the core network releases the tunnel in source RAN side.

#### 6.5.2.3	Xn based Handover

Figure 6.5.2.3-1: Xn based handover for "enhanced handover"

The procedure reuse existing Xn based handover, with the following new features.

1-2. 	After the step 5 in Handover Preparation procedure as defined in TS 38.300 [x] is finished, the source RAN node sends an N2 message for handover to the core network to establish the secondary tunnel in target side for the PDU session marked as “enhanced handover”. The N2 message for handover sent by the soure RAN node includes the PDU session IDs to be switched and the list of QoS Flows accepted by the Target RAN node.

3.	When the secondary tunnel is finished (after step 2c), the UPF begin to send duplicate the downlink data and send to both source and target RAN node simultaneously. An indication is sent to the source RAN to indicate the start of redundant transmission. The indication can be included in the GTP-U header of the first redundant packet to the source RAN or  sent to the source RAN before the transmission of the first redundant packet.

4.	The source RAN will trigger Uu handover once it receives the indication for the start of redundant transmission and finishes the transmission of the packets received before indication.

5.  Before the DRB is established, the target RAN node cache the downlink data from UPF.

6.	As long as the DRBs in target Node are established, the target RAN node, UE will transmit the UL and DL data via target RAN node before the handover procedure is totally finished (UE sending handover complete). In dual radio based handover case, the UE will transmit the data via source and target RAN nodes simultaneously before the handover procedure is totally finished.

7.	UE reorder the data from both source and target RAN node.

Editor's note:	how to realize this reordering is FFS.

### 8-13.	When receiving path switch request, the core network release the N3 tunnel to source RAN. The target RAN node triggers the release of resources with the source RAN node if it confirms success of the handover by receving the pasth switch request ACK.

### 6.5.3	Impact on existing entities and interfaces

Editor's note:	Impacts on existing nodes or functionality will be added

*******END OF CHANGE*******