SA WG2 Meeting #128bis	S2-188051

20 – 24 August 2018, Sophia Antipolis, France

Source:	Huawei, Hisilicon

Title:	Update to Solution #15 to add location information of I-SMF

Document for:	Discussion/Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: This contribution Updates Solution #15 to suggest the location information of I-SMF is transmitted to the A-SMF which can be used for policy.

- Discussion

Solution #15 mainly focus on how to interact with PCF. In the impact section, it describes “A-SMF may need to know I-SMF service area”. But in the procedure, it does not describe why this is needed and how to use this information? The service area of the I-SMF is the collection of the UPF Service Areas. It includes a large amount of the TA. It may be impossible and inefficient to send all I-SMF service area information to A-SMF. However, the AF influence policies are related to certain DNAIs, it is not necessary to send all AF influence policies of the whole PLMN to all SMFs.

From our view the PCF and A-SMF need be aware of the place the I-SMF serves, so that only the polices that are relevant to this I-SMF are sent to it, e.g. the region ID where the SMF serves or the administrative domain ID where the SMF serves, etc. Hence, it is proposed that these information is provided to A-SMF and PCF.

When A-SMF receives policies from PCF, it needs to determine which policies should be provided to I-SMF. The A-SMF may do this based on the information that indicates valid area of the policy, e.g. the region ID where the policy is to be applied, or the administrative domain ID where the policy used.

- Proposal

It is proposed to add the following text into TR 23.726.

/*************************** Start of the first change ************************/

6.15	Solution #15: Handling AF influence on traffic routing

6.15.1	Overview

This solution applies to key issue 5: Handling AF influence on traffic routing.

6.15.2	Network Architecture

The proposed solution is based on the network architecture in solution 6.1 of TR 23.726. The proposed solution aims to avoid changes to PCF and so the proposal does not require the PCF to be aware of any insertion or removal of an I-SMF(s) i.e. PCF forwards PCC rules corresponding to the AF request to the anchor SMF irrespective of any I-SMF insertion.

Figure 6.1.2-1: Non-Roaming system architecture in reference point representation

The A-SMF provides information that indicates the area that I-SMF serves to PCF, e.g. region ID or administrative domain ID of the area the SMF serves. PCF provides to A-SMF the PCC rules that apply to the I-SMF and A-SMF forwards them to I-SMF.

6.15.3	Procedures

6.15.3.1	AF policy processing at the A-SMF without PCF impact

Below changes are required at the A-SMF and to the interface between A-SMF and I-SMF i.e. on Nxx:

-	PCF forwards PCC rules corresponding to the AF request to the anchor SMF, irrespective of insertion and/or removal of intermediate SMF(s).

-	I-SMF is inserted e.g. as explained in other solutions in this TR. A-SMF learns insertion of I-SMF(s) and provides to PCF the information that indicates the area where the I-SMF serves e.g. region ID or administrative domain ID of the area that SMF serves.

-	PCF receives from A-SMF the information that indicates the area where the I-SMF serves e.g. region ID or administrative domain ID of the area that SMF serves, and sends PCC rules that apply to the I-SMF.

-	A-SMF forwards to I-SMF any required information, based on the received PCC rules, over its Nxx interface. A-SMF forwards to I-SMF only the PCC rules that apply to the I-SMF.

Figure 6.15.3.1-1: AF policy processing at the A-SMF without PCF impact

1-5.	Same as in TS 23.502 [3], Figure 4.3.6.2-1: Processing AF requests to influence traffic routing.

6.	Later, an I-SMF is inserted. The information, that indicates the area where the I-SMF serves e.g. region ID or administrative domain ID of the area that SMF serves, is provided to A-SMF in this step.

7.	A-SMF invokes Npcf_SMFPolicyControl_Update request to provide the information that indicates the area where the I-SMF serves to PCF.

8.	PCF may update PCC rules towards A-SMF.

9.	A-SMF forwards to I-SMF any policy information where the policy would have to be enforced at the I-SMF, considering any PCC rules received previously from the PCF.

10.	I-SMF acknowledges.

11.	Possible I-UPF selection or insertion of UL-CL, user plane reconfiguration, etc., based on received policies in the above step.

******************************* 2nd Change *********************************

6.15.4	Impact of the solution to existing entities

Editor's note:	This clause describes impacts to existing entities and interfaces.

Nxx interface between A-SMF and I-SMF needs to be enhanced.

A-SMF needs to know the information that the area that the I-SMF serves, and sends to I-SMF the policies that apply to I-SMF.

A-SMF provides information that indicates the area that I-SMF and A-SMF serve to PCF.

PCF determines Policies based on the area that SMF serves.

******************************* End of Changes *********************************