3GPP TSG-SA WG2 Meeting #128bis	S2-188200

20 - 24, August, 2018, Sophia Antipolis, France

Source:	CATT

Title:	Supervision of IoT devices for risk precaution

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA / Rel-16

Abstract of the contribution: This paper would like to propose a solution on Supervision of IoT devices for risk precaution by using NWDAF.

1.	Discussion

This document proposes to a new solution to address part of Key Issue 8: performance improvement and supervision of mIoT terminals in TR 23.791.

* * * Start of Changes * * *

6.x	Solution for Key Issue #8: <Supervision of IoT devices for risk precaution>

6.x.1	Description

Editor's note:	Describe the solutions. Sub-clause(s) may be added to capture details, procedural flow, etc.

6.x.1.1	General

In the key issue 8, it is mentioned that the IoT devices may be misused or hijacked, and hence brings security issues to the 3GPP system or application layer services. This solution proposes that, first, the NWDAF analysis is relied to detect the misbehaviours of IoT UE(s) and evaluate the risk brought by the misbehaviour UE(s). Once the NWDAF deems that any risk brought by the misused or hijacked UEs is serious enough to notify or alert the subscribers, the NWDAF notifies the related subscribers (could be network functions or service providers) about the suspicious events or risk, so that the 3GPP system or service provider can adopt an appropriate precaution(s) for the risk.

When the operator or IoT service provider asks the NWDAF for supervision of a specific or a group of IoT devices, the NWDAF performs data analysis on such devices based on the collected information from the application server and the 3GPP network. In order to collect some runtime data like UE current location, data rate etc., the NWDAF may request the 3GPP network to start monitoring the UE states, i.e. the NWDAF sends monitoring request to the NFs to collect the required network data.

After the NWDAF completes the data analysis and detects some UEs with serious misbehaviours, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF determines whether to notify or alert the related 5GC NFs or application layer. The NWDAF may send the PCF a notification about the risk or suspicious events, which triggers the PCF to update the AM/SM policies. The NWDAF can also send the notification directly to the 5GC network function, e.g. AMF or SMF, so that the AMF or SMF can, based on operator policies, adopt pre-defined anti-risk mechanisms. The following table gives examples of policies for risk solving.

Figure 6.x.1.1-1 examples of policies and actions for risk solving

| Risk description | AM/SM policy | Actions of NFs |
| --- | --- | --- |
| Unexpected UE location | Add the area of current UE location into mobility restriction | AMF Applies mobility restriction |
| Unexpected elephant flows | Decrease the MBR for the related QoS flow | SMF updates the QoS rule |
| Unexpected wakeup | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Release the PDU session and Apply SM back-off timer | SMF releases the PDU session and applies SM back-off timer |
| Wrong destination address | Update the packet filter of the related QoS flow to block the wrong SDF | SMF updates the packet filter of the related QoS flow and configures the UPF |

Based on the subscription of the application server, the NWDAF sends the notification/alert to the application server, which may trigger application layer to activate some risk precaution methods.

6.x.1.2	Procedures

The procedure of using NWDAF to identify the misbehaviour IoT devices and trigger risk precaution for such IoT devices is shown in the Figure below.

Figure 6.x.1.2-1 procedure of using NWDAF to supervise IoT devices and enable risk precaution

1.	The application layer SCS/AS or the 3GPP system may subscribe the risk analytic report from the NWDAF, the subscription message can be UE specific or UE group specific.

2.	The NWDAF starts the data analysis for the UEs indicated in the subscription message.

3.	In order to improve the accuracy of the analytic result, the NWDAF may request some runtime data of the UE(s) by sending monitoring request to the related NFs.

4.	The monitoring request may be sent to the NFs via NEF.

5.	The 5GC NFs (e.g. AMF, SMF) send event reports to the NWDAF based on the report requirements received in the monitoring request.

6. 	The NWDAF analyses the collected data for the IoT devices, and may detect some suspicious events which may be a risk, e.g., unexpected UE location, abnormal traffic pattern, wrong destination address. Based on the risk type and operator policies, The NWDAF determines whether to send a notification or alert to the application layer and the 3GPP network.

7.	The notification or alert sent to the application layer indicates the risk ID, which is used by the SCS/AS to choose appropriate security precaution methods, e.g. isolating the misbehaviour UE, locking the account etc.

8.	The notification or alert may be sent to the PCF, so that the PCF can promptly adjust the related AM/SM policies (e.g. as described in the Table 1) to prevent the deterioration of the security issue.

9.	The updated AM/SM policies are provisioned to the AMF/SMF via PCF service Npcf_AM/SMPolicycontrol_Update Notify.

10.	The AMF/SMF enforce the updated policies.

11. The notification or alert containing the risk ID may be sent to the related 5GC NFs directly, e.g. AMF or SMF.

12.	Once the AMF or SMF receives the risk ID, based on the operator policies, it adopts appropriate actions (e.g. as described in the Table 1) to relieve the risk.

6.x.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.x.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * Start of Changes * * *