SA WG2 Meeting #128-bis	S2-188979

Sophia Antipolis, France, 20 - 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0570 | rev | 2 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network controlled NSSAI for SR-related Access Stratum connection establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NEC, Telecom Italia, Huawei, InterDigital, AT&T, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-07-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | as discussed in tdoc S2-187889, we need to conclude on the topic of what NSSAI include in the RRC connection establishment when this is triggered by a SR (whithout this, the Specification is incomplete). This CR proposes a way forward in alignment with that paper |  |  |  |  |  |  |  |  |
| Summary of change: |  | a PLMN can instruct the UE on which behaviour to adopt in a Registration Accept Message by including Access Stratum Connection Establishment NSSAI Inclusion Mode information that instructs the UE to: 1.- Include Access stratum Connection Establishment a NSSAI = allowed NSSAI for the UE, or 2. Include in Access Stratum Connection Establishment only the S-NSSAIs of the Network Slices that trigger the RRC Connection Establishment, or 3. Include no NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behaviour |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 5.15.x	Inclusion of NSSAI in Access Stratum connection establishment driven by a Service Request

The NSSAI to be included in the access stratum connection establishment driven by a Service Request is defined by the operator based on the initial access policies it implements in the (R)AN. During the Registration procedure, the AMF provides the UE in the Registration Accept message, an Access Stratum Connection Establishment NSSAI Inclusion Mode parameter indicating whether the UE shall include in the Access Stratum Connection Establishment (e.g. a RRC connection Establishment defined in TS 38.331) caused by a Service Request:

a)	NSSAI set to the Allowed NSSAI for the UE; or

### b)	NSSAI including the S-NSSAI(s) of the Network Slice(s) that trigger the Connection Establishment; or

### c)	No NSSAI.

NOTE: 	The indication is based on the policies implemented in the (R)AN nodes in the Registration area: if at least some (R)AN node in the Registration area is configured with policies that require knowledge of all the NSSAIs in the Allowed NSSAI, Mode a) applies, otherwise Mode b) applies unless no Initial access policy is deployed in the RA, in which case Mode c) applies. As another example: if the operator requires knowledge of the Allowed NSSAI to implement the AMF overload control defined in clause 5.19.5.2, then Mode a) applies.

**** END OF CHANGES ****