SA WG2 Meeting #128bis	S2-187999

20 – 24 August, Sophia Antipolis, France

Source:	Ericsson

Title:	Comparing the Service Framework Solutions for KI#3

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution identifies the specific difference between the proposed solutions and get the basis for comparing solutions and next steps.

# 1 Proposal

This pCR aims to pinpoints the specific differences in the proposed solutions for key issue #3 on improvements to service framework related aspects.

The following changes to TR 23.742 are proposed

* * * Start of Change * * *

## 7.X	Comparison of solutions for improvements of the service framework

### 7.X.1	Introduction

This clause provides a comparison of the proposed solutions for key issue #3 on improvements to service framework related aspects.

The respective solutions evaluated in this clause are:

- Solution 2: Amendments to NF Service Interaction model (see clause 6.2)

- Solution 3: Distributed Service Framework (see clause 6.3)

- Solution 4: Distributed 3GPP Aware Service Framework (see clause 6.4)

- Solution 5: Flexible Service Framework Deployment (see clause 6.5)

- Solution 6: Services Framework enhanced with a Service Agent (see clause 6.6)

### 7.X.2	Characteristics

To compare the different proposals, the following, categorized characteristics are used to analyse the solution proposals:

- Functional architecture

  - Owner of the service registry
Whether the NRF is kept or not, if the NRF is part of the proposed service framework, etc.

  - Location of common support functions
Whether common support functions such as discovery, instance selection, load balancing, authorization, policy enforcement, etc. are logically considered as components of the service framework or the services

  - Reduction of service complexity
Level of simplification of services

  - Supported communication patterns
Whether direct, in-direct or both inter-service communication is supported by the solution

  - Supported use cases
Whether scenarios such as roaming, network slicing, etc. are supported/considered by the solution

  - Additional components
Whether additional entities such as agents, adaptation layers, etc are part of the solution and their respective deployment options

- Procedure/message flow

  - Registration and discovery procedure
Whether procedures for service registration and/or discovery are exposed by the service framework or hidden, whether they are mandatory, optional, or not supported

  - Message routing
How request messages are routed towards the service producer instance and which parameters are used for the selection/routing decisions

- Compatibility

  - Interworking between service frameworks
Whether the proposed service framework supports interworking with other service framework instances

  - Rel-15 backward compatibility
Support for legacy consumer versions, i.e. Rel-15 NFservices, and API’s, i.e. Rel-15 NRF API’s

- Standardization effort

  - Expected changes for Rel-15 SBA
Items that need to be changed in or added to existing SBA specifications

  - Future extensions
Whether a concept for future enhancements is proposed, i.e. additional parameters etc.

- Items for clarification
Identified unclarities of the solution proposal which need further explanation.

### 7.X.3	Comparison result

Table 7.X.3-1 provides a summary on the key characteristics per proposed solution for improvements of the service framework.

NOTE: The abbreviation “SF” used in the table refers to the term “Service Framework”.

Table 7.X.3-1: Comparison of solution proposals for improvements of the service framework

| Category | Criteria | Solution 2 | Solution 3 | Solution 4 | Solution 5 | Solution 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Functional architecture | Owner of the service registry | Service framework; SF specific implementation | Service framework; SF specific implementation | Deployment option, either SF or standalone Rel-15 NRF | Service framework, with Rel-15 NRF as component | Standalone Rel-15 NRF |
|  | Location of common support functions | Service framework | Framework agents and Service framework | Service framework; or NRF plus services like in Rel-15 | Service framework; or services like in Rel-15 | Mainly in services (as per Rel-15), plus new alternative of new “service agent” (load balancing and instance selection) |
|  | Reduction of service complexity | Selection by services is not required; common support functions as part of the SF | Selection by services is not required. But selection is required in new “Framework agents” (to be standardized?); common support functions as part of the SF | Two alternatives are supported: a) similar to Rel-15; b) or common support functions as part of the SF, while the services do not implement Discovery and Selection | Two alternatives are supported: a) similar to Rel-15; b) or common support functions as part of the SF, while the services do not implement Selection | Selection by services is not required, Selection is externalized to new service agent |
|  | Supported communication patterns | In-direct, via service framework | In-direct, via framework agents | In-direct, via service framework, and direct, depends on SF deployment and configuration | In-direct, via service framework, and direct, depends on SF deployment and configuration | In-direct, via service agents, and direct, depends on deployment of service agents |
|  | Supported use cases | Roaming, via dedicated Roaming-SAPo; Slicing, SF functions per NW slice or shared | Not specified | Not specified | Roaming, via Rel-15 SBI and SEPP | Roaming, via Rel-15 SBI and SEPP |
|  | Additional components | Service Access Point (SAPo); Two deployment options, SAPo either as part of the SF or implemented with the services | Framework agents; Two deployment options, either standalone or implemented with the services | Service adaptation layer, new component of the SF, API to be standardized, function not to be standardized; new common support functions of the SF, not to be standardized | In case of direct communication deployment: None In case of indirect communication deployment: new common support functions of the SF, not to be standardized | Service agent, standalone component as GW between services and FW |
| Procedure/message flow | Registration procedure | Mandatory; Service producers register via SAPo API | Mandatory; Service producers register via framework agent API | Mandatory; Service producers register via Rel-15 SBI | Mandatory; Service producers register via Rel-15 SBI | Mandatory; Service producers register via service agents with Rel-15 SBI |
|  | Discovery procedure | No discovery procedure supported for service consumers; Discovery as SF internal function | No discovery procedure supported for service consumers; Discovery procedure between framework agents and the SF | Optional; Rel-15 SBI discovery procedure supported, but can be omitted, based on service consumer configuration; Discovery as SF internal function | Mandatory; Service consumers send discovery request via Rel-15 SBI | Mandatory; Service consumers send discovery request via service agents with Rel-15 SBI |
|  | Message routing | Instance selection and message routing as SF internal function, based on producer NF/NFService type | Instance selection and message routing split between framework agent and SF, based on user identity and producer NF/NFService type | Two different options. a) Instance selection and message routing as SF internal function, based on producer NF/NFService Profile parameters; b) Instance selection in Rel-15 service consumers supported | Two different options. a) Instance selection and message routing as SF internal function, based on producer NF/NFService Profile parameters; b) Instance selection in Rel-15 service consumers supported | Instance selection in service consumers, load balancing in producer service agent |
| Compatibility | Interworking between service frameworks | Via dedicated (Roaming-)SAPo, SEPP needs to be adapted to SAPo API; Not compatible with Rel-15 SBI | Not specified; Not compatible with Rel-15 SBI due to new interfaces | Not specified; Assumed to be compatible with Rel-15 SBI | Compatible with Rel-15 SBI | Compatible with Rel-15 SBI |
|  | Backward compatibility | Compatibility of new SAPo API replacing NRF API FFS, not backward compatible if different from Rel-15; Services need to be designed to be interoperable with the new SF, e.g. need to be adapted to skip discovery; Not backward compatible with Rel-15 SBI service implementations | New interfaces between services and framework agents and between service agents and SF; Services need to be designed to be interoperable with the new framework agents; Not backward compatible with Rel-15 SBI service implementations | Alternatives: a) Backward compatible with Rel-15 SBI; b) Services need to be designed to be interoperable with the new SF discovery procedure, e.g. need to be configurable to skip discovery | Alternatives: a) Backward compatible with Rel-15 SBI | Alternatives: a) Backward compatible with Rel-15 SBI |
| Standardization effort | Expected changes for Rel-15 SBA | New functional component of SAPo; New SAPo API | New functional component of framework agent; New API for framework agent; New API for SF | Two options. a) No effort with no enhancements b) Enhanced information/data models (e.g. NF/NFService profiles) to support enhanced discovery and message routing functions | Two options: a) No effort with no enhancements b) Enhanced information/data models (e.g. NF/NFService profiles) to support enhanced discovery and message routing functions | New functional component of service agent |
|  | Future extensions | FFS | FFS | FFS | FFS | FFS |
| Items for clarification | N/A | Roaming with rel 15 not described. It is unclear how instance selection will be done in case a network wide SDL is not used. | No description for roaming or network slicing scenarios; Not clear how SF interworking can be achieved | No description for roaming or network slicing scenarios; Does the SF adaptation layer need to be aware of and expose all (NF) service API's? In implicit mode, it is unclear how instance selection will be done in case a network wide data layer is not used. | No description for network slicing scenarios | No description for network slicing scenarios |

End of changes (all new text)