SA WG2 Meeting #128bis	S2-188509

August 20 – 24, 2018, Sophia Antipolis, France	(Revision of S2-188422)

Source:	China Mobile

Title:	Further clarification on Solution 8

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA / Rel-16

Abstract of the contribution: This paper would like to propose further clarification towards the solution for performance improvement and supervision of mIoT terminals.

1.	Discussion

This document aims to propose further clarification towards the solution for performance improvement and supervision of mIoT terminals.

2 Proposal

It is proposed to make the following changes to the TR 23.791.

* * * Start of Changes * * *

6.8.1.1	Information for the support of Performance improvement and supervision of mIoT terminals

The UE behavioural information from 5GC NFs for performance improvement and supervision of mIoT terminals is defined in Table 6.8.1.1-1.

Table 6.8.1.1-1: UE behavioural information collected from 5GC NF(s)

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| UE ID | M | AMF/SMF | Could be e.g. SUPI, which is used by NWDAF to correlate the UE behavioural information from different 5GC NFs. |
| Location info |  |  |  |
| >Timestamp | O | AMF | The timing for the UE |
| >Location | O | AMF | The location info for the UE e.g. Cell ID or TA ID |
| Communication Pattern Info |  |  |  |
| >Communication start time | O | SMF | Start time when the UE is available for communication |
| >Communication end time | O | SMF | End time when the UE is unavailable for communication |
| Network Configuration Info |  |  |  |
| >UL or DL Packet Latency | O | SMF | Indicating the delay for uplink or downlink packets transfers for the UE |

6.8.1.2	Procedure for expected UE behavioural information provisioning

Based on large amount of UE behavioural information from 5GC NF(s) corresponding to UEs in a SUPI list, NWDAF could clusters the UEs into one or more UE groups, where each UE group has an expected UE behavioural information, as defined in Table 6.8.1.2-1.

Table 6.8.1.2-1: Expected UE behavioural information for a UE group provided by the NWDAF

| Information | Presence | Description |
| --- | --- | --- |
| Stationary indication | O | Identifies whether the UE is stationary or mobile, e.g. only on demand. [TS 23.682[5], clause 5.10.1] |
| UE Moving Trajectory | O | Identifies the UE's expected geographical movement [TS 23.502[3], clause 4.15.6] Example: A planned path of movement |
| Periodic communication indicator | O | Identifies whether the UE communicates periodically or not, e.g. only on demand. [TS 23.682[5], clause 5.10.1] |
| Communication duration time | O | Duration interval time of periodic communication [may be used together with 1)] [TS 23.682[5], clause 5.10.1] Example: 5 minutes |
| Periodic time | O | Interval Time of periodic communication [may be used together with 1)] [TS 23.682[5], clause 5.10.1] Example: every hour |
| Scheduled communication time | O | Time zone and Day of the week when the UE is available for communication [TS 23.682[5], clause 5.10.1] Example: Time: 13:00-20:00, Day: Monday |
| Maximum Latency | O | Indicating maximum delay acceptable for downlink data transfers [TS 23.682[5], clause 4.5.21] |
| Maximum Response Time | O | Indicating the time for which the UE stays reachable to allow the AF to reliably deliver the required downlink data [TS 23.682[5], clause 4.5.21] |
| Suggested Number of Downlink Packets | O | Indicating the number of packets that the UPF shall buffer in case the UE is not reachable [TS 23.682[5], clause 4.5.21] |

Figure 6.8.1.2-1: Procedure for expected UE behavioural information provisioning

1. The UDM may subscribe expected UE behavioural information to the NWDAF by invoking Nnwdaf_EventsSubscription_Subscribe service operation. The input for the service operation should be,

-	Event ID

-	Event Filter Information: SUPI List

-	Target (list of SUPI, any UE)

2. Based on analytical result for large amount of UE behavioural information from 5GC NF(s) as defined in Table 6.8.1.1-1, NWDAF clusters the UE behavioural information into one or more UE groups in which each UE group has an expected UE behavioural information.

3. The NWDAF notifies the expected UE behavioural information as defined in Table 6.8.1.2-1 to the UDM by invoking the Nnwdaf_EventsSubscription_Notify service operation. The UDM stores the expected UE behavioural information as part of the subscription data for the corresponding UE(s).

Editor's note:  It is FFS how to resolve conflicts between behaviour data received from NWDAF with behaviour data being subscribed or received from the AF.

6.8.1.3	Procedure for mIoT terminals misused or hijacked Recognition

Figure 6.8.1.3-1: Procedure for Massive UE are misused or hijacked Recognition

0. As specified in 6.8.1.2-1, the NWDAF provides the expected UE behavioural information as defined in Table 6.8.1.2-1 for each UE to the UDM. The UDM stores the expected UE behavioural information as part of the subscription data for the corresponding UE. This contains also NWDAF subscription onto NF(s) about the Event ID = Deviation from UE behaviour.

1. UDM/UDR notifies the updated subscriber data i.e. the expected UE behavioural information to the corresponding 5GC NFs (e.g., AMF, SMF)

2. During a UE communicates with the 5GC, NF e.g. AMF, the NF determines whether the UE’s behavioural information matches its expected UE behavioural information, if not, the UE is regarded as an exception UE, then the NF should notify the NWDAF per the subscription received in step 1;

Editor's note:   It is FFS how the NF (e.g. AMF) determines whether the UE’s behavioural information matches its expected UE behavioural information.

3. The NWDAF may perform further data analysis on certain UE exception UE behavioural information and find that does not match the expected UE behavioural information, then abnormal behaviour could be expected e.g. be UE misused or hijacked.

4.	Based on analytical result, the NWDAF notifies exception UE ID list, corresponding Exception ID, corresponding Exception level to the AF. Based on these information, the AF may adjust e.g. recommended TCP Window Size, recommended Service Start and End time(e.g. for background data transfer), etc. For the untrusted AFs, the message can be send via the NEF.

5.1. Upon the subscription of notification, the NWDAF notifies exception UE ID list, corresponding Exception ID and Exception level to the relevant UDM/UDR. Taking these received information into account, the UDM/UDR may further update the UE subscription information, e.g., Supported SSC mode, default SSC mode, Subscribed S-NSSAI and periodic registration update timer. UE subscription change is sent to the registered NF (Serving AMF, serving SMF).

5.2. Upon the subscription of notification, the NWDAF notifies exception UE ID list, corresponding Exception ID and Exception level to the relevant PCF(s). Taking these received information into account, the PCF(s) may create or update the UE policy accordingly.

* * * End of Changes * * *