3GPP TSG-SA2 Meeting #128Bis		S2-188840

# Sophia Antipolis, France, August 20th - 24th  2018 		               (revision of S2-188713)

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0724 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification on terminology in NF service framework procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS-Ph1 |  |  |  |  |  | Date: |  |  | 2018-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is an ambiguity in the terminolorgy used in TS 23.502, when describing the procedures of the Network function Service Framework, for instance, in the NFservice registration procedure with the NRF in section 4.17.1. The ambiguity resides in the usage of the term “NF Service consumer” that is misleading for the following reasons: - Clause 4.17, like other clauses of section 4, is to specify end-to-end system procedures, so the call flows should not show the role (consumer/producer) of the NFs involved in a specific service. - In all the procedures of section 4.17, the described services are exposed by the NRF regardless wether the registering NF is consumer and/or producer of services. (Example: The registration service is exposed by the NRF to all NFs regardless wether the NF is consumer or producer) - There is not a clear definition of what is “NF service consumer” in TS 23501 and TS 23502, which makes the use of this term confusing. In addition, in the description of step 1 of NF registration service procedure in section 4.17.1, it is mentionned that the NF profile includes the following based on the description in TS 23.501: “According to clause 6.2.6 of TS 23.501 [2], the NF profile of NF service consumer includes NF type, FQDN or IP address of NF, Names of supported services, Endpoint information of instance(s) of each supported service and other service parameter” However, the following parameters are optional in stage 3 for the NF profile and especially not applicable if the registering NF is not a service producer (e.g. AF, UPF): “Names of supported services, Endpoint information of instance(s) of each supported service and other service parameter” |  |  |  |  |  |  |  |  |
| t |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - use "NF" in the callflow procedures instead of “NF service consumer” - use "NF" in the description text of procedures instead of “NF service consumer” - indicate “if applicable” before the optional parameters of the NF profile in step 1 of NF registration service procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | misunderstanding of the NF service framework procedures, which may affect implementation of the services |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17, 4.17.1, 4.17.2, 4.17.3, 4.17.4, 4.17.5, 4.17.7, 4.17.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 4.17	Network Function Service Framework Procedure

### 4.17.1	NF service Registration

Figure 4.17.1-1: NF Service Registration procedure

1.	NF/NF Service e.g. AMF instance sends Nnrf_NFManagement_NFRegister Request message (the NF profile of NF service consumer) to NRF to inform the NRF of its NF profile when the NF/NF Service consumer becomes operative for the first time. According to clause 6.2.6 of TS 23.501 [2], the NF profile of NF includes NF type, FQDN or IP address of NF, and if applicable, Names of supported services, Endpoint information of instance(s) of each supported service and other service parameter.

In case of UDR, the request message may include Range(s) of SUPIs and/or the Data Set Identifier(s) served by the UDR instance.

NOTE 1:	The profile of NF/NF Service that interacts with NRF are configured by OAM system.

2.	The NRF stores the profile of NF/NF Service and marks the NF/NF Service available.

NOTE 2:	Whether the profile sent by NF/NF Service to NRF needs to be integrity protected by the NF/NF Service and verified by the NRF is to be decided by SA3.

3.	The NRF acknowledges NF Registration is accepted via Nnrf_NFManagement_NFRegister response.

### 4.17.2	NF service update

Figure 4.17.2-1: NF Service Update procedure

1.	NF/NF Service e.g. AMF instance sends Nnrf_NFManagement_NFUpdate Request message (the updated NF profile of NF/NF Service) to NRF to inform the NRF of its updated NF profile (e.g. with updated capacity) when e.g. triggered after a scaling operation.

NOTE:	The updated NF profile of NF instance are configured by OAM system.

2.	The NRF updates the profile of NF/NF Service

3.	The NRF acknowledge NF Update is accepted via Nnrf_NFManagement_NFUpdate response.

### 4.17.3	NF service deregistration

Figure 4.17.3-1: NF Service Deregistration procedure

1.	NF/NF Service e.g. AMF instance sends Nnrf_NFManagement_NFDeregister Request message to NRF to inform the NRF of its unavailability when e.g. it's about to gracefully shut down or disconnect from the network.

2.	The NRF marks the NF/NF Service unavailable. NRF may remove the profile of NF/NF Service according to NF management policy.

3.	The NRF acknowledge NF Deregistration is accepted via Nnrf_NFManagement_NFDeregister response.

### 4.17.4	NF/NF service discovery in the same PLMN

Figure 4.17.4-1: NF/NF service discovery in the same PLMN

1.	The NF/NF Service intends to discover services available in the network based on service name and target NF type. The NF invokes Nnrf_NFDiscovery_Request (Expected NF service Name, NF Type of the expected NF instance, NF type of the NF consumer) from an appropriate configured NRF in the same PLMN. The parameter may include optionally SUPI, Data Set Identifier(s), External Group ID (for UDM, UDR discovery), Routing ID part of SUCI for UDM discovery and AUSF discovery, S-NSSAI, NSI ID if available, and other service related parameters. In addition, for AMF discovery, the parameters may include AMF Region ID, AMF Set ID, TAI.

NOTE 1:	The use of NSI ID within a PLMN depends on the network deployment.

NOTE 2:	The need for other service related parameters depends on the NF type of the expected NF instance(s) and refer to the clause 6.3 " Principles for Network function and Network Function Service discovery and selection" in TS 23.501 [2]. It is up to NF implementation whether one or multiple NF service instances are registered in the NRF.

2.	The NRF authorizes the Nnrf_NFDiscovery_Request. Based on the profile of the expected NF/NF service and the type of the NF/NF Service, the NRF determines whether the NF/NF Service is allowed to discover the expected NF instance(s). If the expected NF instance(s) or NF service instance(s) are deployed in a certain network slice, NRF authorizes the discovery request according to the discovery configuration of the Network Slice, e.g. the expected NF instance(s) are only discoverable by the NF in the same network slice.

3.	If allowed, the NRF determines the discovered NF instance(s) or NF service instance(s) and provides the information of a set of discovered NF instance(s) or NF service instance(s) to the NF/NF Service via Nnrf_NFDiscovery_Request Response message. The information includes: FQDN, IP address, or end point addresses (i.e. URLs) for the set of discovered NF instance(s) or NF service instance(s). In addition, it can include GUAMI(s), TAI(s), if available.

In case the target NF is UDR, UDM or AUSF, if SUPI was used as optional input parameter in the request, the NRF shall provide the corresponding UDR, UDM or AUSF instance(s) that matches the optional input SUPI. Otherwise, if SUPI is not provided in the request, the NRF shall return all applicable UDR instance(s) (e.g. based on the Data Set Id, NF type), UDM instance(s) or AUSF instance(s) (e.g. based on NF type) and if applicable, the information of the range of SUPI(s) and/or Data Set Id each UDR instance is supporting.

NOTE 3: The NF/NF Service doesn’t need to be aware if information provided by NRF is based on NF instance(s) or NF service instance(s).

### 4.17.5	NF/NF service discovery across PLMNs

In case that the NF/NF Service intends to discover the NF/NF service in home PLMN, the NRF in serving PLMN needs to request "NF Discovery" service from NRF in the home PLMN. The procedure is depicted in the figure below:

Figure 4.17.5-1: NF/NF service discovery across PLMNs

1.	The NF/NF Service in the serving PLMN invokes Nnrf_NFDiscovery_Request (Expected Service Name, NF type of the expected NF, home PLMN ID, serving PLMN ID, NF type) to an appropriate configured NRF in the serving PLMN. The request may also include optionally S-NSSAI, NSI ID if available, and other service related parameters, as described in service definition in clause 5.2.7.3.2.

NOTE:	The use of NSI ID within a PLMN depends on the network deployment.

2.	The NRF in serving PLMN identifies NRF in home PLMN (hNRF) based on the home PLMN ID, and it requests "NF Discovery" service from NRF in home PLMN according the procedure in Figure 4.17.4-1 to get the expected NF instance(s) or NF service instance(s) deployed in the home PLMN. As the NRF in the serving PLMN triggers the "NF Discovery" on behalf of the NF service consumer, the NRF in the serving PLMN shall not replace the information of the service requester NF, i.e. NF consumer ID, in the Discovery Request message it sends to the hNRF.

The hNRF may further query an appropriate local NRF in the home PLMN based on the input information received from NRF of the serving PLMN. The FQDN of the local NRF or Endpoint Address of local NRF's NF Discovery service in the home PLMN may be configured in the hNRF or may need to be discovered based on the input information.

3.	The NRF in serving PLMN provides the information e.g. FQDN of a set of the discovered NF or NF service instance(s) in NF Discovery Response message. Same as step 3 in clause 4.17.4 applies.

**** Second Change ****

### 4.17.7	NF/NF service status subscribe/notify in the same PLMN

Figure 4.17.7-1: NF/NF service status subscribe/notify in the same PLMN

1.	The NF/NF Service subscribes to be notified of newly registered/updated/deregistered NF instances along with its NF services. The NF invokes Nnrf_NFManagement_NFStatusSubscribe Request from an appropriate configured NRF in the same PLMN.

2.	The NRF authorizes the Nnrf_NFManagement_NFStatusSubscribe Request. Based on the profile of the expected NF/NF service and the type of the NF/NF Service, the NRF determines whether the NF is allowed to subscribe to the status of the target NF instance(s) or NF service instance(s).

3.	If allowed, the NRF acknowledges the execution of Nnrf_NFManagement_NFStatusSubscribe Request.

4.	NRF notifies about newly registered/updated/deregistered NF instances along with its NF services to the subscribed NF/NF Service.

NOTE:	The NF/NF Service unsubscribes to receive NF status notifications invoking

### 4.17.8	NF/NF service status subscribe/notify across PLMNs

In the case that the NF/NF Service intends to subscribe to the status of NF/NF service instance(s) in home PLMN, the NRF in serving PLMN needs to request "NF status subscribe" service from NRF in the home PLMN. The notification is sent from the NRF in the home PLMN to the NF/NF Service in the serving PLMN without the involvement of the NRF in the serving PLMN. The procedure is depicted in the figure below:

Figure 4.17.8-1: NF/NF service status subscribe/notify across PLMNs

NOTE 1:	The NRF in the home PLMN communicates with the NRF and the NF consumer in the serving PLMN via the SEPPs in the respective PLMNs. For the sake of clarity, SEPPs are not depicted in the flow.

1.	The NF/NF Service in the serving PLMN invokes Nnrf_NFManagement_NFStatusSubscribe Request from an appropriate configured NRF in the serving PLMN.

2.	The NRF in serving PLMN identifies NRF in home PLMN (hNRF) based on the home PLMN ID, and it requests "NF status subscribe" service from NRF in home PLMN. As the NRF in the serving PLMN triggers the "NF status subscribe" on behalf of the NF/NF Service the NRF in the serving PLMN shall not replace the information of the service requester NF, i.e. NF consumer ID, in the status subscribe Request message it sends to the hNRF.

3.	The NRF in serving PLMN acknowledges the execution of Nnrf_NFManagement_NFStatusSubscribe Request to the NF consumer in the serving PLMN.

4.	NRF in the home PLMN notifies about newly registered/updated/deregistered NF instances along with its NF services to the subscribed NF/NF Service in the serving PLMN.

NOTE 2:	The NF/NF Service unsubscribes to receive NF status notifications invoking Nnrf_NFManagement_NFStatusUnSubscribe service operation.

**** End of Changes ****