SA WG2 Meeting #128	S2-186786

July 2 - 6, 2018, Vilnius, Lithuania

Source:	Huawei, Hisilicon

Title:	Reduce latency to setup the user plane

Document for:	Discussion/Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: This contribution discusses how to reduce the latency to setup the user plane.

- Discussion

In S2#127bis meeting, a new key issue on “Latency to setup the user plane” has been approved:

- how to identify the delay-sensitive PDU Sessions and how the network authorizes the use of this feature.

- how to avoid the increase of delay to setup the user plane if and when the UE moves outside of the SMF Service Area while in Idle mode

In this paper we propose how to reduce the latency to setup the user plane.

- Proposal

It is proposed to add the following text into TR 23.726.

/*************************** Start of the first change ************************/

6.X	Solution #X: Reduce latency to setup the user plane

6.X.1	Overview

This solution addresses the key issue 6 "latency to setup the user plane",

- how to identify the delay-sensitive PDU Sessions and how the network authorizes the use of this feature.

- how to avoid the increase of delay to setup the user plane if and when the UE moves outside of the SMF Service Area while in Idle mode

The additional delay when the UE returned from IDLE can be categorized into two case:

-	Case 1: Additional I-UPF selection if the same SMF can still serve the UE. This is same as Rel-15.

-	Case 2: Additional I-SMF selection if the serving SMF can not control the N3UPF which connect to the UE serving RAN area.

Comparing above two case if the I-UPF is not changed, the additional SMF does not need be added or removed. So if the new I-UPF is not added/removed during the Service Request procedure, the additional latency can be reduced. The possible solutions that can solve the key issue includes:

6.X.2	Description of the solution

Alternative 1: deployment based solution

The UPF that supports the latency sensitive services can be planed properly in order to avoid the I-SMF/I-UPF change during service request procedure, i.e. to align the UPF service area same as the AMF service area. Thus when the UE does not change the serving AMF, no matter how to allocate TA list to the UE, the SMF/UPF will not be changed when the mobility procedure is triggered. This deployment requires that UPF has the same area coverage as the AMF service area.

The SMF that supports the latency sensitive services can be planed properly in order to avoid the I-SMF/I-UPF change during service request procedure, e.g. to align the SMF service area same as the AMF service area. Thus when the UE does not change the serving AMF, no matter how to allocate TA list to the UE, the SMF will not be changed when the mobility procedure is triggered. This deployment requires that SMF has the same area coverage as the AMF service area.

Alternative 2: Update the RA based on UPF service area of the PDU sessions that are latency sensitive

When the UE is registered, the RA is allocated to the UE as the normal procedure.

When one PDU Session is to be established, the SMF determines whether the PDU session is latency sensitive, e.g. based on indication received from UE, PCF, AF, or based on local policy.

The SMF selects the UPF and gets the UPF service area as the normal procedure. If the SMF determines a PDU session is latency sensitive, the SMF subscribes the UE mobility and provides the AMF with the UPF service area information. The AMF recalculates the RA based on the service area of UPF to assure that the UE will not change the UPF when the Service Request procedure is triggered. If the recalculated RA is different than the RA that has been provided to UE, the AMF triggers UE configuration update procedure to provide the new RA to UE.

During mobility procedure, AMF notifies UE new location information to the SMF. Per UE new location, the SMF determine whether the N3UPF need be reallocated. If the N3UPF is to be reallocated, the SMF provides the AMF with the new UPF service area information. The AMF based on that information calculates the RA and notifies it to the UE via the UE configuration update procedure

6.X.3	Impact of the solution to existing entities

Alternative 1: deployment based solution

- No impact on the existing entities, but has deployments restriction.

Alternative 2: Update the RA based on UPF service area of the PDU sessions that are latency sensitive

AMF impact:

- The AMF need recalculate RA based on UPF service area and triggers UE configuration update procedure to update RA.

SMF impact:

- The SMF need determine whether a PDU session is latency sensitive.

- The SMF sends the AMF of the UPF Service Area information to trigger the AMF recalculate the RA.

6.X.4	Evaluation of the solution

******************************* End of Changes *********************************