SA WG2 Meeting #129-Bis	S2-1812231

26 – 30 November 2018, West Palm Beach, USA

Source:	Telstra

Title:	5WWC: Conclusion for KI#16

Document for:	Discussion

Agenda Item:	6.7

Work Item / Release:	FS_5WWC/Rel-16

Abstract of the contribution: The document adds a conclusion that solution #13 is endorsed as a basis for normative work on AGF and UPF co-location.

# Introduction

The KI#2 requested solutions for optimising the user plane packet handling when W-5GAN is used as trusted access to 5G core.

The relevance of this requirement was confirmed by BBF in the LS-1811672, which called SA2’s attention to this issue being covered in BBF’s study SD-407:

- Support for a collocated AGF and UPF (see section 13.9)

The current solution #13, “Co-location of UPF and FAGF” provides a solution for the co-location, by extending signalling on N2 and the N_smf interfaces and the UPF capability registration. This solution actually supports UPF co-location either for BBF’s AGF or FMIF, both covered by the term “FAGF” for 5GC integration.

It is thus proposed to consider the solution #13 as basis for normative work for addressing KI#16 and the corresponding BBF scenario of interest.

# 2	Proposal

It is proposed to include the following changes in TR 23.716.

***** First Change *****

### 8.2.x	Conclusions for user plane optimisation

It is proposed to consider the solution #13 in clause 6.23 as basis for normative work for ensuring that the FAGF is co-located with UPF for a PDU session. This addresses KI#16, as well as the “support for co-located AGF and UPF” as per BBF SD-420v3.

***** End of Changes *****