SA WG2 Meeting #129BIS	S2-1812289

26 - 30 November 2018, West Palm Beach, USA

Source:	Samsung

Title:	Evaluation of CAPIF-based Service Framework

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: Evaluation of Solution 20 (Service Framework based on CAPIF).

# Introduction

Evaluation text is proposed for Solution 20 in TR 23.742.

# Proposal

The following changes are proposed to TR 23.742:

<<<< Start of changes >>>>

### 6.20.6	Impacts on existing Services and Interfaces

There are some NRF parameters that will require enhancement to Rel-15 CAPIF in order to provide the same NF service registration and discovery capabilities.

It may be necessary to evolve Rel-15 CAPIF to support the interaction of a CAPIF CCF in a serving PLMN with a CAPIF CCF in the home PLMN to support roaming cases.

### 6.20.7	Evaluation of the Solution

- At a high level there are overlaps between the functionality provided by the CAPIF Core Function and requirements placed on the Service Framework in the present document. There are also functions provided by the CAPIF Core Function that are not presently required for the Service Framework, and Service Framework requirements that aren’t part of the CAPIF Core Function. The CAPIF Core Function could therefore potentially be part of the Service Framework.

- Adopting the CAPIF Core Function would require interworking with existing NRF deployments.

- Full support for existing NRF registration and discovery would require updates to 23.222 to the CAPIF Core Function.

- Roaming solutions where both the serving and home PLMN’s deploy a CAPIF Core Function would require changes to 23.222.

<<<< End of changes >>>>