SA WG2 Meeting #129-bis	S2-1812636

26 – 30 November, West Palm Beach, FL, US	(revision of S2-1812558)

Source:	Vodafone

Title:	Update of solution 5

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: This pCR proposes an update of solution #5 for FS_UDICOM

# 1 Proposal

Solution #5 is addressing procedures for the scenario in which N26 interface is supported between EPC and 5GC and this pCR introduces the support of interworking scenarios without N26.

Additionally, it is proposed to introduce a new key for retrieving the SUPI associated to an IMPU that may have been used for a specific IMS interaction.

******************First change***********************

### 6.5.1	Introduction

This solution applies to Key Issue 1 and describes how HSS FE and UDM interact when data repositories are separate.

******************Next change***********************

#### 6.5.3.4	Handover procedures

##### 6.5.3.4.0	General

For handover procedures, three cases are differentiated according to TS 23.501 [x] and TS 23.502 [2]:

- Single registration mode with N26 support

- Single registration mode without N26 support

- Dual registration

For single registration mode with N26 support, any registration/attachment shall convey a cancelation of the location in the source system. The UE subscription data, with the exception of the Intersystem Continuity Data, will be valid in only one of the systems at a time.

For single registration without N26 support, registration/attachment shall also convey a cancelation of the location in the source system if simultaneous registration of both MME and AMF is not supported, according to TS 23.501 clause 5.17.2.3.1.

NOTE 1: 	Since UDM and HSS FE are split and separate repositories exist, simultanous registration of MME and AMF should always be possible, unless it is disabled by configuration in UDM, HSS FE and in the proposed Access Data Layer

This solution proposes that, in single registration modes with and without N26, the PGW-C+SMF with corresponding APN/DNN is replicated in both repositories and updated/queried by either party from their associated back-end repository. I.e. at handover 5GS-EPS, HSS FE will query from EPS UDR and at EPS-5GS, UDM will query the 5G UDR.

This replication, together with an indication regarding the UE being served by 5GC or EPC facilitates all handover scenarios with single registration.

This can be achieved as follows:

Figure 6.5.3.4.0-1 Storage of PGW-C+SMF and APN/DNN with UE camping in EPS in single registration mode

While for 5GS, it is stored as following:

Figure 6.5.3.4.0-2 Storage of PGW-C+SMF and APN/DNN with UE camping in 5GS in single registration mode

NOTE 2: 	To avoid impacts on HSS FE and its related procedures, the data is written directly by the Data Access Layer. HSS FE will find the information available when UE camps back in EPS and requests continuity for single registration without N26

For handover procedures in dual registration mode, the UE will remain registered/attached to the two system simultaneously with valid subscription data in both systems at the same time. Moving sessions between systems remains as a UE implementation option.

##### 6.5.3.4.1	5GS to EPS with N26

In 5GS to EPS handovers, the attach in the EPS will generate a Ud Request to modify data by HSS FE followed by a Ud Response by the EPS UDR and a Ud notification to the Data Access Layer. This will trigger a Nudr_DM_Notify to the UDM to indicate the UE is now under EPS control. UDM should in turn modify the location data in the 5G profile and will proceed to deregister the AMF the UE moved from.

Figure 6.5.3.4.1-1 Notification to UDM at UE attachment to EPS

******************Next change***********************

##### 6.5.3.4.2	EPS to 5GS with N26

In EPS to 5GS handovers, the registration in the 5GS will generate a Nudr request and subsequent notification when data is stored in the 5G repository. The Data Access Layer will capture that and generate a notification towards the HSS FE indicating the UE has left the EPS.

Figure 6.5.3.4.2-1 Notification to HSS FE at UE registration in 5GS

NOTE 1:	HSS FE subscribed to changes regarding UEs having 4G and 5G profiles.

The procedure follows the specification in TS 23.502 [2] clause 4.11.1.2.2 with the modification that it is the HSS FE the one initiating the relevant cancel location towards the MME node the UE has moved from.

NOTE 2:	The mapping of the acknowledgement from the 5G repository to the Ud Notify towards the HSS FE depicted in the previous chart is left for implementation based on operator deployment.

As an alternative to the previous flow, the Data Access Layer can modify the UE data in the EPS UDR, which in turn would trigger a notification towards the HSS FE (HSS FE would have subscribed to changes in the EPS UDR). This alternative would save impacts in the HSS FE that would only need to subscribe to changes in the EPS UDR and be notified when the data is updated.

##### 6.5.3.4.3	5GS to EPS without N26 – single registration

For single registration without N26, HSS FE will be configured as not supporting dual registration. Thus, the attach to the EPS when UE moves from 5GS to EPS, will imply a cancelation of location in the AMF from where the UE moved.

This procedure is aligned with what is described in clause 6.5.3.4.1 of this solution.

The EPS attach is expected to trigger a Ud Update operation towards the EPS UDR, and a response to HSS FE including the PGW-C+SMF address (together with APN/DNN information) being used.

The proposal with this solution is to replicate the information in both repositories. This way EPS UDR will contain the information about PGW-C+SMF needed for the handover cases and will return it to the MME in the Update Location Response. See general clause 6.5.3.4.0 of this solution.

##### 6.5.3.4.4	EPS to 5GS without N26 – single registration

For single registration without N26, UDM will be configured as not supporting dual registration. Thus, the registration to the 5GS when UE moves from EPS to 5GS, will imply a cancelation of location in the MME from where the UE moved.

The registration in the 5GS, would trigger a notification to the Access Data Layer that, in turn, would imply a Ud notification to the HSS FE to cancel the location in the MME. This procedure is aligned with what is described in clause 6.5.3.4.2 of this solution.

******************Next change***********************

##### 6.5.3.4.5	Handovers without N26 – dual registration

For handover procedures in dual registration mode, the UE will remain registered/attached to the two system simultaneously with valid subscription data in both systems at the same time. Moving sessions between systems remains as a UE implementation option.

##### Attachment/Registration of the UE in either system does not convey any cancellation in the source system. This is achieved by analysing the UDM and HSS FE configuration to support or not dual registrations.

#### 6.5.3.5	IMS interaction

##### 6.5.3.5.1	Access to IMS data

******************Next change***********************

##### 6.5.3.5.2	P-CSCF Restoration procedures

The P-CSCF restoration procedure via UDM is an optional mechanism according to TS 23.380 [8].

The following two options are considered regardless the support of N26 and UE mode of operation (single or dual registration).

Option 1: When UDM and HSS FE are split, the preferred P-CSCF restoration procedure would be that performed by detection of the failure by SMF/UPF according to TS 23.380 [8] clause 5.8.3, or using the PCF based P-CSCF restoration as specified in clause 5.8.5 of TS 23.380 [8]

NOTE 1:	The adaptation of IMS to service based architecture, making use of proper virtualisation/cloud mechanisms for e.g.self-healing, high availability, redundancy, etc, would most likely make unnecessary the use of the current restoration procedures.

Option 2: To enable the P-CSCF restoration via either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the S-CSCF to request the relevant P-CSCF restoration mechanism to the relevant entity. This procedure is according to TS 29.228 [10] clause 6.4.

Figure 6.5.3.5.2-1

NOTE 2:	This assumes that UDM implements the Cx operations SAR/SAA to trigger the P-CSCF restoration. Potential conversion of Diameter Cx operation to an Nudm service operation is out of the scope of this study.

NOTE 3:	It is assumed that S-CSCF is aware of the IMS registration status of the UE prior to request P-CSCF restoration.

Both HSS FE and UDM will check the UE registration status in the corresponding network in order to either execute the procedure (according to TS 23.380 [8]) or reject it.

No other impact is expected for restoration.

##### 6.5.3.5.3	Terminating domain selection information for IMS voice

According to TS 23.501 [6], clause 5.16.3.6, the UDM/HSS shall be able to query the serving AMF for T-ADS related information, and AMF shall respond to the query with the relevant information unless the UE is detached:

-	whether or not IMS voice over PS Session is supported in the registration area(s) where the UE is currently registered;

-	whether or not IMS voice over PS Session Supported Indication over non-3GPP access is supported in the WLAN where the UE is currently registered;

-	the time of the last radio contact with the UE; and

-	the current Access Type and RAT type.

Retrieval of T-ADS information is initiated by the IMS AS.

To retrieve the T-ADS information from either the UDM or the HSS FE, the SLF functionality is invoked in order to perform a user identity to HSS FE identity or UDM identity resolution.

The SLF will return the list of different servers (UDMs and HSS FEs) that may be in charge of the UE at that point in time, for the IMS AS to request the information for access domain selection. This procedure is according to TS 29.328 [11] clause 6.5.

If the UE is attached to IMS via 5GS, the T-ADS information request to the IMS HSS/SLF will end up in a Sh request (UDR/UDA) to UDM, which in turn will invoke the AMF service Namf_MT_ ProvideDomainSelectionInfo as specified in TS 23.501 [6] clause 5.16.3.6 and TS 23.228 [7] annex Y.

To translate IMPU to SUPI, for the cases in which the Sh request is based on IMPU, the 5G UDR can incorporate a new data key (=IMPU) for the Subscription Data data set to be used for the Nudr_DM_Query operation, in order to request identifier translation from IMPU to SUPI.:

The query to the 5G UDR is simply converted by the Access Data Layer into a Ud request to the IMS data repository, in order to retrieve the IMSI that is associated with the IMPU received in the Sh request.

The request from IMS AS, when the UE is attached to the EPS, should follow the same procedures as defined in TS 23.228 [7] clause 4.2.4a.

NOTE 1:	It is assumed that IMS AS is aware of the IMS registration status of the UE prior to request T-ADS information.

Both UDM and HSS FE will check the registration status in the corresponding network in order to either execute the procedure (according to TS 23.228 [7]) or reject it.

In case of dual registration, and according to annex D of TS 29.238 [11], the IMS AS will expect the T-ADS information containing 0 or 1 RAT types, depending on whether IMS Voice over PS is supported or not. So, in order to avoid impacts on the IMS AS, this solution proposes that UDM and HSS FE are configured with indication regarding being the default system for IMS voice. Both will inspect this default indication, in order to execute the T-ADS information retrieval procedure, or reject it.

Figure 6.5.3.5.3-1

NOTE 2:	This assumes that UDM implements the Sh operations UDR/UDA to trigger the T-ADS information retrieval procedure. Potential conversion of Diameter Sh operation to an Nudm service operation is out of the scope of this study.

******************Next change***********************

#### 6.5.4.2	Summary of impacts on UDM

-	For handovers without N26 support:

Add configuration indication regarding support or not of dual registrations

- 	For T-ADS information retrieval: Add Sh pair UDR/UDA in case of single and dual registrations

Add indication regarding default system for IMS Voice over PS for dual registration mode of operation.

-	For P-CSCF restoration:

For Option 1, no impact (i.e. make use of PCF based restoration or detection of failure by SMF/UPF)

For Option 2: Add Cx pair SAR/SAA for P-CSCF restoration

#### 6.5.4.3	Summary of impacts on HSS FE

No impact on network procedures.

For handovers without N26 support:

- Add configuration indication regarding support or not of dual registrations

For T-ADS information retrieval in dual registration mode of operation:

- Add indication regarding default system for IMS Voice over PS.

Depending on operator choice either:

-	A new subscription/notification interaction between HSS FE and the Data Access Layer in order to be informed when a 5G UE that camped in the EPS has moved back to 5GS system (in order to trigger a cancel location procedure), OR

-	No impact expected, if alternative depicted in clause 6.5.3.4.2 is decided.

#### 6.5.4.4	Summary of impacts on EPS UDR or 5G repository

The impacts on the EPS UDR or 5G repository are left for implementation, since the protocols and technologies to access their data is implementation specific.

******************End of changes***********************